3GPP SA WG2 Meeting #164	S2-2408414

Maastrich, NL Aug 18 – 23, 2024           	(revision of S2-240xxxx)

Source: 	NTT DOCOMO

Title:	Conclusions update for KI#4

Document for: 	Approval

Agenda Item: 	19.3.1

Work Item / Release:    FS_XRM_Ph2 / Rel-19

Abstract of the contribution: Update the conclusions for KI#4. Add support for RTP SDES "MID" header extension.

# Discussion

The use of RTP in the context of WebRTC is described in IETF RFC 8834.  According to RFC 8834, WebRTC implementations are required to support multiplexing of all RTP streams into a single transport connection:

Implementations are therefore also REQUIRED to support transport of all RTP packet streams, independent of media type, in a single RTP session using a single transport-layer flow, according to [RFC8860]

According to the IETF RFC 8860, Payload Type is never used for distinguishing RTP streams, instead this is done based on SSRC values:

As explained below, it is important to note that the RTP payload type is never used to distinguish RTP streams. The RTP packets are demultiplexed into RTP streams based on their SSRC; the RTP payload type is then used to select the correct media-decoding pathway for each RTP stream.

The WebRTC implementation that uses multiplexing of RTP streams into a single transport connection is expected to negotiate the use in SDP and use the RTP SDES header extension "MID" to differentiate the multiplexed RTP streams and to associate the RTP streams with the corresponding m-line sections in the SDP, see section 15 in IETF RFC 8843.

Based on the above, it is proposed to remove the Payload Type from the list of attributes that shall be supported by the additional packet filter and to add the RTP header extension SDES "MID" instead as defined in RFC 8843. IETF RFC 8872 can be removed from the list of RFCs to be supported, as it just provides some further guidelines for RTP multiplexing.

# Proposal

It is proposed to capture the following changes in XRM Ph2 TR 23.700-70.

*** First change ***

## 8.4	Interim agreements for Key Issue #4

The following aspects are concluded as principles for normative work:

1.	The AF may provide QoS requirements for media flows and additional packet filters as part of "AF session with required QoS". The detectable media flows that require different QoS treatment are mapped to distinct QoS Flows. If the AF does not provide any QoS requirement for the media flows or it provides the same QoS requirement, then the media flows can be mapped to the same QoS Flow. For the multiplexed media flows which cannot be differentiated by using the legacy IP Packet Filter, the AF may provide additional Packet Filter in the "AF session with required QoS" procedure to the NEF/PCF.

2.	In order to unique identify each media flow, the additional packet filter is used together with the legacy packet filter. The additional Packet Filter is used to differentiate the media flow among multiple media flows that share the same legacy packet filter both in uplink (in UE) and downlink (in UPF) in the following case:

a)	For the media flows that are transported in (S)RTP, (S)RTCP and other associated protocols that are multiplexed into a single UDP/IP traffic flow as specified in IETF RFCs 5761, 5764, 7983, 8843 and 9443. The additional Packet Filter SHALL be the Synchronization Source (SSRC), the Media Identification (MID) and RTP-M header fields.

3.	The PCC rule and N4 rules are enhanced to support the additional Packet Filter along with legacy packet filter.

Editor's note:	Whether there is a need for a new UE capability indication for UL is FFS (e.g. for legacy UE handling and network handling).

4.	The SMF may provide the received additional Packet Filter along with legacy packet filter to the UPF in the N4 rules. The PDI in PDR is extended to support the additional Packet Filter.

5.	The SMF provides QoS rules for the uplink traffic for the UE. QoS rules are enhanced to include additional Packet Filter when required (additional packet filter is not expected to be interpreted by a legacy UE; in case of legacy UE, it will use legacy packet filter). If supported by the UE, the UE uses both the legacy packet filter and the additional packet filter for QoS Flow mapping.

NOTE:	It is recommended that CT1 specifies additional packet filter in a backward compatibility manner so that legacy UE can match the XRM traffic to a QoS flow with the QoS requirement defined in r18 XRM.

*** END of changes ***