3GPP TSG-SA WG2 Meeting #164	S2-2408429

# Maastricht, NL, Aug 19 - 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1450 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of IMS DC event exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | KI#1 on "Extensible IMS mechanism supporting IMS events in the context of DC communication" and KI#2 on “Impact on IMS architecture, interfaces and procedures to support IMS service exposure in the context of IMS data channel session” were concluded during study phase. Conclusion principles can be found in clauses 8.1 and 8.2 of TR 23.700-77. This CR implements the conclusions principles in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Conclusion principles from TR 23.700-77 are implemented in TS 23.228. Specifically, Nhss and Nimsas services are enhanced and new procedures to support IMS DC event expoure are specified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#1 and KI#2 conclusions from TR 23.700-77 are not implemented in normative phase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.1.1, AA.2.1.2.1, AA.2.1.2.2, AA.2.1.X, AA.2.4.1, AA.2.4.Y, AC.2.1, AC.2.2.4, AC.2.2.6, AC.2.3, AC.Z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>START OF CHANGES <<<<

### AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| imsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| imsUE Context | Registration | Request/Response | S-CSCF, IMS AS |
| Management | DeregistrationNotification | Subscribe/Notify | S-CSCF |
| (_ImsUECM) | Deregistration | Request/Response | S-CSCF, IMS AS |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
| ImsUE Authentication | Get | Request/Response | S-CSCF |
| ImsEvent Exposure (_ImsEE) | Subscribe | Subscriber/Notify | NEF |
|  | Unsubscribe | Subscriber/Notify | NEF |

# >>>>NEXT CHANGE <<<<

#### AA.2.1.2.1	Nhss_ImsUECM_Registration service operation

Service operation name: Nhss_ImsUECM_Registration

Description: This service operation registers the serving S-CSCF or the serving IMS AS assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

Inputs, Required: Public Identity, S-CSCF name or IMS AS name, Registration Type (e.g. Initial Registration, Unregistered).

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

#### AA.2.1.2.2	Nhss_ImsUECM_Deregistration service operation

Service operation name: Nhss_ImsUECM_Deregistration

Description: This service operation deregisters the S-CSCF or the IMS AS allocated to a public identity.

Inputs, Required: S-CSCF name or the IMS AS name, Deregistration Type.

Inputs, Optional: User Identity (Private Identity and/or Public Identity), P-CSCF Restoration indication, Session Priority.

Outputs, Required: Result indication.

Outputs, Optional: None.

# >>>>NEXT CHANGE <<<<

### AA.2.1.X	Nhss_ImsEE service

#### AA.2.1.X.1	Nhss_ImsEE_Subscribe service operation

Service operation name: Nhss_ImsEE_Subscribe

Description: The NF consumer subscribes for the notification of a subscriber specific event. HSS shall store the subscription request and locate an IMS AS assigned to the subscriber.

Inputs, Required: Subscriber Identity, Event Type, Notification Target Address.

Inputs, Optional: Subscription Correlation ID (in the case of modification of the existing subscription), subscription expiry time.

Outputs, Required: Result indication, When the subscription is accepted, Subscription Correlation ID.

Outputs, Optional: None.

#### AA.2.1.X.2	Nhss_ImsEE_Unsubscribe service operation

Service operation name: Nhss_ImsEE_Unsubscribe

Description: The NF consumer unsubscribes for the notification of a subscriber specific event.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

# >>>>NEXT CHANGE <<<<

### AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |
| Nimsas_ImsEventExposure | Subscribe | Subscriber/Notify | HSS, NEF |
|  | Unsubscribe | Subscriber/Notify | HSS, NEF |
|  | Notify | Subscriber/Notify | NEF |

NOTE:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

# >>>>NEXT CHANGE <<<<

### AA.2.4.Y	Nimsas_ImsEventExposure service

#### AA.2.4.Y.1	Nimsas_ImsEventExposure_Subscribe service operation

Service operation name: Nimsas_ImsEventExposure_Subscribe

Description: The NF consumer subscribes for the notification of a subscriber specific event.

Inputs, Required: Subscriber Identity if the consumer is HSS, Event Type, Notification Target Address.

Inputs, Optional: Subscription Correlation ID (in the case of modification of the existing subscription), subscription expiry time.

Outputs, Required: Result indication, When the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

#### AA.2.4.Y.2	Nimsas_ImsEventExposure_Unsubscribe service operation

Service operation name: Nimsas_ImsEventExposure_Unsubscribe

Description: The NF consumer unsubscribes for the notification of a subscriber specific event.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### AA.2.4.Y.3	Nimsas_ImsEventExposure_Notify service operation

Service operation name: Nimsas_ImsEventExposure_Notify

Description: This service operation enables IMS AS to notify consumers about a subscribed event information.

Inputs, Required: Event Type, Notification Target Address.

Inputs, Optional: None.

Outputs, Required: Event information.

Outputs, Optional: None.

# >>>>NEXT CHANGE <<<<

## AC.2.1	Architecture

Figure AC.2.1-1 shows the data channel architecture when using the service-based DC media function.

Figure AC.2.1-1: Architecture option of IMS supporting DC usage with MF

Figure AC.2.1-2 shows the data channel architecture when using the MRF.

Figure AC.2.1-2: Architecture option of IMS supporting DC usage with MRF

NOTE 1:	MDC2 may connect different IMS security domains.

NOTE 2:	DC5 reference point is not specified in 3GPP.

NOTE 3:	MDC1, MDC2 and MDC3 reference points are not specified in 3GPP.

NOTE 4:	DC3 and DC4 reference points are not specified in this Release.

NOTE 5:	Enhancements to the N33 reference point for support of data channel services are not specified in this Release.

NOTE 6:	Event exposure via N73 is applicable for 5G and LTE access.

NOTE 7:	 Nhss services as indicated in Figure AC.2.1-1 are only applicable to N70, N71 and N72 reference points.

# >>>>NEXT CHANGE <<<<

### AC.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications.

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2 or wit MRF via Mr'/Cr for data channel media resource management.

-	The IMS AS interacts with HSS for retrieving and storing DC enhanced IMS AS service specific data via N71/Sh.

-	The IMS AS receives subscriber specific event subscription requests from HSS via N71/Sh.

-	The IMS AS receives non-subscriber specific event subscription requests from NEF via N74.

-	The IMS AS notifies the NEF about subscriber and non-subscriber specific events via N74.

# >>>>NEXT CHANGE <<<<

### AC.2.2.6	HSS

In order to support data channel service, the HSS is additionally enhanced with the following functionalities:

-	The HSS stores IMS Data Channel subscription data as transparent data.

-	The HSS interacts with DCSF during service data retrieval by DCSF via N72/Sc.

-	The HSS receives subscriber specific event subscription requests from NEF via N73.

-	The HSS stores subscriber specific event subscription requests per subscriber.

-	The HSS interacts with IMS AS during service data retrieval by IMS AS via N71/Sh.

-	The HSS sends subscriber specific event subscription requests to IMS AS via N71/Sh.

# >>>>NEXT CHANGE <<<<

## AC.2.3	Reference points

The following reference points are defined to support data channel service in IMS:

-	DC1:	Reference point between the DCSF and the IMS AS.

-	DC2:	Reference point between the IMS AS and MF.

-	DC3:	Reference point between the DCSF and NEF.

-	DC4:	Reference point between the DCSF and DC Application Server.

-	DC5:	Reference point between the DCSF and DCAR.

-	N72/Sc:	Reference point between the DCSF and HSS.

-	N73:	Reference point between the HSS and NEF.

-	N74:	Reference point between the IMS AS and NEF.

The following reference points are updated to support data channel signalling control in IMS:

-	N70/Cx/Dx:	Reference point between the CSCF and HSS.

-	N71/Sh:	Reference point between the IMS AS and HSS.

The following reference points are defined for data channel media handling:

-	MDC1:	Reference point for transport of data channel media between data channel media function (either MF or MRF) and DCSF.

-	MDC2:	Reference point for transport of data channel media between data channel media function (either MF or MRF) and DC Application Server.

-	MDC3:	Reference point for transport of data channel media between DCSF and DC Application Server.

The following reference point is updated to support data channel media handling:

-	Mr'/Cr:	SIP based reference point between IMS AS and MRF.

# >>>>NEXT CHANGE <<<<

# AC.Z	Support of IMS event exposure

## AC.Z.1	General

IMS event exposure allows a consumer (e.g., a DC AS) to subscribe to and being notified about subscriber specific and non-subscriber specific IMS DC related events and to invoke IMS DC specific services exposed by the IMS network via the NEF.

A subscriber specific request includes the targeted subscribers. The targeted subscribers can be either one distinct subscriber or multiple subscribers indicated by their IMS public user identities. The events in the subscriber specific request are common for all targeted subscribers. The NEF sends one event subscription request per targeted subscriber to the HSS serving the given subscriber. The NEF may determine the serving HSS based on configuration.

A non-subscriber specific event subscription request does not contain any targeted subscriber. Non-subscriber specific event subscription requests are sent by the NEF directly to all IMS AS instances. The NEF may be configured with available IMS AS instances supporting IMS DC or may use the NRF to discover IMS AS instances supporting IMS DC. The IMS AS instance supporting IMS DC may register with its DC capabilities in the NRF.

Once an IMS subscriber successfully registers in IMS and if an IMS AS supporting data channel services is selected and IMS DC is enabled in the subscriber profile in the HSS for the IMS subscriber, the IMS AS registers its address in the HSS.

NOTE 1:	The IMS AS supporting data channel services is always collocated with the TAS.

For subscriber specific event subscription requests received from the DC AS, the NEF uses Nhss_ImsEE and Nimsas_ImsEventExposure services to manage the subscription requests (e.g. subscribe/unsubscribe) via HSS. HSS shall support the handling of subscriber specific requests. If the HSS receives a subscriber specific event subscription request and the subscriber(s) are not registered in IMS, the HSS stores the event per non-registered subscriber. The information about the subscription events stored in HSS is only relevant and consumed by HSS.

NOTE 2:	In this release of the specification only IMS AS supporting IMS DC registers its instance in the HSS.

Following IMS DC related events are supported:

-	notify when a bootstrap data channel is established or terminated; and

-	notify when an application data channel is established or terminated.

NOTE 3:	Above events can be subscriber specific or non-subscriber specific.

Following subscriber specific services are exposed by the IMS network:

-	request to establish, update, terminate of bootstrap data channels; and

-	request to establish, update, terminate of application data channels.

To support IMS DC event exposure, following reference points are used:

-	N33 enables a DC AS to subscribe to and receive notifications about the requested IMS events of interest.

-	N73 enables the NEF to subscribe at HSS for IMS DC related events for a specific subscriber.

-	N71/Sh enables the IMS AS to register its address in the HSS for a specific subscriber and the HSS to subscribe at the IMS AS for a subscriber specific IMS DC event.

-	N74 enables the NEF to subscribe at the IMS AS for non-subscriber specific events and the IMS AS to send event notifications, for both subscriber specific and non-subscriber specific events, to the NEF.

## AC.Z.2	Procedure for subscriber specific event subscription and notification

Figure AC.Z.2-1: Procedure for subscriber specific event subscription and notification via NEF and HSS

Figure AC.Z.2-1 depicts the call flow when an DC AS requests to subscribe for a subscriber specific event:

1.	The DC AS subscribes to the NEF for the notification of a subscriber specific event using Nnef_ImsEE_Subscribe operation.

2.	The subscriber specific event subscription request is sent to the HSS using Nhss_ImsEE_Subscribe request by the NEF.

3.	If the UE is not registered in the IMS network, the subscriber specific event subscription request is stored in the HSS, until the UE is registered, and an IMS AS allocated for the UE is registered in HSS. If the UE has been registered in the IMS network and an IMS AS is allocated for the UE, step 3 can be skipped.

NOTE:	The IMS AS instance assigned to the UE is registered in the HSS using Nhss_ImsUECM_Registration operation.

4.	The HSS locates the IMS AS instance, which is assigned to the UE in the subscriber specific event subscription request.

5.	The HSS sends the subscriber specific event subscription request to the IMS AS instance assigned to the UE using Nimsas_ImsEventExposure_Subscribe request or using Sh interface.

6.	The IMS AS instance replies to the NEF via the HSS on successful subscriber specific event subscription using Nhss_ImsEE_Subscribe response or using Sh interface.

7.	The NEF replies to the DC AS on successful subscription to the subscriber specific event.

8.	A subscribed event has been detected by the IMS AS instance.

9.	The notification related to the subscribed event is sent to the DC AS that triggered the subscription request via the NEF using Nimsas_ImsEventExposure_Notify and Nnef_ImsEE_Notify operations.

## AC.Z.3	Procedure for non-subscriber specific event subscription and notification

Figure AC.Z.3-1: Procedure for non-subscriber specific event subscription and notification via NEF

Figure AC.Z.3-1 depicts the call flow when an DC AS requests to subscribe for a non-subscriber specific event:

1.	The DC AS subscribes to the NEF for the notification of a non-subscriber specific event using Nnef_ImsEE_Subscribe operation.

2.	The non-subscriber specific event subscription request is sent to all IMS DC capable IMS AS instances using Nimsas_ImsEventExposure_Subscribe request. The NEF may be configured with available IMS AS instances supporting IMS DC or may use the NRF to discover IMS AS instances supporting IMS DC.

3.	IMS AS replies to the NEF on successful non-subscriber specific event subscriptions using Nimsas_ImsEventExposure_Subscribe response.

4.	The NEF responses to the DC AS on successful subscription to the non-subscriber specific event.

5.	A subscribed event has been detected by an IMS AS.

6.	The notification related to the subscribed event is sent to the DC AS that triggered the subscription request via the NEF using Nimsas_ImsEventExposure_Notify and Nnef_ImsEE_Notify operations.

# >>>>END OF CHANGES<<<<