3GPP TSG-SA WG2 Meeting #164	S2-2408446

# Maastricht, Netherlands; 19th – 23rd August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3811 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Split MME architecture of Store and Forward operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of Store and Forward Satellite operation is concluded in clause 8.2 of TR 23.700-29. Following the conclusion, the description for the split MME architecture of Store and Forward operation is proposed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the description for split MME architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Store and Forward Satellite operation with split MME architecture is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.x.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change (All New) * * * *

4.13.x.2	Split MME architecture

4.13.x.2.1	General

In the Split MME architecture, the MME functionality is split into two parts: MME-ground (MME part which is on the ground network) and MME-onboard (MME part which is on board satellite).  The MME-ground acts as an anchor node which is responsible for maintaining the UE context. MME-ground synchronizes the UE context with all the MME-onboard(s). The MME-onboard is responsible for maintaining S1 connection towards RAN node and maintaining associated connection ID per UE towards MME-ground. MME-onboard is also responsible for encoding NAS payload from messages received from RAN and sending it to MME-ground and decoding NAS message received from MME-ground into S1AP payload.The example of the architectures is documented in Annex X

NOTE:	The interface between the MME-onboard and MME-ground is out of 3GPP scope.

For split MME architecture, the eNB and part of MME functions on the satellite, and all the other network functions including the other part of MME functions and HSS are deployed on the ground.

4.13.x.2.2	UE Authentication for S&F Satellite operation

After the UE sends the initial attach request to the MME-onbord as described in clause 4.13.x.1, it stores the "Request Time" when the Attach Reject message was sent. The MME-onboard forwards the stored Attach or TAU Request message and the "Request Time" to the MME-ground when the feeder link is available.

The MME-ground performs the Authentication and security procedures and fetches the authentication vector from HSS. Meanwhile, the HSS checks whether it receives the final Update Location Request as described in clause 4.13.x.1.

After getting the authentication vector and the subscription information from HSS, the MME-ground forwards them to the next available MME-onboard satellite which can reach the UE.

When UE subsequently determines that it is in satellite coverage, and if the wait timer has expired given to the UE, the UE re-attempts to register with the network by sending the Attach or TAU Request message to the satellite valid. When the UE gets RRC connected, the next MME-onboard authenticates the UE using the stored authentication vectors and accepts the Attach Request. As part of an Attach or TAU for S&F operation, the following may be provided to the UE in the accept message:

- Wait timer: Indicates to the UE the time it should wait before attempting signaling and data exchanges in those satellites.

- The list of Satellite IDs, over which the UE may exchange the signaling and data.

To authenticate UE without contacting HSS on the ground, multiple satellites may be involved to finish the Attach or TAU procedure.

4.13.x.2.3	MO data transmission

A UE can send the MO data to the serving satellite after it registes in S&F mode. The MO data can be stored in the MME on-board when the service link is available. As part of the Control Plane Service Request, or Service Request procedures for S&F operation, MME-board may provide the S&F delivery time to the UE.

According to the list of Satellite IDs, multiple satellites may be involved to store and forward the MO data to the ground. After the UE completes the MO data transmission, it either initiates a detach or TAU/RAU procedure if UE wants to use the PSM and request the Periodic TAU/RAU Timer, or sends a data transmission completion notification to the MME-onboard, which may trigger a MME-initiated detach procedure. If the UE doesn't complete the MO data transmission before the serving satellite flies away, it can access another satellite supporting S&F operation, and send the remaining MO data.

The MO data can be transferred to the ground when the feeder link is available. The S-GW/SCEF may receive a data transmission completion notification from the SCS/AS and notify the MME-ground of the successful delivery; otherwise, it should consider the data forwarding fails, and notify the MME-ground of the failure.

For MO SMS, upon reception of the MO SMS the MME-onboard stores the MO-SMS and may immediately send the delivery report(i.e. RP-ACK) to the UE i.e. as if the MO-SMS has already been successfully delivered to the Service Centre (SC).

4.13.x.2.4	MT data transmission

The MT data is stored in the MME-ground or in S-GW when the feeder link is unavailable and transferred to the MME-onboard when the feeder link becomes available. According to the list of Satellite IDs, multiple satellites may be involved to forward the MT data to the MME-onboard.

Before the service link is available, the MT data is stored in the MME-onboard. The MME-onboard will wait for UE to resume the connection by TAU procedure (e.g. after the Periodic TAU/RAU Timer of PSM expires, or return to the satellite's coverage) or re-attach procedure for MT data transfer according to the list of Satellite IDs.

Annex X (informative):

Example Satellite Store and Forward Operation Architectures

Figure 4.2.X-1: S&F architecture based on MME split

Figure 4.2.X-2 depicts the EPS architecture supporting Store & Forward based on split MME. S&F based on split MME is further described in clause 4.13.X.2.

* * * End of Changes * * * *