3GPP SA WG2 Meeting #164	 S2-2408450

# Maastricht, NL, 19 - 23 Aug 2024											                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5581 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI5_Provision of the DL traffic characteristics information changed dynamically |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | 19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following conclusion was agreed in TR 23.700-70 on KI#5 QoS Handling when Traffic Characteristics Change Dynamically and included in SP-240990 which was approved in SA#104 - Support the provision of the DL traffic characteristics information changed dynamically to the RAN in-band for the DL resource scheduling. 1. The AF may indicate to the NEF/PCF to mark the Burst size. 2. Based on the PCC rule and/or local configuration, the SMF instructs the UPF to identify and mark the Burst size and to further notify NG-RAN on the Burst size. 3. Based on SMF instruction, the UPF identifies and marks the Burst size, based on the input (i.e. Burst size) carried in N6 DL packet header. The UPF sends the Burst size to NG-RAN via DL GTP-U header. NOTE: This conclusion assumes that SA4 will specify Burst size indication in the header extension (i.e. RTP HE). 4. The NG-RAN may use the received Burst size to assist radio resource management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Feature description to support the KI#5 conclusions in clause 8.5 of TR 23.700-70. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature not implemented in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.x, 5.37.1, 6.3.3.3, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.37.x	QoS Handling when Traffic Characteristics Change Dynamically

5.37.x.1	General

For the QoS Handling when Traffic Characteristics Change Dynamically, 5GS support the provision of the DL traffic characteristics information changed dynamically to the RAN in-band for the DL resource scheduling.

The AF may indicates the Burst size marking together with the QoS Requirement and protocol description, as described in clause 6.1.3.27 of TS 23.503 [45]. The Burst size marking indication indicates the UPF to identify and mark the Burst size via the GTP-U EH of the DL packects, and send to the NG-RAN to assist radio resource management.

Upon the Burst size marking indication received from the AF, the PCF authorizes the PCC rules including the Burst size marking information, as described in clause 6.3.1 of TS 23.503 [45].

Based on the PCC rule provided by the PCF and/or local configuration, the SMF instructs the UPF to identify and mark the Burst size for the DL packets and to further notify NG-RAN on the Burst size.

Based on SMF instruction, the UPF identifies and marks the Burst size, based on the input (i.e. Burst size) carried in N6 DL packet header from AS. The UPF sends the Burst size to NG-RAN via the GTP-U EH of the DL packets.

Editor's note:	Details of the Burst size provision will be update based on the SA4 specification, e.g. Burst size is indicated in the header extension (i.e. RTP HE) via the first few packets of the burst. TS  26.522 [179] not yet updated by SA4.

Editor's note:	For a GBR QoS Flow using the Delay-critical resource type, whether and how the changed MDBV is provided via UPF to RAN, e.g. if the data burst is exceeding the MDBV, is FFS.

# * * * * Second change * * * *

5.37.1	General

This clause provides an overview of 5GS functionalities for support of XR services (AR/VR applications) and interactive media services that require high data rate and low latency communication, e.g. cloud gaming and tactile/multi-modal communication services according to service requirements documented in TS 22.261 [2]. The standardized 5QI characteristics for such interactive services are provided in Table 5.7.4-1 and TSCAI is used to describe the related traffic characteristics as defined in clause 5.27.2. Further enhancements for these interactive media services are as follows:

-	The 5GS may support QoS policy control for multi-modal traffic, see clause 5.37.2.

-	The 5GS may support network information exposure which can be based on ECN markings for L4S, see clause 5.37.3 or 5GS exposure API, see clause 5.37.4.

-	The 5GS may support PDU Set based QoS handling including PDU Set identification and marking, see clause 5.37.5.

-	The 5GS may ensure that the UL and DL packets together meet the requested round trip delay and also update the delay for UL and DL considering QoS monitoring results, see clause 5.37.6.

-	The 5GS may perform per-flow Packet Delay Variation (PDV) monitoring and policy control according to AF provided requirements, see clause 5.37.7.

-	The 5GC may provide traffic assistance information to the NG-RAN to enable Connected mode DRX power saving, see clause 5.37.8.

-	The 5GC may support the provision of the DL traffic characteristics information changed dynamically to the RAN in-band for the DL resource scheduling, see clause 5.37.x.

# * * * * Third change * * * *

6.3.3.3	Selection of an UPF for a particular PDU Session

The following parameter(s) and information may be considered by the SMF for UPF selection and re-selection:

-	UPF's dynamic load.

-	Analytics (i.e. statistics or predictions) for UPF load, Service Experience analytics and/or DN Performance analytics per UP path (including UPF and/or DNAI and/or AS instance) and UE related analytics (UE mobility, UE communication, and expected UE behavioural parameters) as received from NWDAF (see TS 23.288 [86]), if NWDAF is deployed.

-	UPF's relative static capacity among UPFs supporting the same DNN.

-	UPF location available at the SMF.

-	UE location information.

-	Capability of the UPF and the functionality required for the particular UE session: An appropriate UPF can be selected by matching the functionality and features required for an UE.

-	Data Network Name (DNN).

-	PDU Session Type (i.e. IPv4, IPv6, IPv4v6, Ethernet Type or Unstructured Type) and if applicable, the static IP address/prefix.

-	SSC mode selected for the PDU Session.

-	UE subscription profile in UDM.

-	DNAI as included in the PCC Rules and described in clause 5.6.7.

-	Local operator policies.

-	S-NSSAI.

-	Access technology being used by the UE.

-	Information related to user plane topology and user plane terminations, that may be deduced from:

-	5G-AN-provided identities (e.g. CellID, TAI), available UPF(s) and DNAI(s);

-	Identifiers (i.e. a FQDN and/or IP address(es)) of N3 terminations provided by a W-AGF or a TNGF or a TWIF;

NOTE 1:	A W-AGF or a TNGF may provide Identifiers of its N3 terminations when forwarding over N2 uplink NAS signalling to the 5GC. The AMF may relay this information to the SMF, as part of session management signalling for a new PDU Session.

-	Information regarding the user plane interfaces of UPF(s). This information may be acquired by the SMF using N4;

-	Information regarding the N3 User Plane termination(s) of the AN serving the UE. This may be deduced from 5G-AN-provided identities (e.g. CellID, TAI);

-	Information regarding the N9 User Plane termination(s) of UPF(s) if needed;

-	Information regarding the User plane termination(s) corresponding to DNAI(s).

-	RSN, support for redundant GTP-U path or support for redundant transport path in the transport layer (as in clause 5.33.2) when redundant UP handling is applicable.

-	Information regarding the ATSSS Steering Capability of the UE session (e.g. any combination of ATSSS-LL capability, MPTCP capability, MPQUIC capability) and information on the UPF support of RTT measurements without PMF.

-	Support for UPF allocation of IP address/prefix.

-	Support of the IPUPS functionality, specified in clause 5.8.2.14.

-	Support for High latency communication (see clause 5.31.8).

-	Support for functionality associated with high data rate low latency services, eXtended Reality (XR) and interactive media services, specified in clause 5.37 (for example, ECN marking for L4S, specified in clause 5.37.3, PDU Set Marking, specified in clause 5.37.5, UE power saving management, specified in clause 5.37.8, Burst size marking, specified in clause 5.37.x).

-	User Plane Latency Requirements within AF request (see clause 5.6.7.1 and clause 6.3.6 of TS 23.548 [130]).

-	List of supported Event ID(s) for exposure of UPF-related information via service based interface (see clause 7.2.29 and clause 5.2.26.2 of TS 23.502 [3]).

NOTE 2:	How the SMF determines information about the user plane network topology from information listed above, and what information is considered by the SMF, is based on operator configuration.

NOTE 3:	In this release the SMF uses no additional parameters for UPF selection for a PDU Session serving TSC or Deterministic Networking. If a PDU Session needs to connect to a specific UPF hosting a specific TSN 5GS bridge or 5GS router, this can be achieved e.g. by using a dedicated DNN/S-NSSAI combination.

If there is an existing PDU Session, and the SMF receives another PDU Session request to the same DNN and S-NSSAI, and if the SMF determines that interworking with EPC is supported for this PDU Session (as specified in clause 4.11.5 of TS 23.502 [3]), the SMF should select the same UPF if it supports all capabilities required for the new PDU Session. Otherwise, if the SMF determines that interworking with EPC is not supported for the new PDU Session or the UPF of the existing PDU Session does not support all capabilities required for the new PDU Session, a different UPF may be selected according to operator policy.

For the same DNN and S-NSSAI if different UPFs are selected at 5GC, when the UE is moved to EPC network, there is no requirement to enforce APN-AMBR. Whether and how to apply APN-AMBR for the PDN Connection associated with this DNN/APN is implementation dependent, e.g. possibly only AMBR enforcement per PDU Session applies.

# * * *  End of changes * * * *