3GPP TSG-SA WG2 Meeting #164	S2-2408459

19–23 August, Maastricht, The Netherlands 						rev of S2-2406560

Source:	NEC

Title:	KI#1 Conclusions

Document for:	Approval

Agenda Item:	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution porposes conclusions on KI1 of the FS_MASSS study.

1. Introduction

This contribution proposes the conclusion for KI#1 of MASSS study.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-54.

# 8	Conclusions

* * * Start of Change * * *

The following bullets are recommended for normative work for KI#1:

1. The UDM maintains association between the two USIMs related to the dual steer subscription. The UDM may mark one USIM as primary and another as secondary depending on the operator policy. 2. For each USIM subscription information the UDM maintains whether the UE is subscribed for the dual steer feature or not.

3. The UE initiates registration procedure of two USIM sequentially.

4. Depending on the operator policy, the network provides provides information to the dual steer UE whether a USIM is primary or secondary during the registration procedure during a registration procedure. The UE stores this information in the UE. From next power cycle the UE first performs registration to the primary USIM then secondary USIM.

If the Operator choose not assign the primary and secondary identification to the USIMs in this case the network doesn’t provide any identification to the UE. Depending on the dual steer UE implementation, the UE can choose any of the two USIMs for the first registration to the network. The network indicates whether the dual steer procedure is allowed or not for the UE in registration accept message.

5 The UE supporting dual steer feature sends its dualsteer capability to network in Registration Request message.

6. The network sends eURSP (extended URSP) after the successful registration of the first USIM. The eURSP rule indicates the condition when the second USIM will be registered.

7. The UE initiates registration procedure for the second USIM when the condition to initiate dual registration procedure meet as per the received eURSP policy.

NOTE: How the UE selects PLMN for second USIM is left to SA1 and CT1 decision.

8. The UE and the network make sure that the two corelated USIMs are in same device before initiating the Dual Steer feature. This part is defined in the Solution #1.8.

9. A separate URSP is sent to the each SUPIs. The existing URSP is enhanced to support the dual steer feature.

10. The principle of dual steer feature should be based on the ATSSS feature. The network sends Dual steer policy during the PDU session to initiate and control steering or switching procedure.

11. During the PDU sessions establishment procedure to support dual steer feature same anchor SMF and UPF are chosen. This can be achieved by including the correlation PDU session id during the PDU session establishment procedure of the two PDU sessions. The AMF selects the same SMF for the PDU sessions including this correlation PDU session id. This PDU session correlation id is also passed to the SMF to select same UPF.

12. The existing MA PDU session establishment procedure is used as basis to support dual steer procedure with modifications specific to the dual steer feature e.g., the network sends dual steer rules to the UE to enforce the UL user traffic for the UE and also PCF sends the dual steer rule for N4 interface. 13. PCC rules are updated to support the DualSteer. The DualSteer PCC rules include the applicable steering modes (i.e.  Active-Standby and Smallest Delay). The rules also include charging and monitoring keys for the 2nd 3GPP access. The rules also include 1st and 2nd access combination, including RAT and access type. The rules are determined based on the matching of the application traffic. (Solution #1.1, Section 6.1.1.1.4)

14. The extended URSP (eURSP) rules indicate that this is a Dual Steer PDU session, including all the Dual Steer applicable information, such as DNN/S-NSSAI, time and location validity criteria and others. They also include Traffic Descriptors.

15. When the DualSteer PDU session is established, all the decisions regarding traffic switching/steering are determined based on the DualSteer rules. The DualSteer rules include Traffic Descriptor as well as RAT combination/access type. The DualSteer rule, derived by SMF, will reflect the UE capability to support simultaneous transmission (as explained in e.g., part of Section 6.1.5.1.5	PDU Session establishment aspects).

16. DualSteer rules are sent during the DualSteer PDU session establishment or during the PDU Session modification.

17. The DualSteer rules are updated based on the PCC rule update.

* * * End of Change * * *