3GPP TSG SA WG2 Meeting #164	S2-2408461

# 19 – 23 August, 2024, Masstricht, NL

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1452 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authenitcation and Authorization of third party identities during IMS Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the TR 23.700-77 under KI#4, it was concluded how the IMS network is enhanced to support the usage of thrid party idnetities including authentication and authorization during IMS Session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Normative description for authentication and authorization of third party identities and its usage in IMS Session |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 specififcation will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex AX (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.002: "Network Architecture".

[2]	CCITT Recommendation E.164: "Numbering plan for the ISDN era".

[3]	CCITT Recommendation Q.65: "Methodology – Stage 2 of the method for the characterisation of services supported by an ISDN".

[4]	ITU Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[5]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[6]	Void.

[7]	3GPP TS 23.221: "Architectural Requirements".

[8]	3GPP TS 22.228: "Service requirements for the IP multimedia core network subsystem".

[9]	3GPP TS 23.207: "End-to-end QoS concept and architecture".

[10]	Void.

[10a]	3GPP TS 24.229: "IP Multimedia Call Control based on SIP and SDP; Stage 3".

[11]	3GPP TS 29.214: "Policy and Charging Control over Rx reference point".

[11a]	3GPP TS 29.207: "Policy control over Go interface".

[12]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[13]	IETF RFC 3986: "Uniform Resource Identifiers (URI): Generic Syntax".

[14]	IETF RFC 4282: "The Network Access Identifier".

[15]	IETF RFC 3966: "The tel URI for Telephone Numbers".

[16]	IETF RFC 3761 (April 2004): "The E.164 to Uniform Resource Identifiers (URI) Dynamic Delegation Discovery System (DDDS) Application (ENUM)".

[16a]	IETF RFC 4941: "Privacy Extensions for Stateless Address Autoconfiguration in IPv6".

[17]	ITU Recommendation G.711: "Pulse code modulation (PCM) of voice frequencies".

[18]	ITU Recommendation H.248: "Gateway control protocol".

[19]	3GPP TS 33.203: "Access Security for IP-based services".

[20]	3GPP TS 33.210: "Network Domain Security: IP network layer security".

[21]	Void.

[22]	3GPP TR 22.941: "IP Based Multimedia Services Framework".

[23]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[24]	3GPP TS 23.003: "Technical Specification Group Core Network; Numbering, addressing and identification".

[25]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[26]	3GPP TS 32.260: "Telecommunication Management; Charging Management; IP Multimedia Subsystem (IMS) charging".

[27]	3GPP TS 22.071: "Technical Specification Group Services and System Aspects, Location Services (LCS); Service description, Stage 1".

[28]	3GPP TS 23.271: "Technical Specification Group Services and System Aspects, Functional stage 2 description of LCS".

[29]	3GPP TS 23.078: "Customised Applications for Mobile network Enhanced Logic (CAMEL) Phase 3 - Stage 2".

[29a]	3GPP TS 22.340: "IMS Messaging; Stage 1".

[30]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message contents".

[31]	3GPP TS 23.240: "3GPP Generic User Profile - Architecture; Stage 2".

[32]	3GPP TS 22.250: "IP Multimedia Subsystem (IMS) group management"; Stage 1".

[33]	IETF RFC 2766: "Network Address Translation-Protocol Translation (NAT-PT)".

[34]	IETF RFC 2663: "IP Network Address Translator (NAT) Terminology and Considerations".

[35]	Void.

[36]	3GPP TS 23.141: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Presence Service".

[37]	Void.

[38]	IETF RFC 3840: "Indicating User Agent Capabilities in the Session Initiation Protocol (SIP)".

[39]	IETF RFC 3323: "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[40]	IETF RFC 3325: "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Network".

[41]	IETF RFC 3312: "Integration of resource management and Session Initiation Protocol (SIP)".

[42]	IETF RFC 3841: "Caller Preferences for the Session Initiation Protocol (SIP)".

[43]	IETF RFC 3428: "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[44]	IETF RFC 3263: "Session Initiation Protocol (SIP): Locating SIP Servers".

[45]	IETF RFC 5245: "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols".

[46]	IETF RFC 5766: "Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)".

[47]	IETF RFC 5389: "Session Traversal Utilities for NAT (STUN)".

[48]	IETF RFC 5626: "Managing Client Initiated Connections in the Session Initiation Protocol (SIP)".

[49]	IETF RFC 5627: "Obtaining and Using Globally Routable User Agent URIs (GRUUs) in the Session Initiation Protocol (SIP)".

[50]	IETF RFC 5628: "Registration Event Package Extension for Session Initiation Protocol (SIP) Globally Routable User Agent URIs (GRUUs)".

[51]	IETF RFC 4787: "Network Address Translation (NAT) Behavioural Requirements for Unicast UDP".

[52]	3GPP TS 23.279: "Combining Circuit Switched (CS) and IP Multimedia Subsystem (IMS) services; Stage 2".

[53]	3GPP TS 22.173: "IMS Multimedia Telephony Service and supplementary services; Stage 1".

[54]	3GPP TS 23.203: "Policy and Charging Control architecture".

[55]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[56]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access".

[57]	IETF RFC 4769: "IANA Registration for an Enumservice Containing Public Switched Telephone Network (PSTN) Signaling Information".

[58]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[59]	3GPP TS 29.333: "Multimedia Resource Function Controller (MRFC) - Multimedia Resource Function Processor (MRFP) Mp Interface; Stage 3".

[60]	3GPP2 X.S0011: "cdma2000 Wireless IP Network Standard".

[61]	3GPP2 C.S0001-D: "Introduction to cdma2000 Spread Spectrum Systems - Revision D".

[62]	3GPP2 C.S0024-A: "cdma2000 High Rate Packet Data Air Interface Standard, April 2004".

[63]	3GPP2 C.S0084-000: "Overview for Ultra Mobile Broadband (UMB) Air Interface Specification".

[64]	3GPP TS 24.167: "3GPP IMS Management Object (MO); Stage 3".

[65]	IETF RFC 3022: "Traditional IP Network Address Translator (Traditional NAT)".

[66]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) Centralized Services".

[67]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity".

[68]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[69]	3GPP TS 31.103: "Characteristics of the IP Multimedia Services Identity Module (ISIM) application".

[70]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[71]	3GPP TS 23.218: "IP Multimedia (IM) session handling; IM call model; Stage 2".

[72]	IETF RFC 3264: "An Offer/Answer Model with Session Description Protocol".

[73]	3GPP TS 23.333: "Multimedia Resource Function Controller (MRFC) - Multimedia Resource Function Processor (MRFP) Mp interface: Procedures Descriptions".

[74]	3GPP TS 23.334: " IMS Application Level Gateway (IMS-ALG) - IMS Access Gateway (IMS-AGW) interface: Procedures Descriptions".

[75]	3GPP TS 29.162: "Interworking between the IM CN subsystem and IP networks".

[76]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[77]	3GPP TS 22.153: "Multimedia priority service".

[78]	ETSI ES 282 003: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control Sub-System (RACS): Functional Architecture".

[79]	3GPP TS 29.328: "IP Multimedia (IM) Subsystem Sh Interface; Signalling flows and message contents".

[80]	3GPP TS 23.380: "IP Multimedia Subsystem (IMS); IMS Restoration Procedures".

[81]	3GPP TS 24.525: "Business trunking; Architecture and functional description".

[82]	3GPP TS 23.402: "Architecture Enhancements for non-3GPP accesses".

[83]	3GPP TS 33.328: "IP Multimedia (IM) Subsystem media plane security".

[84]	IETF RFC 8825: "Overview: Real Time Protocols for Brower-based Applications".

[85]	W3C: "WebRTC 1.0: Real-time Communication Between Browsers", W3C Recommendation, 26 January 2021, https://www.w3.org/TR/2021/REC-webrtc-20210126/.

[86]	W3C: "Cross-Origin Resource Sharing", W3C Proposed Recommendation, 05 December 2013, http://www.w3.org/TR/2013/PR-cors-20131205/.

[87]	ITU-T Recommendation T.140: "Protocol for multimedia application text conversation".

[88]	IETF RFC 6455: "The WebSocket Protocol".

[89]	IETF RFC 7118: "The WebSocket Protocol as a Transport for the Session Initiation Protocol (SIP)".

[90]	IETF RFC 4571: "Framing Real-time Transport Protocol (RTP) and RTP Control Protocol (RTCP) Packets over Connection-Oriented Transport".

[91]	3GPP TS 24.610: "Communication HOLD (HOLD) using IP Multimedia (IM) Core Network (CN) subsystem".

[92]	3GPP TS 23.179: "Functional architecture and information flows to support mission critical communication services; Stage 2".

[93]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[94]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[95]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[96]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[97]	3GPP TS 23.632: "User data interworking, coexistence and migration; Stage 2".

[98]	3GPP TS 29.563: "5G System (5GS); Home Subscriber Server (HSS) services for interworking with Unified Data Management (UDM); Stage 3".

[99]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description".

[100]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification".

[101]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description".

[102]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[103]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[104]	3GPP TS 26.264: "IMS-based AR Real-Time Communication".

[105]	3GPP TS 24.186: "IMS Data Channel applications; Protocol specification".

[XXX]	IETF draft-ietf-stir-passport-rcd-26: "PASSporT Extension for Rich Call Data".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

[YYY]	IETF draft-ietf-sipcore-callinfo-rcd-08: "SIP Call-Info Parameters for Rich Call Data".

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

**** Next Change ****

## 3.1	Definitions

Refer to TS 23.002 [1] for the definitions of some terms used in this document.

For the purposes of the present document the terms and definitions given in TR 21.905 [68] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [68].

For the purposes of the present document, the following terms and definitions given in TS 23.003 [24] apply:

-	Distinct Public Service Identity

-	Public User Identity

-	Wildcarded Public User Identity

-	Wildcarded Service User Identity

Alias Public User Identities: A set of Public User Identities that belong to the same alias group as specified in TS 29.228 [30].

ALG: Application Level Gateway (ALG) is an application specific functional entity that allows communication between disparate address realm or IP versions, e.g. an IPv6 node to communicate with an IPv4 node and vice versa, when certain applications carry network addresses in the payloads like SIP/SDP. NA(P)T-PT or NA(P)T is application unaware whereas ALGs are application specific translation entities that allow a host running an application to communicate transparently with another host running the same application but in a different IP version or IP address realm. See IETF RFC 2663 [34] for more details.

For IMS, an IMS ALG provides the necessary application function for SIP/SDP protocols in order to communicate between different address realms or IP versions, e.g. IPv6 and IPv4 SIP applications.

Application data channel: A data channel within an IMS session used to transfer data of data channel applications between UEs or between the UE and the network.

AR Application Server: An application server used to control the service logic related to AR communication via IMS data channel.

Bootstrap data channel: A data channel established within an IMS session between the UE and the network, to transfer a graphical user interface that can include a list of data channel applications.

Business trunking: as defined in TS 24.525 [81].

Data channel application: A HTML page including JavaScript(s) and optionally image(s) and style sheet(s). It is downloaded from the network to the UE through the bootstrap data channel.

DC Application Server: An application server that interacts with the DCSF and the DC media function for data channel traffic handling.

Distinct Public User Identity: used in relation to wildcarded Public User/Service Identities to denote an explicitly provisioned Public User/Service Identity. See more details in TS 23.003 [24].

Entry point: In the case that border control concepts are to be applied in an IM CN subsystem, then these are to be provided by capabilities within the IBCF, and the IBCF acts as an entry point for this network (instead of the I-CSCF). In this case the IBCF and the I-CSCF can be co-located as a single physical node. If border control concepts are not applied, then the I-CSCF is considered as an entry point of a network. If the P-CSCF is in the home network, then the I-CSCF is considered as an entry point for this document.

Exit point: If operator preference requires the application of border control concepts then these are to be provided by capabilities within the IBCF, and requests sent towards another network are routed via a local network exit point (IBCF), which will then forward the request to the other network (discovering the entry point if necessary).

Geographical Identifier: A Geographical Identifier identifies a geographical area within a country or territory. See more details in clause E.8.

Geo-local service number: A local service number that is used to access a service in the roamed network (a local service where the subscriber is located).

Home local service number: A local service number is used to access a service that is located in the home network of the user.

HSS Group ID: This refers to one or more SBI capable HSS instances managing a specific set of IMPIs/IMPUs.

IMC: IMS Credentials as defined in TR 21.905 [68].

IMS application reference: An IMS application reference is the means by which an IMS communication service identifies an IMS application.

IMS application: An IMS application is an application that uses an IMS communication service(s) in order to provide a specific service to the end-user. An IMS application utilises the IMS communication service(s) as they are specified without extending the definition of the IMS communication service(s).

IMS communication service identifier: An IMS communication service identifier uniquely identifies the IMS communication service associated with the particular IMS request.

IMS communication service: An IMS communication service is a type of communication defined by a service definition that specifies the rules and procedures and allowed medias for a specific type of communication and that utilises the IMS enablers.

IMS enabler: An IMS enabler is a set of IMS procedures that fulfils specific function. An IMS enabler may be used in conjunction with other IMS enablers in order to provide an IMS communication service.

Instance identifier: An identifier, that uniquely identifies a specific UE amongst all other UEs registered with the same Public User Identity.

Inter-IMS Network to Network Interface: The interface which is used to interconnect two IM CN subsystem networks. This interface is not constrained to a single protocol.

IP Flow: Unidirectional flow of IP packets with the following properties:

-	same destination IP address and port number;

-	same source IP address and port number;

-	same transport protocol (port numbers are only applicable if used by the transport protocol).

IP-Connectivity Access Network: refers to the collection of network entities and interfaces that provides the underlying IP transport connectivity between the UE and the IMS entities. An example of an "IP-Connectivity Access Network" is GPRS.

IP SM GW (IP short message gateway): An IP SM GW is an AS providing the support of Short Message Service of the IMS domain. See more details in TS 23.204 [56].

Local Service Number: A local service number is a telephone number in non-international format. A local service number is used to access a service that may be located in the home network of the user (home local service number) or the roamed network of the user (geo-local service number).

Media Flow: One or more IP flows carrying a single media instance, e.g. an audio stream or a video stream. In the context of this specification the term Media Flow is used instead of IP Flow regardless of whether the actual IP packet corresponds to media plane information (e.g. audio RTP flow) or control signalling (e.g. RTCP or SIP Signalling).

MPS: Based on TS 22.153 [77]. Multimedia Priority Service allows authorized users to obtain and maintain radio and network resources with priority, also during national security or emergency situations when PLMN congestion may occur.

MPS session: A session (e.g. voice, video, data session) for which priority treatment is applied for allocating and maintaining radio and network resources.

MPS-subscribed UE: A UE having a USIM with MPS subscription.

NAT-PT/NAPT-PT: NAT-PT uses a pool of globally unique IPv4 addresses for assignment to IPv6 nodes on a dynamic basis as sessions are initiated across the IP version boundaries. NAT-PT binds addresses in IPv6 network with addresses in IPv4 network and vice versa to provide transparent routing between the two IP domains without requiring any changes to end points, like the UE. NAT-PT needs to track the sessions it supports and mandates that inbound and outbound data for a specific session traverse the same NAT-PT router.

NAPT-PT provides additional translation of transport identifier (e.g. TCP and UDP port numbers, ICMP query identifiers). This allows the transport identifiers of a number of IPv6 hosts to be multiplexed into the transport identifiers of a single assigned IPv4 address. See IETF RFC 2766 [33] for more details.

Network Address Translation (NA(P)T): method by which IP addresses are mapped from one group to another, transparently to end users. Network Address Port Translation, or NA(P)T is a method by which many network addresses and their TCP/UDP (Transmission Control Protocol/User Datagram Protocol) ports are translated into a single network address and its TCP/UDP ports. See RFC 3022 [65] for further details.

Outbound: Managing Client Initiated Connections in the Session Initiation Protocol (Outbound) defines behaviours for User Agents, registrars and proxy servers that allow requests to be delivered on existing connections established by the User Agent. See RFC 5626 [48] for further details.

Preferred Circuit Carrier Access: An IMS service that allows a specific long distance circuit carrier to be selected for a long distance call.

Preferred Circuit Carrier Selection: An IMS service that allows the subscriber to select a long distance circuit carrier per call when dialling a call origination.

Rich Call Data: The data which contains the additional information of the calling party which helps in deciding the called party to accept the incoming call.

Service User: According to TS 22.153 [77].

Stand-alone Non-Public Network: A non-public network not relying on network functions provided by a PLMN.

STUN: Simple Traversal of UDP Through NAT (STUN), provides a toolkit of functions. These functions allow entities behind a NAT to learn the address bindings allocated by the NAT, to keep those bindings open, and communicate with other STUN-aware devices to validate connectivity. See RFC 5389 [47] for further details.

STUN Keep-alive: Is a usage of STUN, to keep NAT bindings open.

STUN Relay: Is a usage of STUN, that allows a client to request an address on the STUN server itself, so that the STUN server acts as a relay. See IETF RFC 5766 [46] for further details.

Subscriber: A Subscriber is an entity (comprising one or more users) that is engaged in a Subscription with a service provider. The subscriber is allowed to subscribe and unsubscribe services, to register a user or a list of user authorized to enjoy these services, and also to set the limits relative to the use that users make of these services.

STI-AS: Is a kind of IMS Application Server which is used for signging the calling party at the originating side.

STI-VS: Is a kind of Application Server which is user for verification of the signing done for the calling party at the terminating side.

Transport address: A unique identifier of transport-layer address, i.e. a combination of a network address, protocol identifier and port number. For example an IP address and a UDP port.

**** Next Change ****

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [68] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [68].

5GS	5G System

API	Application Program Interface

APN	Access Point Name

AS	Application Server

A2P	Application to Person

BCSM	Basic Call State Model

BG	Border Gateway

BGCF	Breakout Gateway Control Function

BS	Bearer Service

CAMEL	Customised Application Mobile Enhanced Logic

CAP	Camel Application Part

CDR	Charging Data Record

CN	Core Network

CS	Circuit Switched

CSCF	Call Session Control Function

CSE	CAMEL Service Environment

DC	Data Channel

DCAR	Data Channel Application Repository

DCMTSI	Data Channel Multimedia Telephony Service for IMS

DCSF	Data Channel Signalling Function

DHCP	Dynamic Host Configuration Protocol

DNN	Data Network Name

DNS	Domain Name System

ECN	Explicit Congestion Notification

ENUM	E.164 Number Mapping

GGSN	Gateway GPRS Support Node

GLMS	Group and List Management Server

GMLC	Gateway Mobile Location Centre

GRUU	Globally Routable User Agent URI

GUP	Generic User Profile

HSS	Home Subscriber Server

IBCF	Interconnection Border Control Function

I-CSCF	Interrogating-CSCF

IETF	Internet Engineering Task Force

IM	IP Multimedia

IMC	IMS Credentials

IMS	IP Multimedia Core Network Subsystem

IMS ALG	IMS Application Level Gateway

IMSI	International Mobile Subscriber Identifier

IN	Intelligent Network

IP	Internet Protocol

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

IP-CAN	IP-Connectivity Access Network

IP-SM-GW	IP Short Message Gateway

ISDN	Integrated Services Digital Network

ISIM	IMS SIM

ISP	Internet Service Provider

ISUP	ISDN User Part

IWF	Interworking Function

NP	Number portability

MAP	Mobile Application Part

MGCF	Media Gateway Control Function

MGF	Media Gateway Function

MRB	Media Resource Broker

MRFC	Multimedia Resource Function Controller

MRFP	Multimedia Resource Function Processor

NAI	Network Access Identifier

NAPT	Network Address Port Translation

NAT	Network Address Translation

NA(P)T-PT	Network Address (Port-Multiplexing) Translation-Protocol Translation

II-NNI	Inter-IMS Network to Network Interface

OSA	Open Services Architecture

P2A	Person to Application

P2P	Person to Person

P-CSCF	Proxy-CSCF

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCRF	Policy and Charging Rules Function

PDN	Packet Data Network

PDP	Packet Data Protocol e.g. IP

P-GRUU	Public Globally Routable User Agent URI

PLMN	Public Land Mobile Network

PSI	Public Service Identity

PSTN	Public Switched Telephone Network

QoS	Quality of Service

RAB	Radio Access Bearer

RCD	Rich Call Data

RFC	Request for Comments

SCS	Service Capability Server

S-CSCF	Serving-CSCF

SDP	Session Description Protocol

SGSN	Serving GPRS Support Node

SLF	Subscription Locator Function

SNPN	Stand-alone Non-Public Network

SSF	Service Switching Function

SS7	Signalling System 7

SIM	Subscriber Identity Module

SIP	Session Initiation Protocol

S-GW	Signalling Gateway

STI-AS	Secure Tepephone Identity-Authentication Service

STI-VS	Secure Tepehone Identity-Verification Service

TAS	Telephony Application Server

T-GRUU	Temporary Globally Routable User Agent URI

THIG	Topology Hiding Inter-network Gateway

TrGW	Transition Gateway

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

URL	Universal Resource Locator

USIM	UMTS SIM

**** Next Change ****

## AA.2.1	HSS Services

### AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| imsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| imsUE Context | Registration | Request/Response | S-CSCF |
| Management | DeregistrationNotification | Subscribe/Notify | S-CSCF |
| (_ImsUECM) | Deregistration | Request/Response | S-CSCF |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
| ImsUE Authentication | Get | Request/Response | S-CSCF |
| imsParameter Provision | Update | Request/Response | NEF, AF |
|  | Create | Request/Response | NEF, AF |
|  | Delete | Request/Response | NEF, AF |

### AA.2.1.x	Nhss_ImsParameterProvision service

#### AA.2.1.x.1	Nhss_ImsParameterProvision_Update service operation

Service operation name: Nhss_ImsParameterProvision_Update

Description: This service operation is used by the AF or NEF to update the parameters (e.g. third party identities like RCD information for the target IMPU.

Inputs, Required: Public User Identity, RCD information, Transaction Reference ID(s).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

### AA.2.1.y	Nhss_ImsParameterProvision service

#### AA.2.1.y.1	Nhss_ImsParameterProvision_Create service operation

Service operation name: Nhss_ImsParameterProvision_Create

Description: This service operation is used by the AF or NEF to create the parameters (e.g. third party identities like RCD information for the target IMPU.

Inputs, Required: Public User Identity, RCD information, Transaction Reference ID(s).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

### AA.2.1.z	Nhss_ImsParameterProvision service

#### AA.2.1.z.1	Nhss_ImsParameterProvision_Delete service operation

Service operation name: Nhss_ImsParameterProvision_Delete

Description: This service operation is used by the AF or NEF to delete the parameters (e.g. third party identities like RCD information for the target IMPU.

Inputs, Required: Public User Identity, RCD information, Transaction Reference ID(s).

Inputs, Optional: None.

Outputs, Required: Result Indication.

Outputs, Optional: None.

Annex AX (normative):
Support of authentication and authorization of third party identities in IMS

# AX.1	General

This clause describes the required functions to support the authentication and authorization of third party identities and it’s usage during IMS session. The IMS network may support the autthentication and authorization of third party identities used by usersbelonging to third party (e.g. One Enterprise) before they start availing IMS services. Following are the enhancement at IMS for the support of third party identities:

-	Support authorization check in IMS AS or S-CSCF whether the calling party (PBX / user) is authorized to use Third Party specific identity information based on the user profile retrieved from the HSS.

-	The definition of Third Party specific user identity information used in IMS follows the definitions in draft-ietf-sipcore-callinfo-rcd-10 [YYY].

-	Signing and verification of Third Party specific user identity information follows draft-ietf-stir-passport-rcd-26 [XXX].

-	Support the configuration of either an RCD address or optionally RCD information in the HSS repository data per IMPU or wildcarded IMPU.

-	Optionally, for some users (i.e. that have individual subscriptions in HSS), the operator ro third party may provision per IMPU RCD information like caller name, job title, organization, and location information in the HSS repository data via the NEF.

-	Optionally, for some users behind a PBX (i.e. managed via a wildcarded IMPU in HSS), the operator or third party may provision RCD information per wildcarded IMPU in the HSS repository data via the NEF.

NOTE:	Support for multi RCD records per IMPU or wildcarded IMPU is out of scope of this release.

-	Optionally, for some users, the operator or third party may provision an RCD address per IMPU in the HSS repository data.

-	The RCD address can be used by the IMS AS to fetch Third Party specific user identity information from a Third Party server. The RCD address is either the Third-Party server address, in which case the IMPU is provided to the Third Party server in a request to fetch user specific RCD information to be signed and included in the outgoing SIP INVITE message. Alternatively, the RCD address is an URL pointing to the user specific RCD information stored on the Third Party server, in which case the fetched RCD information or optionally the RCD address itself is signed and included in the outgoing SIP INVITE message.

-	The RCD address or RCD information is downloaded by the IMS AS from the HSS during IMS session setup and, based on local policy, forwarded by the IMS AS to the S-CSCF.

-	For inter domain calls, the IBCF or IMS AS invoke signing and verification of Rich Call Data information based on local policies and trust relationship with the Third Party network.

-	The fetched RCD address or RCD information is used by the Signing AS for signing the RCD PASSporT and by the Verification AS in the terminating network to verify the signed RCD PASSporT.

-	If signing and verification for intra domain calls is required, based on operator policy, S-CSCF or IMS AS invoke signing and verification.

# AX.2	Procedure

Support for authentication and authorization of third party identities, additional functionalities in the S-CSCF, STI-AS, STI-VS, HSS as will be depicted below.

Figure AX.2-1: Authentication and Authorization of third party identity during an IMS session

The detailed procedure is described in Figure AX.2-1.

1.	The AF (e.g. third party like an Enterprise) will provision the third party identities/RCD information by invoking the service operation Nhss_ImsParameterProvision service for the target user (i.e for single IMPU or Wildcard IMPU).

2.	UE1 is already registered.

3.	UE1 initiate an IMS session by sending INVITE to UE2.

4.   S-CSCF or IMS AS download the subscription information of the IMPU from HSS.

5.	S-CSCF or IMS AS checks the subscription information whether the user is authorized to use the third party identity service. If the parameter is enabled, then S-CSCF extract the third party identity from the PAI/From/Call-Info header and validates against the stored third party identities/RCD infromation for the IMPU in the subscription profile.

If the third party identity/RCD information is not present in subscription profile then S-CSCF contacts configured third party server to retrieve the third party identity/RCD information.

6.	Upon a successful match, S-CSCF or IMS AS will invoke the Signing server (STI-AS) . This Signing server address, S-CSCF may get it from subscription information.

7.	Signing server after receiving the INVITE from IMS originating network/S-CSCF, will add identity header and sign it as defined in the TS 24.229 [10a].

8.  S-CSCF forwards the INVITE to the terminating side.

9.	S-CSCF or IMS AS at terminating side will invoke the verification server based on the presence of Identity header and forwards the INVITE request to STI-VS

10.	Verification server (STI-VS) will verify the Identity header using certificates and provides the validity status along with the INVITE to S-CSCF.

11.	S-CSCF at terminating side sends the INVITE towards UE2 based on the successful validation result. S-CSCF may reject the call for the unsuccessful result received from verification server.

12.	Successful call happens between UE1 & UE2.