3GPP TSG-SA WG2 Meeting #164	S2-2408462

19th–23rd August, 2024 Maastricht, The Netherlands						rev of S2-2406548

Source:	NEC

Title:	KI#1 Term definition of primary and secondary SUPI.

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This contribution porposes a term definition of primary SUPI and secondary SUPI.

1. Introduction

This proposes the term definition of primary SUPI and secondary SUPI.

2. Discussion

SA1 requirement introduces two USIMs to support dual steer feature. These USIMs are corelated in the UDM. The following discussion proposes why we need these two USIMs as a primary USIM and secondary USIMs. Primary USIM will be a USIM to which all basic services will be tied up e.g. IMS services (IMS call),of the user has been tied up and secondary USIM will be a USIM which will be used for the dual steer feature along with primary USIM. This pCR also introduces term definition of Primary SUPI and secondary SUPI.

Following are the reasons we need to clasify the two USIMs as primary and seconday USIMs.

- SA1 requirement implicity supports Primary and Secondary USIM concept.

The original requirement fromSA1 to support dual steer feature was for single USIM. Second USIM will be introduced to handle the handover case of two 3GPP acccesses related to single USIM to the target slegacy network which can’t handle two handover commands for a single SUPI. So a second USIM is introduced to handle handover of the two 3GPP access connections to the legacy network.

- Coveniencce to the user and less complexity.

If a single MSISDN number is given to the user which subscribed to the dual steer subscription and this MSISDM number is tied to the primary USIM then the user can accesses the IMS services like currently there won’t be any change.

If two MSISDNs one for each USIM are given to the user subscribed to the dual steer feature then it will be difficult for the user to manage two MSISDN number. In order to effectively handle the functions related two independent IMS call, the UE dual steer UE needs to implement dual USIM features. Considering the current TU budget it is not possible to implement all dual USIM feature in dual steer device.

- Better network resource utilization and less power consumption in the UE.

Primary USIM will always be registered with network to provide the basic features to the user and the secondary USIM will registered only when the dual steer feature needs to be used. This will reduce the network resource and UE battery power consumption.

If both USIMs are registered and dual steer feature is not required at the moment then it will cause unnecesary signaling to the network related to periodic registration procedure and mobility management. This will also impact the UE power consumption infact the power consumption in the UE will increase significantly.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-54 V0.3.0.

# 3	Definitions of terms and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Non-Integrated Non-3GPP Access: a type of non-3GPP access network that provides direct IP connectivity between the UE and the UPF without any intermediate NF such as Non-3GPP InterWorking Function (N3IWF) and Trusted Non-3GPP Gateway Function (TNGF). When a UE is connected using Non-Integrated Non-3GPP Access, the UE data flows can be routed via this access to operator services.

Primary USIM:	A USIM of the dual steer subscription which provides basic services (IMS service, internet services) to a subscriber. The UDM of the HPLMN classify whether a USIM is a primary USIM. This USIM is registered to access the basic services. The UDM of the HPLMN classify whether the USIM is primary USIM.

Secondary USIM: A USIM of the dual steer subscription which provides dual steer services along with primary USIM. This USIM is registere to the PLMN when dual servie is required at the UE. The UDM of the HPLMN classify whether the USIM is secondary USIM.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

NIN3A	Non-Integrated Non-3GPP Access

* * * Start of Change * * *