3GPP SA WG2 Meeting #164	S2-2408486

# Maastricht, NL, 19-23 August 2024	               										    (was S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0478 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe Layer-3 Multi-hop UE-to-UE Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of 5G ProSe Layer-3 Multi-hop UE-to-UE Relay based on conclusions in TR 23.700-03. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 2: references to relevant IETF MANET RFCs Clause 4.2.x: reference architecture Clause 5.14.X (new): 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication Clause 6.4.3.X (new): Layer-2 link management for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature not implemented in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2; 4.2.x (new); 5.14.x (new); 6.4.3.x; (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[3]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[6]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[7]	3GPP TS 22.278: "Service requirements for the Evolved Packet System (EPS)".

[8]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[10]	Void.

[11]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[12]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[13]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in idle mode".

[14]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol Specification".

[17]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[18]	IETF RFC 3927: "Dynamic Configuration of IPv4 Link-Local Addresses".

[19]	IETF RFC 826: "An Ethernet Address Resolution Protocol".

[20]	Void.

[21]	3GPP TR 23.752: "Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS)".

[22]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

[23]	3GPP TS 24.554: "Proximity-services (ProSe) in 5G System (5GS) protocol aspects; Stage 3".

[24]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[25]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[26]	Void.

[27]	Void.

[28]	3GPP TS 38.351: "NR; Sidelink Adaptation Layer Protocol".

[29]	3GPP TS 33.503: "Security Aspects of Proximity based Services (ProSe) in the 5G System (5GS)".

[30]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[31]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[32]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[33]	3GPP TS 22.268: "Public Warning System (PWS) requirements".

[34]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[xx]	IETF RFC 7181: "The Optimized Link State Routing Protocol Version 2".

[yy]	IETF RFC 6130: "Mobile Ad Hoc Network (MANET) Neighborhood Discovery Protocol (NHDP)".

NEXT CHANGE

### 4.2.x	5G ProSe Multi-hop UE-to-UE Relay reference architecture

Figure 4.2.x-1 shows the Layer-3 5G ProSe UE-to-UE Relay reference architecture. The 5G ProSe End UEs communicate with each other via a set of interconnected 5G ProSe Multi-hop UE-to-UE Relays forming a MANET network as defined in IETF RFC 7181 [xx].

Figure 4.2.x-1: Reference architecture for Layer-3 5G ProSe Multi-hop UE-to-UE Relay

Based on capability and configuration the 5G ProSe UE can act as a 5G ProSe End UE, a 5G ProSe Multi-hop UE-to-UE Relay or both.

A 5G ProSe UE acting as 5G ProSe UE-to-UE Relay shall support the protocols defined in IETF RFC 7181 [xx] and IETF RFC 6130 [yy] on the PC5 interfaces towards other 5G ProSe Multi-hop UE-to-UE Relays. In addition, the 5G ProSe UE-to-UE Relay shall support a dedicated MANET message (i.e. [MANET] Discovery Info message defined in [FFS]) on the PC5 interfaces towards other 5G ProSe Multi-hop UE-to-UE Relays for propagation of DNS information, as described in clause 5.14.x.2.

Editor's note:	It is up to CT1 WG to determine where and how to specify the [MANET] Discovery Info message.

Each 5G ProSe End UE and the 5G ProSe UE-to-UE Relay may have subscriptions from the same PLMN or different PLMNs.

NEXT CHANGE

### 5.14.x	5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication

#### 5.14.x.1	General

Editor's note:	To be completed.

#### 5.14.x.2	IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication

The RSC configuration as defined in 5.1.5.1 indicates if IP based 5G ProSe Layer-3 Multi-hop UE-to-UE Relay communication is used.

Based on configuration associated with the RSC, the 5G ProSe UE obtains the IP address/prefix as follows:

-	If the 5G ProSe UE is configured to act as 5G ProSe End UE only, it obtains an IP address/prefix from the 5G ProSe UE-to-UE Relay to which it connects.

-	If the 5G ProSe UE is configured to act as 5G ProSe Multi-hop UE-to-UE Relay the 5G ProSe UE shall be configured with a routable IP address/prefix associated with the RSC.

Mobile Ad-hoc Network (MANET) based routing protocols are used over the IP connections in the relay cloud to provide multi-hop forwarding functionality between 5G ProSe End UEs. Each of the 5G ProSe Multi-hop UE-to-UE Relays acts as a MANET router and supports at least the MANET Neighbor Discovery Protocol (NHDP) [yy] and OLSRv2 [xx].

The 5G ProSe End UE obtains the IP address/prefix of the target 5G ProSe End UE via DNS queries.

When a 5G ProSe End UE is assigned an IP address/prefix by the 5G ProSe Multi-hop UE-to-UE Relay, the 5G ProSe Multi-hop UE-to-UE Relay propagates the DNS information (i.e. User Info ID and associated IP address/prefix) to all other 5G ProSe Multi-hop UE-to-UE Relays connected to the same MANET using a dedicated MANET message (i.e. [MANET] Discovery Info message defined in [FFS]).

Editor's note:	It is up to CT1 WG to determine where and how to specify the [MANET] Discovery Info message.

The 5G ProSe End UE may establish multiple Layer-2 links with different 5G ProSe UE-to-UE Relays supporting the same RSC, in order to maximize the reachability (e.g. in some cases, different 5G ProSe UE-to-UE Relays may serve different disjointed IP subnets). The routing decision in the 5G ProSe End UE related to which Layer-2 link to use when sending a packet is implementation-specific.

NEXT CHANGE

#### 6.4.3.x	Layer-2 link management for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay

##### 6.4.3.x.1	Layer-2 Link establishment and management for IP PDU type

For the 5G ProSe Communication via Layer-3 5G ProSe Multi-hop UE-to-UE Relay as described in clause 4.2.x, the description in clause 6.4.3.7.1 applies with following differences and clarifications:

-	If the 5G ProSe UE is configured to act as 5G ProSe End UE only, it obtains an IP address/prefix from the 5G ProSe UE-to-UE Relay to which it connects.

-	If the 5G ProSe UE is configured to act as 5G ProSe Multi-hop UE-to-UE Relay the 5G ProSe UE shall be configured with a routable IP address/prefix associated with the RSC.

When a new layer-2 link is established between a pair of 5G ProSe UEs configured to act as 5G ProSe Multi-hop UE-to-UE Relay, the 5G ProSe Multi-hop UE-to-UE Relays initiate the MANET Neighborhood Discovery Protocol (NHDP) [yy], followed by the OLSRv2 protocol [xx].

End of CHANGEs