3GPP TSG-SA2 Meeting #164	S2-2408494

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0480 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | multi-hop Layer-3 UE-to-UE Relay for Ethernet PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 23.700-03 has conclusion that multi-hop Layer-3 UE-to-UE Relay for Ethernet PDU Type will be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | multihop U2U relay communication setup procedure with ethernet pdu and Layer-2 link management over PC5 reference point is updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | U2U multi-hop relay for Ethernet PDU type will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.7.x (new), 6.7.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 6.4.3.7.x	Layer-2 link management over PC5 reference point for multi-hop 5G ProSe Layer-3 UE-to-UE Relay

For the 5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-UE Relay as described in clause 6.7.x, the description in clause 6.4.3.7.1 applies with following differences and clarifications:

-	The Direct Communication Request message over the PC5 reference point between source End UE and the first hop UE-to-UE Relay includes, additionally, the End-to-End path Information, which is an (ordered) list of User Info ID of 5G ProSe UE-to-UE Relays selected by the source 5G ProSe End UE based on the path information provided to the source 5G ProSe End UE during 5G ProSe UE-to-UE Relay Discovery procedure, and QoS Info, which indicates the End-to-End QoS Info from the source End UE to the target End UE.

-	The Direct Communication Request message over the PC5 reference point between two UE-to-UE Relays includes, the User Info of source 5G ProSe End UE, the End-to-End path Information, End-to-End QoS Info, the information about the ProSe identifier(s), RSC, Security Information (see TS 33.503 [29]), and the User Info of target 5G ProSe End UE.

-	The Direct Communication Request message over the PC5 reference point between between the target End UE and UE-to-UE Relay includes, additionally, the End-to-End path Information, End-to-End QoS Info.

-	The Direct Communication Accept message over the PC5 reference point between target End UE and UE-to-UE Relay includes, additionally, the End-to-End path Information, per-hop QoS Info for the hop between target End UE and the last UE-to-UE Relay.

-	The Direct Communication Accept message over the PC5 reference point between two UE-to-UE Relays includes, the User Info of target 5G ProSe End UE, the End-to-End path Information, per-hop QoS Info for the hop between two UE-to-UE Relays and the accumulated QoS Info of the previous hops from the target End UE to the current UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the target 5G ProSe Layer-3 End UE, the Ethernet MAC address of the 5G ProSe Layer-3 UE-to-UE Relay itself and may provide the Ethernet MAC address of the source 5G ProSe Layer-3 End UE.

-	The Direct Communication Accept message over the PC5 reference point between the source End UE and UE-to-UE Relay includes, additionally, the End-to-End path Information, per-hop QoS Info for the hop between two UE-to-UE Relays and the accumulated QoS Info of the previous hops from the target End UE to the current UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the target 5G ProSe Layer-3 End UE, and the Ethernet MAC address of the 5G ProSe Layer-3 UE-to-UE Relay itself.

-	The Link Modification Request message over the PC5 reference point between source End UE and the first hop UE-to-UE Relay includes, additionally, the End-to-End path Information, End-to-End QoS Info. If the PC5 link is used for transferring Ethernet traffic, the source 5G ProSe Layer-3 End UE provides its Ethernet MAC address.

-	The Link Modification Request message over the PC5 reference point between two UE-to-UE Relays includes, the User Info of source 5G ProSe End UE, the End-to-End path Information, End-to-End QoS Info, the information about the ProSe identifier(s), and the User Info of target 5G ProSe End UE. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the target 5G ProSe Layer-3 End UE, and the Ethernet MAC address of the 5G ProSe Layer-3 UE-to-UE Relay itself.

-	The Link Modification Request message over the PC5 reference point between the target End UE and UE-to-UE Relay includes, additionally, the End-to-End path Information, End-to-End QoS Info. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the source 5G ProSe Layer-3 End UE and the Ethernet MAC address of the 5G ProSe Layer-3 UE-to-UE Relay itself.

-	The Link Modification Accept message over the PC5 reference point between the target End UE and UE-to-UE Relay includes, additionally, the End-to-End path Information, per-hop QoS Info for the hop between target End UE and the last UE-to-UE Relay.

-	The Link Modification Accept message over the PC5 reference point between two UE-to-UE Relays includes the the End-to-End path Information, per-hop QoS Info for the hop between two UE-to-UE Relays and the accumulated QoS Info of the previous hops from the target End UE to the current UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the target 5G ProSe Layer-3 End UE, the Ethernet MAC address of the 5G ProSe Layer-3 UE-to-UE Relay itself and may provide the Ethernet MAC address of the source 5G ProSe Layer-3 End UE.

-	The Link Modification Accept message over the PC5 reference point between source End UE and the first UE-to-UE Relay includes, additionally, the End-to-End path Information, per-hop QoS Info for the hop between two UE-to-UE Relays and the accumulated QoS Info of the previous hops from the target End UE to the current UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the target 5G ProSe Layer-3 End UE, and the Ethernet MAC address of the 5G ProSe Layer-3 UE-to-UE Relay itself.

-	In the Security Procedure of the PC5 reference point between source End UE and UE-to-UE Relay, the source 5G ProSe Layer-3 End UE provides the QoS Info of the end-to-end QoS to the 5G ProSe Layer-3 UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the source 5G ProSe Layer-3 End UE provides its Ethernet MAC address.

-	In the Security Procedure of the PC5 reference point between two UE-to-UE Relays, a 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info from itself to the target 5G ProSe Layer-3 UE-to-UE Relay. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the source 5G ProSe Layer-3 End UE and the Ethernet MAC address of the 5G ProSe Layer-3 UE-to-UE Relay itself.

-	In the Security Procedure of the PC5 reference point between target End UE and a UE-to-UE Relay, the 5G ProSe Layer-3 UE-to-UE Relay provides the QoS Info of the last hop QoS to the target 5G ProSe End UE. If the PC5 link is used for transferring Ethernet traffic, the 5G ProSe Layer-3 UE-to-UE Relay provides the Ethernet MAC address of the source 5G ProSe Layer-3 End UE and the Ethernet MAC address of the 5G ProSe Layer-3 UE-to-UE Relay itself.

-	For adding new End-to-End QoS flow or modifying existing End-to-End QoS flow, the Link Modification Accept message over each PC5 reference point additionally includes QoS Info of that hop. If the PC5 link is used for transferring Ethernet traffic, the target 5G ProSe Layer-3 End UE may provide its Ethernet MAC address and the MAC address of the source 5G ProSe Layer-3 End UE. For a path between a specific pair of End UEs, if at least one of the PC5 link is released, the 5G ProSe Layer-3 UE-to-UE Relay(s) in the path may release other PC5 link(s) in the path.

-	For Ethernet communication, via multi-hop 5G ProSe Layer-3 UE-to-UE Relay, each 5G ProSe Layer-3 UE-to-UE Relay maintains the association between PC5 links and Ethernet MAC addresses received from the 5G ProSe Layer-3 End UE.

# * * * * Next change * * * *

#### 6.7.X.2	5G ProSe Communication via Multi-hop 5G ProSe Layer-3 UE-to-UE Relay for PDU type Ethernet and Unstructured

Figure 6.7.X.2-1: multi-hop Layer-3 UE-to-UE Relay connection setup for Non-IP Type PDU

1. 5G ProSe Layer-3 End UEs (step 1a) and 5G ProSe Layer-3 UE-to-UE Relays (step 1b) are authorized for multi-hop UE-to-UE Relay service as End UE and as UE-to-UE Relay, respectively, and are provisioned with parameters as specified in clause 6.2.

2. Source End UE may discover and select a multihop path to target End (here, via UE-to-UE Relay1 and UE-to-UE Relay2) as specified in clause 6.3.2.X.

3. Source End UE sends a Direct Communication Request message to U2U Relay1 based on path information acquired in step 2. The parameters included in the Direct Communication Request message are described in clause 6.4.3.7.x.

If there is already a PC5 link for the the same RSC between Source End UE and UE-to-UE Relay1, Source End UE may send a a Link Modification Request message as described in clause 6.4.3.7.x.

4. If needed, security association may be established between Source End UE and U2U Relay1.

5. UE-to-UE Relay1 sends a Direct Communication Request or Link Modification Request message to UE-to-UE Relay2 based on path information received in step 3. The parameters included in DCR or LMR are described in clause 6.4.3.7.x

6. If needed, security association may be established between U2U Relay1 and U2U Relay2.

When there are more U2U Relays are involved for multihop path, step 5 and 6 repeat between U2U Relays.

7. UE-to-UE Relay2 sends a Direct Communication Request or Link Modification Request message to target End UE based on path information. (here, based on path information, UE-to-UE Relay2 is the UE-to-UE Relay directly accessible to target End UE).  The parameters included in DCR or LMR are described in clause 6.4.3.7.x.

8. If needed, security association may be established between U2U Relay2 and target End UE.

9. The target End UE sends a Direct Communication Accept or Link Modification Accept message to the UE-to-UE Relay2. The parameters included in DCR or LMR are described in clause 6.4.3.7.x.

10. After receiving a Direct Communication Accept or Link Modification Accept message, UE-to-UE Relay2 sends  a Direct Communication Accept or Link Modification Accept message to UE-to-UE Relay1 based on path information. The parameters included in DCR or LMR are described in clause 6.4.3.7.x.

When there are more U2U Relays are involved for multihop path, step 10 repeats between U2U Relays.

11. After receiving a Direct Communication Accept or Link Modification Accept message, UE-to-UE Relay1 sends  a Direct Communication Accept or Link Modification Accept message to source End UE based on path information. The parameters included in DCR or LMR are described in clause 6.4.3.7.x.

# * * * * End of change ****