3GPP TSG-SA2 Meeting #164	S2-2408495

# Maastricht, Netherlands, 19 - 23 August 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1176 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: VFL procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | VFL Generic Clause missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | New Clause Added 5.4 Vertical Federated Learning among NWDAFs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No context related to Vertical Federated learning present in the current TS23.288 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 5.4 Vertical Federated Learning (VFL) among NWDAFs and AF

Vertical federated learning (VFL) is a machine learning technique without exchanging/sharing local data set, wherein the local data set in different VFL Participant for local model training have different feature spaces for the same samples (e.g. UE IDs).  This approach allows the participant to train models collaboratively without sharing local datasets and models.

5GC may support vertical federated learning (VFL), in the following scenarios:

-	VFL among NWDAFs in a single PLMN.

-	VFL between AF and NWDAF(s) in a single PLMN.

For Vertical Federated Learning, there is one NWDAF containing MTLF or an AF acting as VFL training server and multiple NWDAFs containing MTLF/AF acting as VFL training client. For inference based on models trained by VFL. there is one NWDAF containing AnLF or an AF acting as VFL inference server and multiple NWDAFs containing AnLF/AF acting as VFL inference clients

VFL Training Server NWDAF

- An NWDAF/MTLF acting as VFL training server discovers and selects NWDAF/MTLF of AF as VFL training clients for model training based on their NRF profile entries and coordinates the VFL process.

- An NWDAF/MTLF acting as VFL training server triggers sample alignment among the VFL training clients and determines the samples to be used during the training process.

- An NWDAF/MTLF acting as VFL training server allocates a unique VFL model correlation ID which is used to correlate the distributed ML Models which are trained together in the VFL joint model training process and need to be used together in the VFL joint inference process.

- An NWDAF/MTLF acting as VFL training server requests VFL training client NWDAFs/AFs to do local model training and to report intermediate results.

- An NWDAF/MTLF acting as VFL training server also trains a model and may have related input data.

- The NWDAF/MTLF acting as VFL training server aggregates intermediate results from VFL training client(s) based on its own ML model or algorithm.

- The NWDAF/MTLF acting as VFL training server sends the intermediate results to VFL training Clients and requests them to perform an additional training iteration if needed.

VFL Inference Server NWDAF

- The consumer will obtain analytics outputs from a NWDAF/AnLF acting as VFL inference server, which is generated via the VFL inference process between NWDAF/AnLF acting as VFL server and corresponding NWDAF/AnLF and/or AFs acting as VFL client(s).

-	The NWDAF/AnLF acting as VFL inference server determines NWDAF/AnLF and/or AFs acting as VLF inference client(s) based on their NRF profile entries and coordinates the VFL inference process.

-	An NWDAF/MTLF acting as VFL inference server requests VFL inference client NWDAFs/AFs to do local model inference and to report intermediate results. The NWDAF/MTLF acting as VFL inference server provides the VFL model correlation ID to the VFL inference client to enable them to use the models that were trained together for the inference.

VFL Server AF

- An AF acting as VFL server provides required analytics IDs and vendor specific feature information for VFL training clients to the NEF, which based on that information discovers and selects NWDAF/MTLF as VFL training clients for model training based on their NRF entries.

- An AF acting as VFL server coordinates the VFL training process.

- An AF acting as VFL server together with the NEF triggers sample alignment and generates the intersection of samples.

- An AF acting as VFL training server allocates a unique VFL model correlation ID which is used to correlate the distributed ML Models which are trained together in the VFL joint model training process and need to be used together in the VFL joint inference process.

- An AF acting as VFL server requests VFL client NWDAFs/MTLFss via the NEF to do local model training and to report intermediate results.

- An AF acting as VFL server sends the intermediate results to VFL training Client NWDAFs/MTLFs via the NEF and requests them to perform an additional training iteration if needed.

- An AF acting as VFL server also trains a model and may have related input data.

- An AF acting as VFL server aggregates intermediate results from VFL training client NWDAFs/MTLFs based on its own ML model or algorithm.

- An AF acting as VFL server sends the intermediate results to VFL training NWDAFs/MTLFs via the NEF and request them  to perform an additional training iteration if needed.

- An AF acting as VFL server provides required analytics IDs and vendor specific feature information for VFL inference clients to the NEF, which based on that information discovers and selects NWDAF/MTLF as VFL inference clients based on their NRF entries.

- An AF acting as VFL server via the NEF requests VFL inference client NWDAFs to do local model inference and to report intermediate results. The NWDAF/MTLF acting as VFL inference server provides the VFL model correlation ID to the VFL inference client to enable them to use the models that were trained together for the inference.

VFL training Client NWDAF

-	An NWDAF/MTLF acting as VFL training client registers to NRF with its NF profile including VFL capability information (VFL capability type as VFL training Clients and vendor specific supported feature information).

-	An NWDAF/MTLF acting as VFL training client locally trains its own ML Model as tasked by the VFL training server with the available local data set,

-	An NWDAF/MTLF acting as VFL training client reports intermediate results to the VFL training server.

-	An NWDAF/MTLF acting as VFL training client receives intermediate results from VFL training server NWDAF and perform an additional training iteration if needed.

VFL Inference Client NWDAF

-	An NWDAF/AnLF acting as VFL inference client registers to NRF with its NF profile including VFL capability information (VFL capability type as VFL inference Client) and vendor specific supported feature information.

-	An NWDAF/AnLF acting as VFL inference client is triggered to perform inference when receiving a request indicating the applicable analytics ID, vendor specific feature information and VFL model correlation ID, and then determines and retrieves the applicable model based on that information and performs inference for that model based on local input data, and reports an intermediate result to the VFL inference server.

VFL Client AF

-	An AF acting as VFL client is registered at the NRF within the NF profile of the NEF through which it is reachable. The profile information includes the VFL capability information (VFL capability type as VFL Clients and vendor specific supported feature information).

-	An AF acting as VFL client locally trains its own ML Model as tasked by the VFL training server with the available local data set,

-	An AF acting as VFL client reports intermediate results to the VFL training server.

-	An AF acting as VFL client receives intermediate results from VFL training server NWDAF and perform an additional training iteration if needed.

-	An AF acting as VFL client is triggered to perform inference when receiving a request indicating the applicable analytics ID, vendor specific feature information and VFL model correlation ID, and then determines the applicable model based on that information and performs inference for that model based on local input data, and reports an intermediate result to the VFL inference server.

# * * * * End of changes * * * *