3GPP TSG-SA2 Meeting #164	S2-2408498

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0481 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support ProSe U2N Multihop Relay Discovery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 23.700-03 has conclusion that multihop U2N relay will be supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause for multi-hop UE-to-Network Relay discovery with Model B. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | U2N multihop relay feature will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.3.1, 6.3.2.3.3a(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.3.2.3	5G ProSe UE-to-Network Relay Discovery

##### 6.3.2.3.1	General

5G ProSe UE-to-Network Relay Discovery is applicable to both 5G ProSe Layer-3 and Layer-2 UE-to-Network relay discovery for public safety use and commercial services. To perform 5G ProSe UE-to-Network Relay Discovery, the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay are pre-configured or provisioned with the related information as described in clause 5.1.

In 5G ProSe UE-to-Network Relay Discovery, the UEs use pre-configured or provisioned information for the relay discovery procedures as defined in clause 5.1.4.1.

The Relay Service Code (RSC) is used in the 5G ProSe UE-to-Network Relay discovery, to indicate the connectivity service the 5G ProSe UE-to-Network Relay provides to the 5G ProSe Remote UE. The RSCs (including the dedicated one(s) for emergency service) are configured on the 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE as defined in clause 5.1.4. The 5G ProSe UE-to-Network Relay and the 5G ProSe Remote UE are aware of whether a RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service and whether a RSC is for emergency service, based the policy as specified in clause 5.1.4. A 5G ProSe UE-to-Network Relay supporting multiple RSCs can advertise the RSCs using multiple discovery messages, with one RSC per discovery message.

NOTE:	When the 5G ProSe UE-to-Network Relay can advertise the emergency RSC(s) is specified in clause 5.4.4.

For multi-hop 5G ProSe UE-to-Network Relay, the following parameters may be used additionally:

- Maximum number of hops which is used to indicate the allowed maximum number of hops per RSC.

- Hop Count which is used to indicate the number of hops from Remote UE or from 5G ProSe UE-to-Network Relay.

- Cumulative Delay information which is used to indicate sum of delay at each PC5 Hop from Remote UE to Intermediate Relay which transmits the 5G ProSe UE-to-Network Discovery message.

- Path information which is used to indicate the Intermediate Relays which are involved for relaying 5G ProSe UE-to-Network Discovery message from Remote UE or from 5G ProSe UE-to-Network Relay.

Additional information not directly used for discovery can also be advertised using the PC5-D protocol stack in single or separate discovery messages of type "Relay Discovery Additional Information" as defined in clause 5.8.3.1.

##### 6.3.2.3.2	Procedure for 5G ProSe UE-to-Network Relay Discovery with Model A

Depicted in Figure 6.3.2.3.2-1 is the procedure for 5G ProSe UE-to-Network Discovery with Model A.

Figure 6.3.2.3.2-1: 5G ProSe UE-to-Network Relay Discovery with Model A

1.	The 5G ProSe UE-to-Network Relay sends a UE-to-Network Relay Discovery Announcement message. The UE-to-Network Relay Discovery Announcement message contains the Type of Discovery Message, Announcer Info and RSC and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

For 5G ProSe Layer-3 UE-to-Network Relay, the 5G ProSe Layer-3 UE-to-Network Relay shall only include a RSC in the UE-to-Network Relay Discovery Announcement when the S-NSSAI associated with that RSC belongs to the Allowed NSSAI of the UE-to-Network Relay.

The 5G ProSe Remote UE (1 to 3) determines the Destination Layer-2 ID for signalling reception. The Destination Layer-2 ID is configured with the UE(s) as specified in clause 5.1.4.1.

5G ProSe Remote UE (1 to 3) monitors announcement messages with the 5G ProSe UE-to-Network RSC corresponding to the desired services.

Optionally, the 5G ProSe UE-to-Network Relay may also send Relay Discovery Additional Information messages as defined in clause 6.5.1.3. The parameters contained in this message and the Source Layer-2 ID and Destination Layer-2 ID used for sending and receiving the message are described in clause 5.8.3.

The 5G ProSe Remote UE selects the 5G ProSe UE-to-Network Relay based on the information received in step 1.

NOTE:	Access Stratum layer information used for 5G ProSe UE-to-Network Relay selection is specified in RAN specifications.

##### 6.3.2.3.3	Procedure for 5G ProSe UE-to-Network Relay Discovery with Model B

Depicted in Figure 6.3.2.3.3-1 is the procedure for 5G ProSe UE-to-Network Relay Discovery with Model B.

Figure 6.3.2.3.3-1: 5G ProSe UE-to-Network Relay Discovery with Model B

1.	The 5G ProSe Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message. The 5G ProSe UE-to-Network Discovery Solicitation message contains the Type of Discovery Message, Discoverer Info, RSC and optionally Target Info and is send using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3. The 5G ProSe Remote UE discovering a 5G ProSe UE-to-Network Relay sends a solicitation message with the RSC which is associated to the desired connectivity service. The RSC is based on the Policy/Parameters specified in clause 5.1.4.1.

How the 5G ProSe UE-to-Network Relays (1 to 3) determine the Destination Layer-2 ID for signalling reception is specified in clause 5.8.3. The Destination Layer-2 ID is configured with the UE(s) as specified in clause 5.1.4.1.

2.	If the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.4.1, of a 5G ProSe UE-to-Network Relay, and the Target Info contained in the solicitation message, if any, matches the 5G ProSe UE-to-Network Relay, the 5G ProSe UE-to-Network Relays (e.g. 1 and 2) respond to the 5G ProSe Remote UE with a UE-to-Network Relay Discovery Response message. The 5G ProSe UE-to-Network Relay Discovery Response message contains the Type of Discovery Message, Discoveree Info and RSC and is sent using the Source Layer-2 ID and Destination Layer-2 ID as described in clause 5.8.3.

For 5G ProSe Layer-3 UE-to-Network Relay, the 5G ProSe UE-to-Network Relay shall only respond to a matching RSC in the UE-to-Network Relay Discovery Solicitation message when the S-NSSAI associated with that RSC belongs to the Allowed NSSAI of the 5G ProSe UE-to-Network Relay.

The 5G ProSe Remote UE selects the 5G ProSe UE-to-Network Relay based on the information received in step 2.

##### 6.3.2.3.3a	Procedure for 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B

Figure 6.3.2.3.3a-1: 5G ProSe multi-hop UE-to-Network Relay Discovery with Model B

- The 5G ProSe Remote UE sends a 5G ProSe UE-to-Network Relay Discovery Solicitation message which includes, additional to the parameters described in clause 6.3.2.3.3 of TS 23.304 [4], maimum number of hops, and an initialized hop counter.

The Source Layer-2 ID is self-assigned by the Remote UE and the Destination Layer-2 ID is set to a Default Destination Layer-2 ID.

- When Intermediate Relay (here Intermediate Relay 1) receives UE-to-Netowork Relay Discovery Solicitation message, it may check whether RSC in the message matches, hop count is less than the maximum number of hops, and User Info ID of itself in order to check duplication of discovery message. When Intermediate Relay forward UE-to-Network Relay Discovery Solicitation message, it may include User Info ID of itself in the path info, Hop Count incremented by 1.

Cumulative delay may be added or updated in the 5G ProSe UE-to-Network Discovery Solicitation message sent by Intermediated Relay(s).

The Solicitation message is propagated by the 5G ProSe Intermediate Relay(s) with (pre)configured RSC(s) matching the RSC in the message while updating the multi-hop counter until the maximum number of hops is reached, in which case the message is dropped. To avoid loops and allow the routing of the Discovery Response, each Intermediate Relay includes its User Info ID in the path info of the message.

The Source Layer-2 ID is self-assigned by the Intermediate Relay and the Destination Layer-2 ID is set to a Default Destination Layer-2 ID.

The 5G ProSe UE-to-Network Discovery Solicitation message contains the Type of Discovery Message, Discoverer Info, the maximum number of hops and optionally Target Info. The Target Info may contain the User Info of UE-to-Network Relay. The maximum number of hops can be a constant value or a variable similar to TTL which will be decreased by 1 per hop.

- When 5G ProSE UE-to-Network Relay(s) receives 5G ProSE UE-to-Network Relay Discovery Solicitation message, it may check RSC contained in the Solicitation message matches the 5G ProSe UE-to-Network Relay configuration and Hoc Count is less than the maximum number of hop and send a Response which is relayed back to the Remote UE by the Intermediate Relays. The Response message may include the Hop Count, path info, and calculated end to end delay.

When 5G ProSE UE-to-Network Relay receives multiple 5G ProSE UE-to-Network Relay Discovery Solicitation messages from Remote UE via multi-hop path of different Intermediate Relays, it may select a solicitation message based on link quality, hop count, and cumulative delay,

- When a Intermediate Relay receive a 5G ProSE UE-to-Network Relay Discovery Response message from 5G ProSE UE-to-Network Relay or Intermediate Relay, the Intermediate Relay relay the received Response message to the Intermediate Relay at next hop or to the Remote UE according to the path information included in the Response message.

When receiving a 5G ProSe UE-to-Network Relay Discovery Response message including User Info ID of 5G ProSe UE-to-Network Relay, the Remote UE store the path information associated to the 5G ProSe UE-to-Network Relay. And the Remote UE may use the stored path information to setup end-to-end connection with the UE-to-Network Relay via multi-hop.

# * * * * End of changes * * * *