3GPP SA WG2 Meeting #164	S2-2408505

Maastricht, Netherlands, August 19 - 23, 2024

Source:	Nokia

Title:	Available Data rate definition for KI#9 Conclusion

Document for:	Approval

Agenda Item:	19.3.1

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract of the contribution: This pCR defines the Available Data rate in more details for the KI#9 conclusion.

# Discussion

Currently, there is an interim agreement for XRM Key Issue #9 concluded for the normative work:

For GBR QoS Flow, exposure of available data rate:

The NG-RAN may be requested to provide the available data rate for GBR QoS Flow to UPF. The NG-RAN determines the available data rate of a GBR QoS flow and reports it to PSA-UPF via the GTP-U header of UL packets. The PSA-UPF may be requested to expose the available data rate to an AF directly or via NEF.

Observation 1: The intention is to report the measured and the near future available data rate of the GBR QoS flow in the NG-RAN in the dynamic conditions. For a GBR QoS flow, only the GFBR is guaranteed, by definition, and any data rate above the GFBR is treated as best-effort.

Proposal 1: Define the available data rate for a GBR QoS Flow based on the GFBR of the discrete set of Alternative QoS profiles (AQP) agreed between AF and 5GS.

NG-RAN tries to always guarantee the desired GFBR, but if it can’t fulfil the request due to some dynamic network conditions, it can select an alternative QoS profile and report/expose the GFBR of the alternative QoS profile to AF. In case the AQPs are not configured, the NG-RAN reports the GFBR rate of the GBR QoS FLow.

# Proposal

Update the Conclusion for KI#9 in TR 23.700-70 as follows:

# * * * * First change * * * *

## 8.9	Agreements for Key Issue #9

The following aspects are concluded for the normative work:

-	For GBR QoS Flow, exposure of available data rate:

-	Based on the AF requests, the PCF instructs SMF to enable the available data rate report and exposure for a GBR QoS Flow.

-	Following the instruction of SMF:

-	The NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes, based on mechanisms defined in clause 5.8.2.17 of TS 23.501 [2], the available data rate to the AF directly or via the local NEF.

The available data rate is defined based on the GFBR values of the discrete set of Alternative QoS profiles (AQP) agreed between AF and 5GS. The NG-RAN selects the QoS profile it can serve and reports the GFBR value of the selected QoS profile. In case the AQPs are not configured, the NG-RAN reports the GFBR rate of the GBR QoS Flow.

NOTE:	It is recommended that the available data rate exposure is not reported very frequently.

Editor's note:	This may be refined based on the potential feedback from RAN WG3.

*** END of changes ***