3GPP TSG-WG SA2 Meeting #164	S2-2408515

# Maastricht, Netherlands, August 19 – 23, 2024	 (revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1178 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New analytics ID “UE signalling” to support Network Abnormal Behaviour Mitigation and Prevention |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed in TR 23.700-84, NWDAF supports assistance to signalling storm mitigation and prevention by statistically learning the expected normal level of signalling and identifying a significant deviation indicating a signalling storm caused by massive signalling of UEs. It also agreed: - It will be determined in the normative phase whether to define a new Analytics ID or enhance an existing Analytics ID to support Signalling storm Mitigation and Prevention caused by massive signalling of UEs. Since there is UE related analytics in clause 6.7 in TS 23.288, it proposes to add a new analytics ID “UE signalling” which can be regarded as one of UE related analytics, to help prevent and imitigate the signalling storm caused by massive UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New “UE signalling storm” analytics is added in “UE related anlytics”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for signalling storm mitigation and prevention caused by massive signalling of UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1, 6.7.6 (All New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.7.1	General

This clause specifies the UE related analytics which can be provided by NWDAF:

-	UE mobility analytics;

-	UE communication analytics;

-	Expected UE behavioural parameters related network data analytics;

-	Abnormal behaviour related network data analytics; and

-  UE signalling analytics.

The NWDAF service consumer may request for these analytics separately, or in a combined way. As an example, an NWDAF service consumer may learn from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3 of TS 23.502 [3], by requesting analytics for both UE mobility (see clause 6.7.2) and for UE communication (see clause 6.7.3).

Depending on local regulations, the NWDAF retrieves user consent for the UE with UDM prior to data collection as defined in clause 6.2.2.2 or clause 6.2.2.3. If user consent to collect data is not granted by the UE, the NWDAF rejects/cancels any analytics subscriptions to any of the UE related analytics with target for analytics set to the SUPI or GPSI of that UE. If the target for analytics is either an Internal or External Group Id or a list of SUPIs or "any UE", the NWDAF skips those SUPIs that do not grant user consent for the purpose of analytics or model training.

NOTE:	Possible uses of such analytics is for the AMF to learn about expected UE behaviour to derive appropriate MICO mode configuration, or for an AF to learn about expected UE behaviour to further provision 5GC with appropriate UE parameters.

# * * * * Second change (All new) * * * *

### 6.7.6	UE signalling analytics

6.7.6.1	General

The NWDAF provides “UE signalling” analytics to assist network to be aware of when the massive signallings from UEs will emerge and distinguish abnormal signallings of UEs (e.g. repeat registrations of IoT devices, redcap devices, etc.), so that the network can take actions to prevent and mitigate potential signalling storm. The signalling requests, timer and state transition related information can be collected from NFs, AF as input for the NWDAF to provide statistics and predictions.

The consumer of this analytics may be AMF, SMF, AF.

The consumer may indicate in the request:

-	Analytics ID = " UE Signalling ".

-	Target of Analytics Reporting: a group of UEs (i.e. Internal, External Group Ids, SUPI/GPSI range);

-	Analytics Filter Information optionally containing:

- Application ID;

- Request type;

- Timer type;

- State transition type;

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics.

#### 6.7.6.2	Input Data

The NWDAF collects UE related signalling information, timer information and state transition information from AMF and SMF as listed in Table 6.7.6.2-1 and Table 6.7.6.2-2.

Table 6.7.6.2-1: Input Data collected from AMF

| Information | Source | Description |
| --- | --- | --- |
| UE group ID or SUPI range | AMF | Identifies a group of particular UEs, or a SUPI/GPSI range of particular UEs |
| Signalling Information | AMF | Signalling information per UE group |
| > Type of requests from UE | AMF | Identifies the request type for N1 interface, e.g. initial registration request, update registration request, mobility and periodic registration request, service request, etc. |
| > Number of requests from UE | AMF | The number of requests for N1 interface per request type. |
| > Timing of request | AMF | Max/min/variance of inter-request periods |
| > Number of successful responses | AMF | The number of successful responses to their corresponding requests. |
| > Number of failed responses | AMF | The number of failed responses to their corresponding requests. |
| > Reason of failed responses | AMF | The reasons of failed responses to their corresponding requests |
| Timer List | AMF | The list description about timers setting for the UE(s) |
| > Type of timer | AMF | The type of timer which has been set for the UE(s), e.g. Periodic Registration Update timer, Back-Off timer, Wait timer. |
| > Timestamp | AMF | Time stamp when the timer starts. |
| > Timer duration | AMF | The time duration that the timer will last. |
| State transition information | AMF | UE related state transition information such as transition type, frequency of CM state changes etc. State transition identifier: - "Access Type change to 3GPP access"; - "Access Type change to non-3GPP access"; - "RM state change to RM-DEREGISTERED"; - "RM state change to RM-REGISTERED "; - "CM state change to CM-IDLE"; - "CM state change to CM-CONNECTED"; - "Handover"; or - "Mobility Registration Update". |

Table 6.7.6.2-2: Input Data collected from SMF

| Information | Source | Description |
| --- | --- | --- |
| UE group ID or SUPI/GPSI range | SMF | Identifies a group of particular UEs, or a SUPI/GPSI range of particular UEs |
| Signalling Information | SMF | Signalling information per UE group |
| > Number of requests for UE SM Policy Update | SMF | Number of requests from PCF to update UE SM Policy, such as QoS, PCC Rules, etc. |
| > Number of failed responses | SMF | The number of failed responses to their corresponding requests. |
| > Reason of failed responses | SMF | The reason of failed responses to their corresponding requests. |
| State transition information | SMF | PDU Session related state transition information such as transition type, frequency SM state changes, etc. State transition identifier: - "PDU Session Establishment"; - "PDU Session Release"; - "Communication failure"; or - "PLMN change". |
| Timer List | SMF | The list description about timers setting for the PDU session(s) of the UE |
| > Type of timer | SMF | The type of timer which has been set for the UE(s), e.g. Back-Off timer |
| > Timestamp | SMF | Time stamp when the timer starts. |
| > Timer duration | SMF | The time duration that the timer will last. |

The NWDAF collects user active/inactive information from AF as listed in Table 6.7.6.2-3.

Table 6.7.6.2-3: Input Data collected from AF

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | Identifies the application providing this information. |
| User activation time information (1…max) | AF | Information of activation time for the users (e.g. IoT users) per application. |
| > Active Time | AF | The time stamp of active time for the users per application. |
| > Inactive Time | AF | The time stamp of active time for the users per application, if applicable. |
| > UE group ID or SUPI/GPSI range | AF | Identifies a group of UEs, e.g. IoT users, internal group ID, external group ID, or a SUPI/GPSI range with the same timer. |

6.7.6.3	Output analytics

The NWDAF provides UE signalling analytics on potential cause of signalling storm with its corresponding network data to consumer NF. The analytics results provided by NWDAF include the UE signalling statistics as defined in Table 6.7.6.3-1 or predictions as defined in Table 6.7.6.3-2.

Table 6.7.6.3-1: UE Signalling statistics

| Information | Description |
| --- | --- |
| List of Report (1..max) | List of observed signalling storm or abnormality report. |
| Cause of Signalling Storm | Possible causes of potential signalling storm, e.g. massive signallings, repeat signallings, low response rate, frequency transition change, etc. NOTE1 |
| Group ID or SUPI/GPSI Range | Identifies the UEs for which the statistics applies by Internal Group ID or SUPI range. |
| > Signalling information | The statistics of signalling information. |
| >> UE Related Request Type | The type of the request related to the UE(s). |
| >> Number of UE Request | The number of request message per request type. |
| >>Timing of request | Max/min/variance of inter-request periods |
| >> Number of successful responses | The number of successful responses to their corresponding requests. |
| >>Number of failed responses | The number of failed responses to their corresponding requests. |
| >> Reason of failed responses | The reasons of failed responses to their corresponding requests |
| > Timer List | The list of timer information |
| >> Type of timer | The type of timer which has been set for the UE(s), e.g. Periodic Registration Update timer, Back-Off timer, Wait timer. |
| >> Timestamp | Time stamp when the timer starts. |
| >> Timer duration | The time duration that the timer will last. |
| > State transition information | The statistics of UE related state transition information |
| >> State transition identifier | The state transition change situation. |
| >> Frequency of state transition change | The times of state transition change. |
| NOTE 1: Cause of signalling storm can be derived by NWDAF as a general cause and each cause can be provided with its corresponding subsets. |  |

Table 6.7.6.3-2: UE Signalling prediction

| Information | Description |
| --- | --- |
| List of Report (1..max) | List of observed signalling storm or abnormality report. |
| Cause of Signalling Storm | Possible causes of potential signalling storm, e.g. massive signallings, repeat signallings, low response rate, frequency transition change, etc. NOTE1 |
| Group ID or SUPI/GPSI Range | Identifies the UEs for which the statistics applies by Internal Group ID or SUPI range. |
| > Signalling information | The predictions of signalling information. |
| >> UE Related Request Type | The type of the request related to the UE(s). |
| >> Number of UE Request | The number of request message per request type. |
| >> Timing of request | Max/min/variance of inter-request periods |
| >> Number of successful responses | The number of successful responses to their corresponding requests. |
| > Number of failed responses | The number of failed responses to their corresponding requests. |
| > Reason of failed responses | The reasons of failed responses to their corresponding requests |
| > Timer List | The list of timer information |
| >> Type of timer | The type of timer which has been set for the UE(s), e.g. Periodic Registration Update timer, Back-Off timer, Wait timer. |
| >> Timestamp | Time stamp when the timer starts. |
| >> Timer duration | The time duration that the timer will last. |
| > State transition information | The predictions of UE related state transition information |
| >> State transition identifier | The state transition change situation. |
| >> Frequency of state transition change | The times of state transition change. |
| > Confidence | Confidence of the prediction |
| NOTE 1: Cause of signalling storm can be derived by NWDAF as a general cause and each cause can be provided with its corresponding subsets. |  |

6.7.6.4	Procedures

Figure 6.7.6.4-1 depicts the procedure for UE signalling analytics provided by NWDAF.

1.	The consumer NF subscribes to “UE signalling analytics” by invoking Nnwdaf_AnalyticsInfo or Nnwdaf_MLModelProvision_Subscribe service (Analytics ID = UE signalling, Target of Analytics Reporting = Internal Group ID or External Group ID or SUPI/GPSI range, Analytics Filter Information as described in clause 6.7.6.1).

2.	The NWDAF subscribes to the input data as described in Table 6.7.6.2-1 by invoking Namf_EventExposure_Subscribe service for collecting signalling information, timer information and state transition information for the group of UEs.

3.	The NWDAF subscribes to the input data as described in Table 6.7.6.2-2 by invoking Nsmf_EventExposure_Subscribe service for signalling information, timer information and state transition information for the group of UEs.

4.	The NWDAF subscribes to the input data from AF in the Table 6.7.6.2-3 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service for collecting user active/inactive time information.

5.	The NWDAF derives requested analytics with the collected data. Analytics output parameters are listed in Table 6.7.6.3-1 and Table 6.7.6.3-2. The NWDAF provides the requested analytics to the NF by either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service, depending on the service used at step 1.

# * * * * End of changes * * * *