3GPP TSG|WG-SA2 Meeting #164	S2-2408519

Maastricht, Netherlands, August 19 – 23, 2024	(revision of S2-20xxxx)

Source:	China Telecom

Title:	Discussion on Ranging/SL Positioning Usage Information Reporting

Agenda Item:	30.2

Document for:	Information

# 1.	Background

Background in SA5

SA5 has studied the charging aspects of Ranging and Sidelink Positioning in TR 28.845, which identified the business roles, potential charging scenarios and requirements, and developed and evaluated the potential solutions for the charging aspects for Ranging and Sidelink Positioning. For the normative work, five work tasks in S5-241827 have been proposed and agreed in SA5 #154 meeting as shown below:

The objective of this work item is to specify the business relationships, requirements and solutions for Ranging and Sidelink Positioning Charging. Specifically, the areas of work for SA5 include:

-	WT-1: Support of architecture for converged charging for Ranging and Sidelink Positioning

-	WT-2: Support of converged charging for UE positioning assisted by Sidelink Positioning and involving 5GC

-	WT-3: Support of converged charging for Ranging/SL Positioning service exposure

These work tasks depend on WID in SA2 to be agreed:

-	WT-4: Support of converged charging for Ranging/Sidelink Positioning UE Discovery

-	WT-5: Support of converged charging for UE only Sidelink Positioning for Target UE using Located UE

According to the conclusion in TR 28.845, the GMLC is agreed to embed the CTF to receive the Ranging/SL Positioning usage information in all above five cases and it is agreed that 5GS may collect the charging information in all above five cases as follow:

| Information Element | Category | Description |
| --- | --- | --- |
| Ranging and Sidelink Positioning Information | OC | This field contain UE parameters in procedure of Ranging and Sidelink Positioning |
| Target UE ID | OC | The identity of Target UE in Ranging/Sidelink positioning |
| SL Reference UE ID | OC | The identity of SL Reference UE in Ranging/Sidelink positioning |
| SL Positioning Server UE ID | OC | The identity of SL Positioning Server UE in Ranging/Sidelink positioning |
| Located UE ID | OC | The identity of Located UE in Ranging/Sidelink positioning |

For WT-1, WT-2 and WT-3, these three work tasks are 5GC NF(s) involved scenarios where the GMLC may get the the Ranging and Sidelink Positioning Usage Information as the procedures defined in TS 23.586, and the GMLC(CTF) further sends the charging information to CHF. However, WT-4 and WT-5 are no 5GC NF(s) involved scenarios where the related UEs do not have mechanism to report Ranging and Sidelink Positioning Usage Information to network as SA2 definition. Thus, WT-4 and WT-5 are removed in S5-241827 in SA #104 and they are waiting for the conclusion from SA2.

Observation 1: SA5 has agreed the conclusion that the Ranging and Sidelink Positioning Usage Information should be reported to GMLC in all Ranging and Sidelink Positioning charging cases.

Observation 2: For no 5GC NF(s) involved scenarios (i.e. WT-4 and WT-5), SA5 is waiting for the definition from SA2 on how UE deliver Ranging and Sidelink Positioning Usage Information to GMLC an d continues normative work on WT-4 and WT-5.

Background in SA2

In Ranging/Sidelink Positioning, three operations are defined in TS 23.586 as followed:

-	In the Network-based Operation, 5GC NF(s) is involved for the service request handling and result calculation.

-	In the UE-only Operation, the service request handling and result calculation are performed by UE.

-	In the Network-assisted Operation, 5GC NF(s) is involved for the service request handling and assist UE for the result calculation.

In the Network-based Operation and the Network-assisted Operation which are 5GC NF(s) involved scenarios, the Namf_Location_ProvidePositioningInfo service operation can be used by AMF to deliver the Ranging and Sidelink Positioning Usage Information to GMLC as described in clause 6.20 in TS 23.273.

Observation 3: In 5GC NF(s) involved scenario (i.e. Network-based Operation and Network-based Operation), the UE can deliver the Ranging/SL Positioning Usage Information to GMLC through AMF using existing procedures.

In the UE-only operation, the service request handling and result calculation are performed by UE. After receiving the service request from SL Positioning Client UE or application layer, Target UE may discover and select SL Reference UE(s) and SL Positioning Server UE if necessary, then the Target UE or SL Positioning Server UE calculates location or ranging information by information exchange with SL Reference UE(s). In this case, target UE does not need to interact with any 5GC NF. Thus, no related interface is defined in SA2 to deliver the Ranging/SL Positioning Usage Information from UE.

During the Ranging/Sidelink Positioning UE Discovery, as described in clause 6.4 in TS 23.586, 5GC NF(s) is not involved either. Similarly, the Target UE which is responsible for the discovery of related UEs cannot deliver such Ranging/SL Positioning Usage Information to the network.

Observation 4: In no 5GC NF(s) involved scenario (i.e. the UE-only operation, UE Discovery), the UE cannot deliver Ranging/SL Positioning Usage Information to network.

In addition, regarding to charging part, here is an Editor’s Note in TS 23.586:

Editor's note:	The charging solutions for the support of Ranging based services and Sidelink Positioning is expected to be defined by SA5, and SA2 will make the alignment based on the outcome of the SA WG5 work.

# 2.	Discussion

Given that the no 5GC NF(s) involved scenario (i.e. the UE-only operation, UE Discover) in Ranging/SL Positioning service uses the similar mechanism as the scenario in ProSe (i.e. ProSe Direct Communication and ProSe UE-to-Network Relay Discovery), the mechanism of usage information reporting for charging purpose in ProSe might be also applied in Ranging/SL Positioning service. In ProSe, the DDNMF is used to provide the necessary charging functionality or charging information for the usage of 5G ProSe Direct Discovery and/or ProSe Direct Communication as described in TS 23.304, for example, receiving the usage report from UE over the user plane and using the PC3a protocol. Considering most charging users are 5G ProSe capable UEs in Ranging/SL service, the DDNMF may also be reused in Ranging/SL service for charging purpose. Specifically, the UE may report Ranging/SL Positioning Information to DDNMF as the method defined in ProSe. From the SA2 perspective, the potential gap is the DDNMF might need to further deliver the Ranging/SL Positioning Information to GMLC, since the CTF is embedded in DDNMF in ProSe while the CTF is embedded in GMLC in Ranging/SL service as the observation 1.

Based on the above description, this paper proposes two options as way forward:

- Option 1: Totally reuse the ProSe charging mechanism to address the issue described in Observation 4.

Action: A CAT “F” CR is proposed in S2-2408518 for defining charging part in no 5GC NF(s) involved scenario of Ranging/SL Positioning service can refer to the charging part in ProSe.

- Option 2: Reuse part of DDNMF mechanism in ProSe for Ranging/SL Positioning charging purpose and enhance DDNMF to deliver Ranging/SL Positioning Information to GMLC, to address the issue described in Observation 2 and Observation 4.

Action: A new TEI19 is proposed in S2-2408520 mainly for DDNMF enhancement to deliver Ranging/SL Positioning Usage Information to GMLC.

In option 1, it is assumed that the CTF is embedded in DDNMF in no 5GC NF(s) involved scenario in Ranging/SL service, which is conflict with the conclusion of the study in SA5 as described in Observation 1. Another potential risk is Application Layer ID cannot be mapped to GPSI, resulting in the charging system being unaware of the available UE identifiers, considering the GMLC is responsible for performing Application Layer ID to GPSI resolution in Ranging/SL service instead of DDNMF. Thus. Option 2 is proposed and preferred.

# 3.	Proposal

To ensure charging for Ranging/SL service properly and SA5 charging group can continue their normative work, it proposes to select one option and its corresponding action as a way forward from above two options in SA2 and Option 2 is preferred.