3GPP TSG-WG SA2 Meeting #164	S2-2408530

Masstricht, NL, 19-23 August, 2024	(revision of S2-240xxxx)

Source:	Qualcomm

Title:	Conclusions for KI #3

Document for:	Approval

Agenda Item:	19.10.1

Work Item / Release:	FS_UAS_Ph3 / Rel-19

Abstract: This pCR add conclusions for KI#3 in TR 23.700-59

# Introduction

This paper proposes conclusion for solutions of KI#3.

# Discussion

Based on the NWM responses and the LS response from RAN, this contribution proposes final conclusion by suggesting solutions for the pending scenarios which are following.

1: Provisioning of NTZ information to UAV UE.

The TR contains multiple options documented in the interim conclusion on how the NTZ information is provided to UAV UE.

Successful deployment of NTZ support by MNOs will depend largely on the complexity and cost for the MNO in deploying any modifications required to support NTZs. Given the resistance from MNO in deploying any form of enforcement for NTZs, we believe minimizing the impact on the MNO network is a key criteria for the successful deployment of solutions for provisioning of NTZ information to the UAV UE. Therefore, solutions that require AMF/MME to be aware on NTZ configuration information are less desirable since they impact a large number of nodes in the MNO network (both in terms of needed upgrades and distribution of configuration information). Therefore, solution where the PCF provides the information to the UAV UE are preferred.

Proposal 1: It is proposed that NTZ are received by the MNO at PCF (via OAM or NEF) and distributed by PCF to the UAV UEs.

2: Provisioning of NTZ information to 5GC.

NTZ information is not expected to be very variable and dynamic, and to be applicable to all UAV UEs without the need to have separate NTZ information for different types of UAV UEs. Therefore, we see no need for solutions based OAM configuration of NTZ information to the MNO.

Proposal 2: It is proposed that OAM configures the NTZ information to the MNO.

Scenario 3: NTZ enforcement by network.

The interim conclusions in the TR already indicate that NTZ enforcement is responsibility of the UAV UE based on  configured NTZ information.

What remains undecided is what actions the network takes wrt a connected mode or idle mode UE entering an NTZ. There are proposals to either provide a reject cause to the UE or to have the network (AMF) detect the UAV entering the NTZ and having the AMF informs SMF to release the PDU and then deregister the UAV.

Regarding deregistering the UAV with an appropriate cause code, if this happens once the UE is inside the NTZ, this automatically violates the concept of NTZ because the MNO will be transmitting to the UAV UE in the NTZ, and thus such solution is not feasible in terms of respecting the NTZ. It is more reasonable and practical to ensure, via appropriate regulations or standards outside the scope of 3GPP, to ensure that UAV UEs that intend to operate in a certain region are configured appropriately before entering operations. This is, by the way, already true for other aviation aspects and thus it is not a stretch at all to assume certified and registered (with CAA) UAV UEs will have such configuration by the UAS operator before entering operations. However, if we want to cater for error cases, we could allow a specific de-registration cause for a UAV UE that is performing registration inside an NTZ.

Regarding having the network detect the UAV UE entering an NTZ and then releasing the related PDU sessions and de-registering the UAV UE, there are two aspects to consider:

- releasing the PDU session implies that the UAV UE will need to perform SM signalling to re-establish the PDU session, which will most probably include performing a UUAA procedure, even if the NTZ are is very limited. This means that in order for the UAV UE to return in contact with USS/UTM for C2, telemetry, and so on, the UAV UE needs to re-establish the PDU session from scratch while in flight, which may be time consuming since it will require re-executing a UUAA procedure. De-registering the UAV UE implies that at all effect the UAV UE “disappears” from the USS/UTM “radar” since the UAV UE context is to all effects removed from the MNO network and the USS/UTM, which is highly undesirable.

- de-registering the UAV UE is not required and actually is a harmful solution. The NTZ may be very limited in size, and deregistering the UAV UE means that in order for the UAV UE to return in contact with USS/UTM for C2, telemetry, and so on, the UAV UE needs to reregister from scratch while in flight, which may be time consuming since it will require re-executing a UUAA procedure. De-registering the UAV UE implies that at all effect the UAV UE “disappears” from the USS/UTM “radar” since the UAV UE context is to all effects removed from the MNO network and the USS/UTM, which is highly undesirable.

Observation: A solution where the UAV UE remains registered and the PDU session is maintained with no user plane and radio resources is preferable since this would be a short transient situation.

- detecting the UAV UE via area of interest mechanisms is a feasible solution, though it requires continuous and precise monitoring of the UAV UE even when there may not be a requirement (e.g. for location services) to monitor all UAV UEs.

Observation: an explicit mechanism, e.g. re-using existing mechanisms, where the UAV UE which is aware is about to enter an NTZ can inform the network (AMF) that it is about to enter the NTZ is preferable since it allows the network to know exactly and instantaneously the state of the UAV UE (i.e. incapable of tx and possibly rx) and take appropriate actions. The UAV UE can then reset the status when existing the NTZ.

Proposal: It is proposed that

- when entering and NTZ, the network does not release the PDU session and does not de-register the UAV UE because of its presence in the NTZ

- before entering an NTZ (i.e. the UAV UE is configured with NTZ information and is aware based on its location and the flight plan that it is about to enter an NTZ), the UAV UE informs the network with a registration update that it is about to enter an NTZ, so that the network can suspend the user plane resources for the UAV UE. The UAV UE, upon exiting the NTZ, will perform an additional registration update to inform the network that it is now outside the NTZ.

- in order to cater for error cases, we could allow a specific de-registration cause for a UAV UE that is performing registration inside an NTZ.

# 3. Text Proposal

It is proposed to capture the following changes to TR 23.700-59.

# * * * * First change * * * *

## 8.3	Conclusion for Key Issue #3: Support of No Transmit Zones

### 8.3.1	General

The following principles are agreed to support NTZ.

1)	NTZ enforcement applies to the UE that is a UAV UE (aka an aerial UE) with aerial subscription.

2)	NTZs are supported for both LTE and NR.

3)	An NTZ may map to one or more cells or a fraction of a cell, or overlap different cells in a mobile operator network.

NOTE:	Replanning of existing tracking areas and cells based on the presence of NTZs is not assumed.

4)	No transmissions in the restricted frequency bands are allowed from a UAV UE located in the NTZ (in three dimensions to consider height restrictions), regardless of the service type.

5)	No UAV UE transmits or attempts to transmit in the restricted frequencies, location and height defined by NTZ. But UAV UEs can transmit/receive normally outside of these restrictions.

6)	UAV UEs supporting NTZ regulations are configured with NTZ information, so that these UAV UEs will be aware of the presence of NTZs.

7)	Reception of downlink data from the network by the UAV UE located in the NTZ is allowed given the regulatory requirements are not violated.

### 8.3.2	Configuring/Provisioning NTZ assistance information to UAV UE and NTZ enforcement by UAV UE

The following NTZ assistance information (or NTZ policy) is configured/provisioned in the UAV UE.

-	A list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and height restrictions).

NOTE 1:	It is assumed UAV UE is configured with NTZ assistance information before accessing to the operator's network, if the local regulation requires NTZ.

The above NTZ assistance information is made available to the UAV UE by using one or more following ways:

-	pre-configured;

-

-	provided/updated by the PCF;

-	provided/updated by the AF (e.g. NTZ AF, USS, TPAE).

The UAV UE is responsible for NTZ enforcement independently of the UAV UE 3GPP Release.

The UAV UE supporting NTZ (i.e. Rel-19 and onward UAV UEs) performs the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	upon applying NTZ enforcement, when within the NTZ (both NTZ area and frequency bands) the UAV UE does not transmit any signalling or data.

c)	does not select/reselect any cells impacted entirely by an NTZ.

d) for the network to know about UAV presence inside NTZ, the UAV UE performs a registration update before entering the NTZ and when exiting the NTZ.

NOTE 3:	Whether indication about UAV UE's NTZ support to the AMF/MME is needed will be decided during normative phase. This work does not have RAN impacts.

NOTE 4:	Whether extension of Service Restrictions for NTZs using Restricted Transmission Area (RTA) mapped from the NTZ information is needed will be discussed during normative phase. This work does not have RAN impacts.

In order to cater for scenarios where the UAV UE may not be appropriately configured with NTZ information, the AMF/MME is responsible for NTZ enforcement in 5GC and EPC respectively:

-	reject the registration by AMF with existing cause code #15 "no suitable cells in tracking area" as specified in TS 24.501 [10];

-	reject the attach request by MME with existing cause code #15 "no suitable cells in tracking area" as specified in TS 24.501 [10];

# * * * * End of changes * * * *