3GPP TSG-WG SA2 Meeting #164	S2-2408556

Maastricht, NL, 19th Aug –23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion and analysis on Temporary ID

Document for:	Discussion

Agenda Item:	19.14

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: “This contribution analysed the need to have AIoT Device Temporary ID and proposed not to support it.

1. Introduction

Currently, there is a discussion regarding whether temporary ID is needed for Ambient IoT Device. This discussion paper would like to analysis and evaluate the feasibility, complexity and potential impacts if the temporary ID mechanism is applied to the Ambient IoT Device. Additionally, the proposal is also provided accordingly.

2. Discussion

The writing operation to EEPROM usually consumes much higher power consumption than reading [2], or similar power consumption but with much lower throughput [3]. In the specification of UHF RFID, the mandatory size of a data block to be written into the memory per writing operation (by the “write” command) is only 16 bits [1]. Consequently, it is recommended to avoid frequent writing operation to the memory of Device 1 in the system design, especially for the information block with a certain size.

According to the specification of RFID tag products from Impinj [4], the read sensitivity is up to -25.5 dBm with a dipole antenna, while the write sensitivity is up to -20 dBm with a dipole antenna. In other words, the power consumption of writing operation to non-volatile memory is three times as many as read operation. If all conditions except communication range remain same, shorter communication range is required to ensure reliable write operation compared with read operation.

Observation 1: For the non-volatile memory in Device 1, the writing operation consumes higher power than reading operation.

Observation 2: The communication range for reliable writing operation is expected to be shorter than reading operation.

According to Impinj RFID tag products feature introduction [5], the efficiency of inventory operation can achieve 1000 items per second. In other words, the reading operation (e.g. EPC reporting) can achieve around 0.33 ms per 32 bits (taking 96 bits of EPC as example). However, the data sheet of Impinj RFID tag [6] also mentions that the memory write speed is 3.2 ms per Write, BlockWrite, Lock or Kill operation, for writing up to 32 bits. In other words, the latency of writing operation can be 10 times as much as the latency of reading operation, leading to much lower number of tags to be interacted with a reader per second due to the writing operation (e.g. only tens of tags per second considering the end to end message interactions for the write operation).

Observation 3: The writing operation introduces much larger latency compared with reading operation, leading to significantly lower service operation efficiency.

In addition, NXP product specifications [9] also list variable RFID tag products performance. Among these products, RFID tag with longer EPC or lager user memory will result in lower read or write sensitivity, i.e. more power consumption is required. Impinj product data sheet [6][7][8] also compares the performance between M700 Series RAIN RFID Tag Chips and M700 Series RAIN RFID Tag Chips with Extended Memory. The performance table also shows that the Tag Chips with extended memory could result in lower read sensitivity. This is due to that larger storage requires larger chip size, which increases complexity and power consumption consequently.

Observation 4: Enlarging non-volatile memory will decrease read / write sensitivity, further leading to increasing power consumption, decreasing throughput and increasing latency.

Impinj RFID tag products feature introduction [7][8] also mentions that for some tag products, the tag memory (non-volatile memory) is designed to endure 10,000 write cycles or retain data for 10 years.

Observation 5: The writing operation should be avoided as much as possible to enable a longer life cycle of an Ambient IoT Device.

Currently, there is a discussion in SA2 Ambient IoT regarding whether a temporary ID is needed for Ambient IoT Devices. Temporary ID is introduced for paging identifier and Device ID reporting to address privacy issues during Inventory procedure and it can be short if it is only allocated or used by specific entity (e.g. CN NF or Reader). Below is the analysis for Temporary ID mechanism.

General idea of using Temporary ID:

In general, the essence of a Temporary ID mechanism is to enable both the Ambient IoT Device and network (e.g. CN NF or Reader) to get a same Temporary ID and use this same Temporary ID for paging or for signalling routing. By maintaining the mapping between the Temporary ID and a Permanent ID in the network, the network can derive the Permanent ID to perform Device management.

Complexity of using temporary ID:

The Temporary ID is writable must only be changed by in an authorised and secure way, otherwise there will be a mismatch between the network and the Ambient IoT Device.

Storing a Temporary ID in non-volatile memory requires the Ambient IoT Device to write to NVM. As analysed above, the write operation can have following impacts:

-	more power consumption (3 times as much as read operation)

-	larger latency (around 10 times as much as read operation)

-	shorten the life cycle of an Ambient Device due to limited times of writing operation

Issues or impacts due to using temporary ID:

Assuming that a Temporary ID mechanism is applied to Ambient IoT, additional complexities and issues are introduced:

1.	Procedure for Temporary ID provisioning is introduced:

-	If the Temporary ID provisioning/update is performed during Inventory operation procedure, significantly larger latency is introduced compared with RFID inventory procedure. In RFID system, up to 1000 tags can be inventoried per second. In 3GPP system, if the Ambient IoT Device shall be provisioned with a or have its Temporary ID updated during inventory procedure, it will at least cost 3.2ms latency per 32 bits. Currently, the length of 5G-GUTI is 80 bits (<MCC><MNC><AMF Region ID><AMF Set ID><AMF Pointer><5G-TMSI>). Writing a 5G-GUTI towards an Ambient IoT Device would cost around 10ms (ceil(80 bits / 32 bits) * 3.2 ms = 9.6ms). Considering that the number of Ambient IoT Devices would be much larger than the number of UEs, the length of a Temporary ID would need be longer than a 5G-GUTI. Therefore, the latency would be even larger. This would cause the inventory efficiency being decreased down to 100 tags per second (or even lower), making Ambient IoT much less competitive than RFID, being only 1/10 efficiency of RFID.

-	If this procedure is a standalone procedure, it means that the network shall periodically trigger inventory operation to detect whether there are Ambient IoT Devices which have not been obtained a Temporary ID and to provision/update the Temporary ID to such Ambient IoT Devices. This actually requires the Ambient IoT Device to maintain a state that whether it has been obtained a Temporary ID and determine whether to respond to the network accordingly. This adds additional complexity to the Ambient IoT Device.

2.	Temporary ID re-allocation is performed when Ambient IoT Device is moved to a new area

-	When an Ambient IoT Device is moved to a new area where the network entity (e.g. CN NF or Reader) in that area is not the one allocating the Temporary ID (e.g. different CN NFs or different operators networks), there is no way for the network entity to use the Temporary ID already allocated to the Ambient IoT Device as it is not aware of it. Therefore, the permanent ID shall be used in this case to perform inventory for the Ambient IoT Devices which has no temporary ID allocated by this network entity.

-	After the Ambient IoT Device is inventoried with the permanent ID, the network entity shall provision the Ambient IoT Device with a Temporary ID. In other words, such Temporary ID provisioning does not only happen once for an Ambient IoT Device. On the contrary, it would be triggered multiple times whenever the Ambient IoT Device is moved to a new area. This absolutely increases power consumption and latency, while using permanent ID to perform inventory is still inevitable.

3.	Synchronization between the Ambient IoT Device and the network

-	As mentioned in the above observation, successful writing operation requires shorter communication range. If the Temporary ID is not written to the Ambient IoT Device successfully due to the lack of reliable communication range, of the network uses such Temporary ID to perform operations they will fail. In this case, the network has no information to determine whether the Ambient IoT Device is not in the coverage area or it has not been provisioned the Temporary ID successfully.

-	In this case, using permanent ID for inventory is still required to detect the occurrence of Ambient IoT Device.

4.	The frequency of temporary ID re-assignment

-	When to trigger the update of the Temporary ID has to be considered. 5GS updates the 5G-GUTI whenever the UE enters CM_CONNECTED.

-	If the Temporary ID is used to address privacy concerns, how frequently the Temporary ID is updated shall be considered. Apart from the case when Ambient IoT Device moves to a new area as mentioned above, updating the Temporary ID is needed even there is no mobility for an Ambient IoT. In theory, whenever the Temporary ID is used, the Ambient IoT Devices Temporary ID shall be updated in order to fully address the privacy issue, otherwise the privacy concern may still exist. If so, the impacts would the same as the analysis mentioned in the first bullet of 1. In order words, making Ambient IoT much less competitive to RFID (decreasing at least 90% inventory efficiency).

-	Frequently performing writing operation would shorten the life cycle of the Ambient IoT Device. As mentioned in [7][8], there is a limited number of writing operation. If a Temporary ID mechanism is applied to the Ambient IoT Device, the life cycle of Ambient IoT Device would be much shorter than RFID tags, making it less competitive than RFID. Additionally, the Ambient IoT Device may have to consider Atomicity [10] of the Temporary ID when writing to NVM in 16bit chunks, which can further increase the number of NVM writes and device complexity.

Observation 6: Using Temporary ID adds more complexity to an Ambient IoT Device, increases power consumption and decreases service operation efficiency significantly because additional procedures are required write the Temporary ID.

Observation 7: Applying a Temporary ID mechanism would shorten the life cycle of an Ambient IoT Device due to the limitation of maximum number of writing operation endured by an Ambient IoT Device.

Observation 8: Additional issues are introduced by a Temporary ID (e.g. synchronization between an Ambient IoT Device and network) which have not been addressed.

Addressing the requirements of shorter ID report and privacy issues can adopt other possible solutions.

To enable shorter reporting lengths, variable Device ID lengths can be supported, The Device ID length can be determined based on service requirement which may not require a fixed or long ID. For example, if an enterprise does not require to purchase a large amount of Device for Ambient IoT services, a shorter Device ID can be allocated for its AIoT Device. This is similar with EPC in RFID, which has different fixed length EPCs and variable length EPCs to address different cases. In addition, a truncated ID report can also be used to report only the part of a device ID not included in a MASK/paging. These two methods do not cost additional power consumption and no NVM writing is required.

In terms of privacy issues, this depends on SA1 requirement and SA3 discussion. The other candidate solutions instead of using temporary ID are under discussion of SA3.

Observation 9: Variable Device ID length and truncated Device ID report design can solve the requirement of shorter ID report. There are also alternatives to address privacy issues instead of using temporary ID.

3. Conclusion and proposal(s)

From the above analysis and observation, following proposal is suggested.

Proposal: The temporary ID is not needed for the purpose of paging identifier and shorter ID report. It is assumed that the privacy issue can be addressed, if any, by SA3 based on SA1 requirements without relying on using temporary ID.

4. References [DELETE IF NOT REQUIRED]

- EPC™ Radio-Frequency Identity Protocols Class-1 Generation-2 UHF RFID Protocol for Communications at 860 MHz – 960 MHz

- A 2 Kbits Low Power EEPROM for Passive RFID Tag IC,  Chinese Journal of Electronics (Vol.31, No.1), Jan. 2022

- An ultra low power non-volatile memory in standard CMOS process for passive RFID tags, IEEE Custom Integrated Circuits Conference, Oct. 2009

- IMPINJ M830/M850 SERIES TAG CHIPS TAG CHIP DATASHEET (https://support.impinj.com/hc/article_attachments/30597440310803)

- About RFID and RAIN RFID (https://www.impinj.com/products/technology/about-rfid)

- Impinj M700 Series RAIN RFID Tag Chips with Extended Memory (https://support.impinj.com/hc/article_attachments/11095292602003)

- IMPINJ M700 SERIES TAG CHIP DATASHEET (https://support.impinj.com/hc/article_attachments/30596888296595)

- TAG CHIP DATASHEET IPJ-M780A-A01 IPJ-M781A-A01 (https://support.impinj.com/hc/article_attachments/30027979278611)

- UCODE® RAIN RFID (https://www.nxp.com/products/rfid-nfc/ucode-rain-rfid-uhf:MC_50483)

- [10]	https://en.wikipedia.org/wiki/Atomicity_(database_systems)