SA WG2 Meeting S2#164	S2-2408557

Maastricht, Netherlands, 19 August - 24 August, 2024

Source:	Nokia

Title:	KI#3: Update on QoS handling in Solution #21

Document for:	Approval

Agenda Item:	19.9

Work Item / Release:	FS_eEDGE_5GC_ph3 / Rel-19

Abstract of the contribution: This contribution updates the solution #21 to KI#3

# Introduction

This contribution updates the solution #21 to the KI#3 in the TR 23.700-49.

# 2	Proposal

It is proposed to add following updates to the TR 23.700-49.

# * * * * First change* * * *

#### 6.21.2.3	QoS handling of the tunnel between different DN parts

The QoS requirement of the tunnel is independent from any UE's PDU session, the QoS requirement is defined per DNAI or per application (identified by Application ID). The EDI in Table 6.2.3.4-1 of TS 23.548 [5] is updated to contain the QoS requirement on a DNAI or application basis.

To guarantee the QoS requirement of the tunnel between the L-PSA UPF and the C-PSA UPF, DSCP approach is used.

Specifically, the AF provides to the SMF via NEF on the requested QoS requirement in the EDI. For a given set of QoS parameter values from the QoS requirement, a DSCP value can be mapped which is used by the transport networks between the L-PSA UPF and C-PSA UPF. The L-EAS marks processed traffic packets with the same DSCP value as the original UE traffic packets.

The SMF configures L-PSA UPF and C-PSA UPF to handling the QoS requirements, by defining the Packet Filter Set in PDRs and the outer header in FARs linked to each PDR. PDRs are defined by the SMF for the L-PSA UPF's and C-PSA UPF's N6 packet detection, specifying a target DSCP value as the packet filter set's TOS for IPv4 or Traffic Class for IPv6 value in each PDR, to classify packets per QoS requirements.

The linked FARs are defined by the SMF to enable the selection of an appropriate tunnel to satisfy the ToS/Traffic class or QoS requirement as each tunnel has specific QoS capabilities to support one or several DSCP markings configured during the tunnel establishment. With a linked FAR, a tunnel is selected, to meet the QoS requirements indicated by the DSCP value detected by the PDR, ensuring that the DSCP value supported by the tunnel has equal or higher priority than the detected DSCP value. The UPF is able to set the IE "transport layer marking" to the mapped DSCP value for transport level marking and to set the IE "outer header" to the TEID of the selected tunnel according to the linked FAR. For processed traffic where the detected DSCP value cannot be matched with any PDRs in the L-PSA UPF or C-PSA UPF, default PDR and FAR are used to select a default tunnel.

For example, the following QoS parameters can be supported by the tunnel:

-	Maximum Bit Rate (MBR) per DNAI (defined in current EDI), or per given application (based on Application ID);

-	Packet Delay Budget (PDB): The delay between two UPFs' terminating N6;

-	Packet Error Rate: Packet sent by a UPF but not successfully received by another UPF.

### 6.21.3	Impacts on existing nodes and functionality

Impacted nodes and functionality are listed as follows:

AF:

-	provides information to 5GC requesting application data forwarding between two parts of the DN (L-DN and a C-DN) via information described in clause 6.21.1.2 along with the QoS requirements in clause 6.21.2.2.

SMF:

-	select PSA UPFs (L-PSA UPF and C-PSA UPF), so that two parts of the DN can be anchored and a tunnel can be established in-between.

-	configure two PSA UPFs (L-PSA UPF and C-PSA UPF) to send/receive traffic at L-DN or C-DN.

-	configure two PSA UPFs (L-PSA UPF and C-PSA UPF) to select a tunnel or a QoS flow of a tunnel based on the DSCP value detected in the packet received at N6.

UPF:

-	In deployments that would require N6 tunnel between EAS/AS and Local/Central PSA-UPF, support new tunnelling mechanisms, and control thereof by SMF.

# * * * * End of changes * * * *