3GPP TSG SA WG2#164	            S2-2408559

Maastricht, 19-23 August 2024 	(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | <CR#> | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Draft CR] Connecting local DN with central DN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To address the outcome of KI#3 in approved eEDGE_5GC_Ph3 WID on means to route the processed application traffics between local DN and the Cloud Server in the central DN, connectivity between local and central DNs is introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce connectivity between a local DN with a central DN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 6.1, 6.2.3.4.1, 6.X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 23.501 [2].

C-DNS	Central DNS

C-DN	Central part of DN

C-NEF	Central NEF

C-PSA UPF	Central PSA UPF

EAS	Edge Application Server

EASDF	Edge Application Server Discovery Function

ECS	Edge Configuration Server

EDC	Edge DNS Client

EEC	Edge Enabler Client

EES	Edge Enabler Server

EHE	Edge Hosting Environment

L-DN	Local part of DN

L-DNS	Local DNS

L-NEF	Local NEF

L-PSA UPF	Local PSA UPF

HR-SBO	Home Routed Session BreakOut

Second CHANGE

## 6.1	General

Edge Computing enables operator and 3rd party services to be hosted in EAS close to the UE's point of attachment. The traffic to EAS can be routed based on the UE position and EAS availability "near to" that position.

The subsequent clauses describe the procedures for supporting Edge Computing in 5G System considering different connectivity models, including:

-	EAS discovery and re-discovery.

-	Edge relocation.

-	Network exposure to Edge Application Server.

-	Support of 3GPP application layer architecture defined in TS 23.558 [5].

-	Support of AF guidance to PCF determination of proper URSP rules.

-	Connecting a local data network (DN) with a central DN.

Third CHANGE

##### 6.2.3.4.1	General

EAS Deployment Information management refers to the capability to create, update or remove EAS Deployment Information from AF and the distribution to the SMF. The NEF is in charge of the management of EAS Deployment Information which may be stored in UDR.

The EAS Deployment Information indicates how edge services are deployed in each Local part of the DN, the description of EAS Deployment Information is shown in Table 6.2.3.4-1.

Table 6.2.3.4-1 Description of EAS Deployment Information

| Parameters | Description |
| --- | --- |
| AF ID | Addressing information of Application Function responsible for the DNAI in the record. [Optional]. See NOTE 1. |
| DNN | DNN for the EAS Deployment Information. [optional] |
| S-NSSAI | S-NSSAI for the EAS Deployment Information. [optional] |
| External Group Identifier/Internal Group Identifier | Group ID for the EAS Deployment information. [optional]. See NOTE 2. |
| Application ID | Identifies the application for which the EAS Deployment Information corresponds to. [optional] |
| FQDN(s) | Supported FQDN(s) for application(s) deployed in the Local part of the DN. |
| DNAI(s) | DNAI(s) for the EAS Deployment information. See NOTE X. [optional] |
| DNS Server Information | list of DNS server identifier (consisting of IP address and port) for each DNAI. [optional] |
| EAS IP address range Information | IP address(es) of the EASs in the Local part of the DN or the IP address ranges (IPv4 subnetwork(s) and/or IPv6 prefix(es) of the Local part of the DN where the EAS is deployed for each DNAI. [optional] |
| N6 traffic routing information | Information about how to forward edge traffic in the local part of DN corresponding to DNAI. [optional] |
| L-DN to C-DN connectivity information | Includes the following parameters: - DNAIs, representing the local and central parts of the DN, to be connected; - QoS requirement(s) as defined in clause 6.X.2.3; - the N6 tunnelling mechanism supported in the two DNAIs to be connected. [optional] |
| NOTE 1: When an AF ID is provided, all DNAI(s) correspond to the same EHE provider. NOTE 2: The AF may provide External Group Identifier, and NEF can map the External Group Identifier into Internal Group Identifier according to information received from UDM. For HR-SBO roaming scenario, External Group Identifier and Internal Group Identifier, cannot be used by AF in VPLMN. NOTE 3: AF ID can be used in case of AF(s) involving different EHE providers, and the source EHE is unaware of other/target EHE specific deployment details. NOTE X: DNAI(s) for the EAS Deployment information includes an indicator whether the DNAI represents a L-DN or C-DN. |  |

The EAS Deployment Information management procedures are described in this clause, the procedures are independent of any PDU Session, including:

-	The procedure for EAS Deployment Information management from AF via the NEF.

-	The procedure for EAS Deployment Information management in the SMF.

-	The procedure for BaselineDNSPattern management in the EASDF.

NOTE:	In order to support EAS discovery when the Edge Hosting Environment is provided by a partner, an SLA is needed between current operator and the partner to provide e.g. the Address(es) and credentials for the DNS servers if the partner hosts the DNS server(s) for the related DNS resolution.

Fourth CHANGE

## 6.X	Support of connecting the local and central parts of a Data Network

### 6.X.1	General

In Edge Computing deployments, there can be use cases that require an application traffic to be first steered to local part of a data network (L-DN) for local processing and then after this initial processing, the application traffic may still need to be further forwarded to the Application Server in the central part of the DN (C-DN) for further processing. The application traffic may not be able to be routed directly between the EAS in the L-DN and the Server in the C-DN in case there is no direct connectivity between the L-DN and C-DN.

In such cases:

-	UL traffic related to an application first routed over EC to Application Server(s) for local-processing, and then further forwarded to a remote Application Server(s) in central part of DN.

-	DL traffic related to an application first routed over central part of DN for processing, then forwarded to Application Server(s) in local EC for local-processing, and finally provided to the UE.

### 6.X.2	Procedures

There may be two ways of establishing the connectivity between the central and local parts of the DN either by using EDI defined in clause 6.2.3.4.1 or by means of AF requesting the tunnels to be established e.g., via Traffic Influence.

#### 6.X.2.1	Procedure to establish the tunnel between different DN parts using EDI

Based on the AF control of the connectivity between different DN parts by using EDI, the 5GS establishes the tunnel between the different parts of the DN as described in this clause.

When SMF receives DNAI information requiring setting up connectivity between different DN parts, the SMF:

- selects a L-PSA UPF based on the received DNAI of the L-DN and a Central PSA UPF (C-PSA UPF) based on the received DNAI of the C-DN;

NOTE:	L-PSA UPF that is selected for the UE PDU session and the L-PSA UPF that establishes the tunnel to the C-PSA UPF can be different.

- uses existing N4 session signalling (providing PDRs and FARs) to the PSA UPF(s) to generate an Up Link Terminal Endpoint Identify (UL TEID) in C-PSA UPF and a Down Link Terminal Endpoint Identify (DL TEID) in L-PSA UPF, so that a tunnel can be established between L-DN and C-DN;

- configures the L-PSA UPF with PDR(s) and FAR(s) as defined below.

Figure 6.X.2.1-1: Procedure of establishing a tunnel between local and central parts of DN using EDI

The steps in the call flow are as follows:

1.	AF provides 5GC with information to provide connectivity between the local and central part of DN and optionally, the tunnelling mechanism at N6, for example, the AF invokes the Nnef_EASDeployment_Create/Update/Delete service operation as the step 1 defined in clause 6.2.3.4.2.

2.	SMF is triggered to retrieve AF provided information. The trigger can be a NEF notification (related with EDI), or a 5GC event (e.g. PDU session establishment/modification, EAS (re-)discovery). SMF retrieves EDI as defined in clause 6.2.3.4.3. Then, the SMF determines whether to establish the tunnel between local and central parts of the DN.

Steps 3 to 11 may take place for each pair of DNAIs (local/central) where the SMF is to establish a tunnel.

3.	The SMF selects two UPFs as the L-PSA UPF and the C-PSA UPF, based on the two retrieved DNAIs.

4.	The SMF initiates a N4 Session Establishment procedure with the selected C-PSA UPF. The N4 message contains PDR-1 and FAR-1 defined in the below Table 6.X.2.1-1 for UL traffic. C-PSA UPF performs the session configuration and UL TEID selection. During the session configuration, UL TEID (i.e. UL TEID of the tunnel) is used for L-PSA UPF to forward L-DN's UL traffic to C-DN via C-PSA UPF. If provided by the AF, the SMF selects one of the tunnelling mechanisms at N6 and establishes the tunnel at N6 towards C-EAS.

5.	The C-PSA UPF responses SMF by sending an N4 Session Establishment Response, including the UL TEID of the C-DN tunnel which is a part of CN tunnel information, and optionally, sending information about the established N6 tunnel via N4, including the selected tunnelling mechanism at N6, the N6 tunnel endpoint address, and the N6 tunnel specific information if needed.

6.	The SMF responds to the NEF with the received information about the established N6 tunnel, and the NEF forwards the information to the AF.

7.	SMF initiates a N4 Session establishment procedure with the selected L-PSA UPF, to provide UL TEID of the tunnel to L-PSA UPF, as well as PDR-2 and FAR-2 defined in Table 6.X.2.1-1 for UL traffic. L-PSA UPF performs the tunnel configuration and DL TEID of C-DN tunnel selection. During the tunnel configuration, DL TEID of the tunnel is selected for L-PSA UPF to forward UL traffic to C-DN via C-PSA UPF. The SMF selects one of the tunnelling mechanisms at N6 provided by the AF and establishes the tunnel at N6 towards L-EAS.

8.	L-PSA UPF provides SMF with the DL TEID of the C-DN tunnel which is a part of CN tunnel information. And optionally, provides information about the established N6 tunnel via N4, including the selected tunnelling mechanism at N6, the N6 tunnel endpoint address, and the N6 tunnel specific information if needed.

9.	The SMF responds to the NEF with the received information about the established N6 tunnel, and the NEF forwards the information to the AF.

10.	SMF initiates a N4 Session Update procedure with the selected L-PSA UPF and provide PDR-3 and FAR-3 defined in Table 6.X.2.1-1 for DL traffic.

11.	L-PSA UPF responses SMF by sending an N4 Session Update Response.

12.	SMF initiates a N4 Session Update procedure with the selected C-PSA UPF and provides DL TEID of the tunnel to C-PSA UPF, as well as PDR-4 and FAR-4 defined in Table 6.X.2.1-1 for DL traffic.

13.	C-PSA UPF responses SMF by sending an N4 Session Update Response.

Table 6.X.2.1-1 shows PDRs and FARs for L-PSA UPF and C-PSA UPF configuration.

Table 6.X.2.1-1: PDRs and FARs for L-PSA UPF and C-PSA UPF

| UPF | Rule | Description |
| --- | --- | --- |
| C-PSA UPF (UL) | PDR-1 | Source Interface = access (N9) Packet Filter Set (optional): Incoming Tunnel ID, IP DA = C-EAS IP (range) GTP-U outer header removal FAR ID = FAR-1’s ID |
|  | FAR-1 | Rule ID C-PSA UPF’s N6 tunnel outer header adding Action = forward Destination interface = N6 |
| L-PSA UPF (UL) | PDR-2 | Source Interface = N6 (optional) Packet Filter Set: Incoming L-PSA UPF’s N6 tunnel info L-PSA UPF’s N6 tunnel outer header removal FAR ID = FAR-2’s ID |
|  | FAR-2 | Rule ID Action = forward Destination interface = N9 GTP-U outer header adding: GTP-u header that the SMF has received from the C-PSA |
| L-PSA UPF (DL) | PDR-3 | Source Interface = access N9 (optional) Packet Filter incoming GTP-U tunnel GTP-U outer header removal FAR ID = FAR-3’s ID |
|  | FAR-3 | Rule ID L-PSA UPF’s N6 tunnel outer header adding Action = forward Destination interface = N6 |
| C-PSA UPF (DL) | PDR-4 | Source Interface = N6 (optional) Packet Filter Set: Incoming C-PSA UPF’s N6 tunnel info C-PSA UPF’s N6 tunnel outer header removal FAR ID = FAR-4’s ID |
|  | FAR-4 | Rule ID 004 Action = forward Destination interface = access (N9) GTP-U outer header adding: GTP-U header that the SMF has received from the L-PSA |

Once a packet is processed by the L-EAS and is to be sent to the C-EAS for further processing, the L-EAS setups L-EAS’s overlay address as the source IP address and C-EAS’s overlay address as the destination IP address of the packet. Using the established N6 tunnel, the L-EAS adds another N6 tunnel outer header, in which the L-EAS’s overlay address is the source address and L-PSA UPF’s tunnel endpoint address is the destination address. By receiving the packet from the L-EAS, the L-PSA UPF applies the defined PDRs and FARs. For packets generated by the C-EAS and to be routed to C-PSA UPF, same approach applies.

#### 6.X.2.2	Procedure to establish the tunnel between different DN parts using AF request

Figure 6.X.2.2-1: Request tunnel over 5GC

1.	AF/EAS requests 5GC to set up a tunnel between two DNAIs, one representing the local part and another one representing the central part of the DN. The EAS/AF only gives IP address ranges of EAS and AS, or FQDNs of these. If not DNAIs are provided, NEF needs to translate the provided info to DNAIs. The AF/EAS provides which tunnelling protocol it supports, e.g. IP in IP, GRE, etc. Also, other relevant data related to the specific tunnel, such as e.g. which optional parameters in GRE to be used, and in case this Key in GRE is used, what value. If AF/EAS supports masque e.g. connect IP (IETF RFC 9484 [X]), AF/EAS can also indicate this. This step can happen at any time e.g. at EAS boot up or restart, when a UE first access the EAS, or via a management procedure in an AF.

2.	NEF finds a SMF supporting both DNAIs and sends a request to this SMF.

3.	SMF follows steps 3-6 in clause 6.X.2.1. With the following differences:

-	SMF selects one of the tunnelling methods provided by the AF/EAS (if 5GC supports at least one of them, else SMF rejects the request)

-	PDRs in Table 6.X.2.1-1 are updated as follows:

Table 6.X.2.2-1: PDRs and FARs for UPF1 and UPF2

| UPF | Rule | Description |
| --- | --- | --- |
| UPF2 (UL) | PDR-1 | Source Interface = access (N9) Packet Filter Set (optional): Incoming Tunnel ID / IP SA = L-EAS IP (range), IP DA = C-EAS IP (range) GTP-U outer header removal FAR ID = FAR-1’s ID |
|  | FAR-1 | Rule ID Action = forward Destination interface = N6 |
| UPF1 (UL) | PDR-2 | Source Interface = N6’ Tunnel outer header removal Packet Filter Set: Incoming Tunnel ID FAR ID = FAR-2’s ID |
|  | FAR-2 | Rule ID Action = forward Destination interface = N9 GTP-U outer header adding: GTP-u header that the SMF has received from the UPF2 |
| UPF1(DL) | PDR-3 | Source Interface = access N9 (optional) Packet Filter: incoming GTP-U tunnel GTP-U outer header removal Tunnel Outer header adding FAR ID = FAR-3’s ID |
|  | FAR-3 | Rule ID Action = forward Destination interface = N6 |
| UPF2(DL) | PDR-4 | Source Interface = core (N6 (optional)) Packet Filter Set: IP SA = C-EAS IP (range), IP DA = L-EAS IP (range) FAR ID = FAR-4’s ID |
|  | FAR-4 | Rule ID Action = forward Destination interface = access (N9) GTP-U outer header adding: GTP-U header that the SMF has received from the UPF1 |

4.	SMF responds with the selected tunnelling mechanism and the tunnel endpoint address of UPF1 and also the tunnel specific information needed, e.g. if Key in GRE is to be used, the Key value. Or if masque is selected, the HTTP proxy address for masque control.

5.	NEF forwards the received information from SMF to the AF/EAS.

#### 6.X.2.3	QoS handling of the tunnel between different DN parts

The QoS requirement of the tunnel is independent from any UE's PDU session, the QoS requirement is defined per DNAI or per application (identified by Application ID). The QoS requirement on a DNAI or application basis is provided via EDI as defined in Table 6.2.3.4-1.

To guarantee the QoS requirement of the tunnel between the L-PSA UPF and the C-PSA UPF, DSCP approach is used. Specifically, the AF provides to the SMF via NEF on the requested QoS requirement in the EDI. For a given set of QoS parameter values from the QoS requirement, a DSCP value can be mapped which is used by the transport networks between the L-PSA UPF and C-PSA UPF. The L-EAS marks processed traffic packets with the same DSCP value as the original UE traffic packets.

The SMF configures L-PSA UPF and C-PSA UPF to handling the QoS requirements, by defining the Packet Filter Set in PDRs and the outer header FARs linked to each PDR. PDRs are defined by the SMF for the L-PSA UPF's and C-PSA UPF's N6 packet detection, specifying a target DSCP value as the packet filter set's TOS for IPv4 or Traffic Class for IPv6 value in each PDR, to classify packets per QoS requirements.

The linked FARs, are defined by the SMF to enable the selection of an appropriate tunnel to satisfy the ToS/Traffic class or QoS requirement as each tunnel has specific QoS capabilities to support one or several DSCP markings configured during the tunnel establishment. With a linked FAR, a tunnel is selected, to meet the QoS requirements indicated by the DSCP value detected by the PDR, ensuring that the DSCP value supported by the tunnel has equal or higher priority than the detected DSCP value. The UPF is able to set the IE "transport layer marking" to the mapped DSCP value for transport level marking and to set the IE "outer header" to the TEID of the selected tunnel according to the linked FAR. For processed traffic where the detected DSCP value cannot be matched with any PDRs in the L-PSA UPF or C-PSA UPF, default PDR and FAR are used to select a default tunnel.

The following QoS parameters can be supported by the tunnel:

-	Maximum Bit Rate (MBR) per DNAI, or per given application (based on Application ID);

-	Packet Delay Budget (PDB): The delay between two UPFs' terminating N6;

-	Packet Error Rate: Packet sent by a UPF but not successfully received by another UPF.

End of CHANGES