3GPP SA WG2 Meeting #164	S2-2408581

Maastricht, Netherlands, 19 – 23 August, 2024

Source:	Nokia, NEC, Samsung

Title:	KI#1 Conclusions

Document for:	Approval

Agenda Item:	19.12

Work Item / Release:	FS_5G_Femto/ Rel-19

Abstract of the contribution: This paper proposes conclusions of KI#1.

# Discussion

Based on the discussions and following evaluations in S2-240XXXX, it is clear that there is a RAN (NG-RAN for 5G to 4G, E-UTRAN for 4G to 5G mobility) impact for a technically feasible solution to enable interworking between a closed 5G Femto CAG and 4G Femto CSG cell.

A technically feasible solution to address the KI#1 should satisfy the following principles:

- In order to mitigate unnecessary handover processing and signaling overhead and prevent degradation on the handover KPI during connected mode mobility, the source system should perform access control as follows:

  - For 5G to 4G mobility, similar to the existing PNI-NPN procedures proposed in Solution#11, the source gNB performs access control.

  - For 4G to 5G mobility, eNB impact is unavoidable which leads to two options:

    - Option 1: Avoid UE impact and consider Solution #11 (MME is impacted).

    - Option 2: Avoid MME impact and consider Solution #5 (UE is impacted).

- A partition of CAG IDs should be reserved to map to CAG IDs to CSG IDs, and vice versa, to enable interworking.

- In order to mitigate unnecessary registration request/tracking area update attempts during idle mode mobility, the UE should have up to date information on allowed CAG and CSG lists.

  - The CSG-CAG ID mapping configuration should be available in UE to enable updates of allowed CAG/CSG list while UE camps on 4G/5G network, respectively, during idle mode mobility.

# Proposal

It is proposed to capture the conclusions principles for KI#1 in the FS_5G_Femto TR 23.700-45.

*** First change ***

## 8.2	Key Issue #1: Support of UE move between CAG cell of 5G Femto and CSG cell

The following principles are concluded for the normative work:

- A partition of CAG IDs should be reserved to map to CAG IDs to CSG IDs, and vice versa, to enable interworking.

- For connected mode mobility:

  - In order to mitigate unnecessary handover processing and signaling overhead and prevent degradation on the handover KPI, the source system should perform access control as follows:

    - For 5G to 4G mobility, similar to the existing PNI-NPN procedures proposed in Solution#11, the source gNB performs access control.

    - For 4G to 5G mobility, eNB impact is unavoidable which leads to two options where one of them should be considered for the normative work:

      - Option 1: Avoid UE impact and consider Solution #11 (MME is impacted); OR.

      - Option 2: Avoid MME impact and consider Solution #5 (UE is impacted).

- For idle mode mobility:

  - In order to mitigate unnecessary registration request/tracking area update attempts during idle mode mobility, the UE should have up to date information on allowed CAG and CSG lists.

    - The CSG-CAG ID mapping configuration should be available in UE to enable updates of allowed CAG/CSG list while UE camps on 4G/5G network, respectively, during idle mode mobility.

*** END of changes ***