3GPP TSG-SA2 Meeting #164	S2-2408590

# Maastricht, Netherlands, 19 - 23 August 2024

Source:	Nokia

Title:	KI#4: No need for new Analytics ID for signalling storm caused by massive UE signalling

Document for:	Approval

Agenda Item:	19.15.2

Work Item / Release:	AIML_CN / Rel-19

Abstract of the contribution: This contribution analyses for key issue 4 whether to define a new Analytics ID for signalling storm caused by massive UE signalling. A related CR is in S2-240xxxx.

# Discussion

In TR 23.700-84, the following conclusion was agreed for KI#4

-	It will be determined in the normative phase whether to define a new Analytics ID or enhance an existing Analytics ID to support Signalling storm Mitigation and Prevention caused by massive signalling of UEs.

This contribution provides additional analysis related to the need of a new Analytics ID or enhance an existing Analytics ID to support Signalling storm Mitigation and Prevention caused by massive signalling of UEs.

Signalling storms occur when there is an excessive amount of signalling traffic in the network, leading to congestion, increased latency, and potential service disruptions. These storms can be caused by various factors, including massive UE signalling due to device behaviour or network conditions.

Observation 1:	…”.

In TS 23.288, clause "6.7 UE related analytics", clause "6.7.5 Abnormal behaviour related network data analytics", an Analytics ID = "Abnormal behaviour" is defined to identify a group of UEs or a specific UE with abnormal behaviour, e.g. being misused or hijacked, with the help of NWDAF. It is noted (NOTE 1) that the misused or hijacked UEs are UEs in which there are malicious applications running or UEs which have been stolen. In a second note (NOTE 2), it is indicated that the expected analytics type generally indicates whether mobility or communication related abnormal behaviour analytics or both are expected by the consumer and the list of exception IDs indicates what specific analytics are expected by the consumer. Either the expected analytics type or the list of Exception IDs needs to be indicated, but they are not presented simultaneously.

The Exception IDs related to communication related Expected analytics type are:

- Unexpected long-live/large rate flows,

- Unexpected wakeup,

- Suspicion of DDoS attack,

- Wrong destination address,

- Too frequent Service Access.

Proposal 1:	.

The signalling storm is also a communication related abnormal behaviour and we can enrich the corresponding list of exceptions to allow its detection/prediction. We can add a new Exception ID "Too much NAS signalling”

Summary of Proposals

Proposal 1:	.

Add a new Exception ID = "Too much NAS signalling" to the list of exception IDs related to the communication related Expected analytics type to the request of analytics consumer, with existing Analytics ID = "Abnormal behaviour" in clause "6.7.5 Abnormal behaviour related network data analytics" of clause "6.7 UE related analytics" of TS 23.288.