3GPP TSG-WG SA2 Meeting #164	S2-2408591

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2136 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Identifying non-3GPP devices behind 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-10-zz |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To specify the support for identifying non-3GPP devices connecting behind a 5G-RG based on the conclusions in clause 8.4 of TR 23.700-32 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specifying conclusions on identification of non-3GPP device for wireline access |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature not implemented in the specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.7.x, 4.10x, 9.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.501 [2] , TS 23.502 [3] and TS 23.503 [4] apply. A term defined in TS 23.501 [2], TS 23.502 [3] or TS 23.503 [4] takes precedence over the definition of the same term, if any, in any other specifications.

RG Level Wireline Access Characteristics: Wireline access technology specific QoS information corresponding to a specific wireline access subscription, which is provided by the AMF to the W-AGF at RG registration.

Wireline access Control Plane protocol (W-CP): Protocol used to transport AS and NAS signalling between the 5G-RG and the W-AGF over the Y4 reference point. W-CP is specified by BBF and CableLabs. There is no assumption that W-CP refers to only a single protocol or only a specific protocol layer.

Wireline access User Plane protocol (W-UP): Protocol used to carry PDU Session user plane traffic between the 5G-RG and the W-AGF over the Y4 reference point. W-UP is specified by BBF and CableLabs. There is no assumption that W-UP refers to only a single protocol or only a specific protocol layer.

Legacy Wireline access Control Plane protocol (L-W-CP): L-W-CP is a legacy control plane protocol between the FN-RG and W-AGF. L-W-CP is specified by BBF and CableLabs. There is no assumption that L-W-CP refers to only a single protocol or only a specific protocol layer.

Legacy Wireline access User Plane protocol (L-W-UP): L-W-UP is a legacy user plane protocol between the FN-RG and W-AGF. W-UP is specified by BBF and CableLabs. There is no assumption that L-W-UP refers to only a single protocol or only a specific protocol layer.

Authenticable Non-3GPP (AUN3) device: A device that does not support NAS signalling, is connected to 5GC via a RG and can be authenticated by 5GC over the RG.

5GS specifications do not support a device using the same subscription to access 5GS as a UE and as an AUN3 device.

Non-Authenticable Non-3GPP (NAUN3) device: A device that does not support NAS signalling, is connected to 5GC via a RG and for which authentication with 5GC is not supported.

Identifiable Non-3GPP (IN3) device: A device that does not support NAS signaling, is connected to 5GC via a RG, and can be individually identified but not authenticated by the 5GS to provide differentiated QoS.

NOTE 1:	AUN3,NAUN3, and IN3 device can connect to RG through WLAN (collocated or not collocated with the RG) and/or wired Ethernet connections.

### NOTE 2:	A device can operate as a UE over NG-RAN and as a AUN3 or NAUN3 or IN3 via a RG, if the device implements UE functionality.

# * * * * Second change (all new text) * * * *

### 4.7.x	IN3 Device Identifier

5GS uses Device Identifiers to identify individual IN3 devices. A Device Identifier is a permanent identifier in the format of a generic string, and the allocation of the Device Identifier is out of scope of 3GPP.

Device Identifiers and their corresponding QoS policies associated with a 5G-RG subscription are provisioned into the UDR by an AF. Traffic from each IN3 device is associated with a Device Identifier and the Device Identifier is used to obtain QoS policies for the traffic.

# * * * * Third change (all new text) * * * *

## 4.10x	Differentiated services for IN3 devices behind 5G-RG

This clause defines the support of IN3 devices, i.e. Identifiable Non-Authenticable Non-3GPP devices as defined in clause 3.1, behind a 5G-RG.

The overall architecture is illustrated in Figure 4.10x-1.

Figure 4.10x-1: Example scenario for IN3 devices behind 5G-RG

Differentiated services for IN3 devices behind 5G-RG are provided as specified below:

-	Traffic from each IN3 device is associated with a Device Identifier.

-	5G-RG binds the Device Identifier to an IN3 device.

NOTE 1: 	How 5G-RG binds Device Identifier to an IN3 device is out of scope of 3GPP.

-	IN3 Device Identifiers and their corresponding QoS policies associated with a 5G-RG subscription are provisioned into the UDR by the AF.

-	QoS differentiation for IN3 devices can be performed within a PDU Session (using different QoS Flows) or between PDU Sessions.

-	After receiving a Nudr_DM_Notify notification of data change from the UDR, the PCF can initiate 5G-RG Policy delivery, as per clause 9.5.2.1. URSP rules can be provided to the 5G-RG to indicate how to associate Device Identifiers to PDU Sessions.

-	If the PCF determines that an existing PDU Session is impacted by the data change in the UDR, the PCF updates the SMF with corresponding new policy information about the PDU Session by invoking Npcf_SMPolicyControl_UpdateNotify service operation.

-	After an IN3 device connects, 5G-RG uses URSP rules to determine if an existing PDU Session can be used or if a new PDU Session needs to be established. For an existing PDU Session, 5G-RG uses the PDU Session Modification procedures to provide the Device Identifier and the user plane address of the IN3 device to the SMF via the NAS-SM message. For a new PDU Session, 5G-RG uses the PDU Session Establishment procedures to provide the Device Identifier and the user plane address of the IN3 device to the SMF via the NAS-SM message.

-	When using an IP PDU Session Type, the user plane address is the 5G-RG IP Address(es)/ Port Number(s) that carry the IN3 device's traffic. When using an Ethernet PDU Session Type, the user plane address is the MAC address and/or the VLAN tag ID that is associated with the IN3 device's traffic.

NOTE 2:	In case 5G-RG performs NAT, 5G-RG will ensure that it does not NAT the traffic from different IN3 devices to the same IP address and port number range combination.

-	SMF forwards the IN3 Device Identifier to the PCF as part of the SM Policy Association procedure. PCF includes the Device Identifier in the Nudr_DM_Query to request for policy control related subscription information from the UDR.

# * * * * Fourth change * * * *

### 9.5.2	UE access selection and PDU Session selection related policy information

#### 9.5.2.1	5G-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6. for 5G-RG.

Only URSP policies are applicable for a 5G -RG.

NOTE:	For example, ANDSP policies, ProSe Layer-3 UE-to-Network Relay Offload Policy are not applicable to 5G-RG,

If the PCF provides a URSP policy to the 5G-RG, the PCF should only provide the following URSP policy information:

-	Rule Precedence.

-	Traffic descriptor/Application descriptors, IP descriptors, Domain descriptors, Non-IP descriptors, DNN, Connection Capabilities.

-	Route Selection Descriptor Precedence.

-	Route selection components/ SSC Mode Selection, Network Slice Selection, DNN Selection, PDU Session Type Selection.

-	Route Selection Validation Criteria/ Time Window.

The 5G-RG shall ignore any of the parameters and policies not listed above, if received from the 5GC.

The 5G-RG shall use the URSP policy as specified in TS 23.503 [4], for example for the association of application and PDU session, slices, etc.

The URSP indicates for the application of Auto-Configuration Server (ACS) which PDU session type, NSSAI and/or DNN is to be used. The 5G-RG establishes the connectivity to the management entity (e.g. ACS) via user plane connection on a PDU session according to the URSP.

UE Policy procedures defined in clause 6.1.2.2 of TS 23.503 [4] are applicable as follows:

-	Roaming is not applicable to W-5GAN access in this release of specification.

-	USRP rule enforcement reporting is not defined for 5G-RG.

In the case of an Authenticable Non-3GPP (AUN3) device behind a 5G-RG only URSP policies are applicable. Each AUN3 device is managed in 5G-RG as a separate device with its own NAS and states, therefore from point of view of 5GC, the AUN3 device is seen as a single device and the URSP received in the NAS dedicated to an AUN3 device shall apply to this AUN3 device only the 5G-RG shall apply URSP received within NAS for AUN3 device acting on behalf of AUN3 device. Therefore to support the case when AUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF for the SUPI associated with the AUN3 device.

UE Route Selection Policy information targeting an AUN3 device (i.e. sent to a 5G-RG in the NAS connection corresponding to an AUN3 device) follows the structure defined in clause 6.2.2 of TS 23.503 [4] with the following difference:

-	As an AUN3 can have only one PDU Session, its URSP shall contain a match all TD.

USRP rule enforcement reporting is not defined for AUN3 devices.

In order to support the case when NAUN3 and IN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF on 5G-RG.

URSP rules for NAUN3 and IN3 devices connected to 5G-RG follow the structure defined in clause 6.6.2 of TS 23.503 [4] and may contain any combination of the following traffic descriptors:

-	IP Descriptors: For IP traffic from NAUN3/IN3 devices connected to 5G-RG, IP descriptors are matched against header information contained in IP packets sent by NAUN3/IN3 devices; IP descriptors are only applicable for traffic from NAUN3/IN3 devices if network address translation (NAT) is performed for that traffic.

-	Non-IP descriptors: For Ethernet traffic from NAUN3/IN3 devices connected to 5G-RG, Non-IP descriptors are matched against header information contained in Ethernet frames sent by NAUN3/IN3 devices.

-	Connectivity Group ID: For traffic from a NAUN3 device connected to 5G-RG, Connectivity Group ID in the URSP rule is matched against the Connectivity Group ID that the NAUN3 device is associated with (see clause 4.10b).

-	IN3 Device Identifier: For traffic from an IN3 device connected to 5G-RG, IN3 Device Identifier in the URSP rule is matched against the Device Identifier that the IN3 device is associated with (see clause 4.10x).

# * * *  End of changes * * * *