3GPP TSG-SA2 Meeting #164 	S2-2408613

Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-240xxxx)

# Source:	CATT

# Title:	New WID on PRU Usage Extension in Core Network and One Transmission Path used for one Positioning Procedure

# Document for:	Approval

# Agenda Item:	30.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	PRU Usage Extension in Core Network and One Transmission Path used for one Positioning Procedure

Acronym:	TEI19_PRUUaOTP

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-19

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don't know |  |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

3	Justification

In SA2#159 meeting, the received RAN1 LS (S2-2310113) includes RAN1 agreements of NR carrier phase positioning in Rel-18 which introduces more scenarios to use PRU. The first scenario is to support UE based positioning using PRU measurements, and the second scenario is that LMF determines UE location based on UE measurements and PRU measurements in indicated time windows when network assisted positioning or UE assisted positioning is used. Based on the existing SA2 specification, the first scenario above is not supported. Furthermore, for the second scenario above, when the serving LMF of target UE and the serving LMF of PRU are different LMFs, the impacts to Nlmf interface are different in the case that network assisted positioning or UE assisted positioning is selected by the serving LMF of target UE.  So it is proposed to extend PRU usage in core network to align SA2 specification and RAN specifications.

After user plane positioning is introduced in Rel-18 5G_eLCS_Ph3, for the messages (LPP messages, supplementary services messages) belonging to one positioning procedure, there is no common understanding on transferring the messages via user plane and control plane. Some companies believe that for the messages above, either control plane or user plane (i.e. one transmission path) should be used by UE and LMF. While some companies think it is flexible for LMF to choose control plane or user plane to transfer the messages above, e.g. some messages are transferred via user plane and the others can be transferred via control plane. The way to go needs to be discussed and specified.

Besides the above issue for user plane positioning, after the LMF selects the user plane or control plane positioning for one positioning procedure, during the positioning procedure, the transmission path may fail caused by control plane congestion in the case that the control plane positioning is selected or user plane positioning dedicated PDU Session release in the case that the user plane positioning is used. In this case, the LMF behaviour is also not clear. Especially, the PDU Session for user plane positioning may be released by different causes which can lead to different LMF behaviour. For example, if the PDU Session release is cause by handover or S-NSSAI congestion, it is impossible to re-establish the PDU Session. In this case, if the LMF re-triggers user plane connection establishment for multiple times, all the requests will be rejected by UE. To avoid the delay, the LMF needs to re-select control plane to re-trigger the positioning procedure if the release cause is handover or S-NSSAI congestion. Another example, the PDU Session may also be released to support service continuity (SSC mode 2), in this case, the PDU Session can be re-established, so the LMF can re-trigger the user plane connection establishment. Based on the analyses above, the LMF behaviour to support different PDU Session release cause also needs to be considered.

Furthermore, the user plane positioning causes different charging information compared with the control plane positioning. The charging difference leads to different user preference on user plane positioning and control plane positioning. To improve the user experience, the user preference above needs to be considered by LMF to select user plane or control plane positioning. However, this is not supported by existing specification.

Considering the issues related to user plane positioning above are not in the scope of Rel-18 5G_eLCS_Ph3 WID, it is proposed to introduce the mechanism to solve the issues in Rel-19.

4	Objective

The objective is to have the 5G System enhanced to support more scenarios to use PRU and to introduce the mechanism of one transmission path for messages of one LCS session. The following enhancements are required:

-	To support more scenarios to use PRU, the enhancements include:

-	To support network assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) when the serving LMF of target UE and the serving LMF of PRU are different: The serving LMF of target UE provides the positioning method, time window(s) (i.e. Time Window Information SRS List and Time Window Information Measurement List specified in TS 38.455), and the identifier of the selected TRPs to the serving LMF of PRU which is responsible to obtain PRU measurements based on the information received.

-	To support UE assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) when the serving LMF of target UE and the serving LMF of PRU are different: The serving LMF of target UE provides the positioning method, time window(s) (i.e. NR-DL-PRS-MeasurementTimeWindowsConfig specified in TS 37.355) to the serving LMF of PRU which is responsible to obtain PRU measurements based on the information received.

-	To support UE-based positioning using PRU measurements: When the LMF determines to use UE-based positioning and to use PRU to improve the positioning result, the LMF provides the assistance data including PRU measurements, the timestamp associated with the measurements and the PRU location to target UE. The target UE uses the assistance data to calculate location.

-	To support one transmission path used for one positioning procedure, the enhancements include:

-	The LMF selects the user plane positioning or control plane positioning for a positioning procedure based on UE preference on control plane and user plane positioning which is determined by UE based on charging information, user plane positioning capabilities and so on. The UE provides the preference along with the user plane  positioning capabilities.

-	When the LMF selects control plane positioning, all the supplementary services messages and LPP messages related to the positioning procedure are transferred via the control plane. During the positioning procedure, if the control plane transmission fails, the LMF determines to select user plane to re-trigger the positioning procedure or re-trigger the positioning procedure over the control plane when the control plane congestion ceases.

-	When the LMF selects user plane positioning, all the supplementary services messages and LPP messages related to the positioning procedure are transferred via the user plane. During the positioning procedure, if the user plane positioning dedicated PDU Session is released. The AMF notifies the PDU Session status and the cause to LMF which is used by LMF to determine to select control plane to re-trigger the positioning procedure or to re-trigger the user plane connection establishment. In this case, if the LMF determines to terminate the connection to the UE, the LMF provides the appropriate rejection cause.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.273 | Clause 4.3.8. Update LMF function to support network assisted positioning and UE assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) when the serving LMF of target UE and the serving LMF of PRU are different, and to support UE-based positioning using PRU measurements. Clause 6.17.4. Update the procedure to support network assisted positioning and UE assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) when the serving LMF of target UE and the serving LMF of PRU are different, and to support UE-based positioning using PRU measurements. Clause 8.3.2.6. Update the Nlmf_Location_MeasurementData service operation to add the optional input parameters to support network assisted positioning and UE assisted positioning based on target UE measurements and PRU measurements in indicated time window(s) when the serving LMF of target UE and the serving LMF of PRU are different. Clause 5.x. Add the mechanism of one transmission path used for one positioning procedure. | TBD | This will be handled as “TEI19”. |

This TEI19_WID requires 0.5 TU.

6	Work item Rapporteur(s)

Hou, Yunjing, CATT, houyunjing@catt.cn

7	Work item leadership

SA2

8	Aspects that involve other WGs

None identified.

9	Supporting Individual Members

Supporting IM name

CATT

OPPO

vivo

China Telecom