3GPP TSG-SA2 Meeting #164	S2-2408616

# Maastricht, Netherlands, 19 August – 23 August, 2024          			(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR |  | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | One Transmission Path used for One Positioning Procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, DUMMY |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | User plane positioning is introduced in Rel-18 5G_eLCS_Ph3 to transfer the LPP messages and supplementary service messages via user plane. For the messages (LPP messages, supplementary services messages) belonging to one positioning procedure, there is no common understanding on how to transfer the messages via user plane and control plane. Some companies believe that for the messages belonging to one LCS session, either control plane or user plane should be used by UE and LMF at the same time. While some companies think it is flexible for LMF to choose control plane or user plane to transfer the messages above, e.g. some messages above are transferred via user plane and the others could be transferred via control plane. If some messages belonging to one LCS session could be transferred via user plane and others could be transferred via control plane, the LMF and UE need to be enhanced to select user plane or control plane before sending a message. So this makes LMF and UE more complicated and increases positioning delay. To reduce complicity and positioning delay, it is proposed that one transmission path (i.e. control plane or user plane) is used for one positioning procedure. The UE and the LMF is enhanced to support the principle. After the transmission path is selected by LMF, during the positioning procedure, if the selected transmission path fails, LMF determines to select another path to re-trigger the positioning procedure or to re-trigger the positiong procedure when the selected transmission path is recovered. For example, when user plane positioning is selected for a positioning procedure, during the positioning procedure, if the user plane positioning dedicated PDU Session is released. Due to different release cause, the LMF behaviour is different: - If the PDU Session is released due to handover or S-NSSAI based congestion control, the LMF selects control plane to re-trigger the positioning procedure. - If the PDU Session is released due to establish a new PDU Session with an identical PDU Session ID, the LMF re-triggers the user plane connection establishment. Furthermore, the user plane positioning causes different charging information compared with the control plane positioning. The charging difference leads to different user preference on user plane positioning and control plane positioning. To improve the user experience, the user preference above needs to be considered by LMF to select user plane or control plane positioning. So it is proposed that the UE provides the UE preference on control plane and user plane for positioning procedure along with the user plane positioning capability to assist LMF to make the decision. Moreover, the LCS-UP load will be increased constantly in the wake of the number grows of user plane connections and location messages via user plane. In such case, the LCS-UP load balance is necessary for the LMF (e.g., consider LCS-UP load during LMF selection, release old LCS-UP connection). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The LMF considers the UE preference provided by UE when determines to use control plane or user plane positioning for a positioning procedure. - For a positioning procedure, one transmission path is used at a time. - During the positioning procedure, if the selected transmission path fails, LMF determines to select another path to re-trigger the positioning procedure or to re-trigger the positiong procedure when the selected transmission path is recovered. - Clarify that LMF LCS-UP load is also taken into account during the LMF selection, and LMF can release the old user plane connection with appropriate reject cause due to LCS-UP congestion control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent behavior between LMF and UE. And the LMF will be overloaded without any mitigation actions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.x (new), 6.18.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * *

5.1	LMF Discovery and Selection

LMF selection functionality is supported by the AMF to determine an LMF for location estimation of the target UE or Ranging/Sidelink Positioning between Target UE and SL Reference UE. The LMF selection functionality is also supported by the LMF if it determines that it is unsuitable or unable to support location for the current UE access network or serving cell for the deferred 5GC-MT-LR procedure for periodic, triggered location events, or Modification of User Plane Connection. The LMF selection functionality may also be supported by the GMLC and the GMLC provides the selected LMF ID to AMF.

LMF reselection is a functionality supported by AMF when necessary, e.g. due to UE mobility.

The LMF selection/reselection may be performed at the AMF or LMF or GMLC based on the locally available information i.e. LMF profiles are configured locally at AMF or LMF or GMLC, or by querying NRF.

The following factors may be considered during the LMF selection:

-	LCS client type.

-	Requested Quality of Service information, e.g.:

-	LCS accuracy,

-	Response time (latency),

-	Access Type (3GPP /N3GPP).

NOTE 1:	Location methods may differ depending on the Access Type, e.g. in the case of WLAN Access Location determination may just correspond to retrieval of IP addressing information from the N3IWF/TNGF; As another example, for Wireline access, Location determination may just correspond to retrieval of geo coordinates corresponding to a GLI as defined in clause 4.7.8 of TS 23.316 [21] or a HFC Node ID.

-	RAT type (e.g. 5G NR, eLTE, or any of the RAT Types specified for NR satellite access) and/or the serving AN node (i.e. gNB or NG-eNB) of the target UE.

-	RAN configuration information.

-	LMF capabilities, including:

-	the support of Uu based positioning as defined in clause 4.3.8;

-	Ranging/Sidelink positioning as defined in clause 4.3.8 of TS 23.586 [40];

-	LMF user plane positioning capabilities (the capability to support LCS-UPP).

-	LMF load.

NOTE X:	If the LMF supports the user plane positioning, the LMF load refers to the control plane load and the user plane load of the LMF.

-	LMF location.

-	Indication of either a single event report or multiple event reports.

-	Duration of event reporting.

-	Network slicing information, e.g. S-NSSAI and/or NSI ID.

-	LMF Service Area consisting of one or more TA(s).

-	Supported GAD shapes.

-	Support LCS when MBSR is involved.

-	Requested UE has maintained user plane connection with certain LMFs.

When receiving a NAS message from UE, including an LMF ID together with a LPP message (refer to step 25 in clause 6.3.1 for event reporting for a deferred 5GC-MT-LR), AMF sends the LPP message to the LMF, as indicated by the LMF ID.

NOTE 2:	Description on how UE encapsulates the LMF ID in the NAS message is documented in TS 24.571 [36].

UDM may store the LMF ID in UE subscription data. During the location procedure, GMLC received the LMF ID from the UDM and provides it to AMF.

GMLC may be configured with the following parameters:

-	LMF ID and/or

-	per group ID and its correlating LMF ID.

The AMF may use locally provisioned configuration to determine LMF based on UE identify or its group information.

NOTE 3:	It is AMF implementation specific for the priority of different selection criteria from GMLC, AMF and LMF.

When the GMLC receives a MT location request from LCS client/AF, GMLC determines the LMF ID based on the configured parameters for an LCS Client/AF. In case a group ID is provided or derived from the location request, GMLC determines the correlating LMF ID based on the provisioned group ID.

GMLC may have configuration with one or several dedicated LMF ID(s), irrelevant to any LCS client/AF. When the GMLC receives a MT location request from LCS client/AF, GMLC only determines the LMF ID within the configuration for all LCS client/AF.

NOTE 4:	When AMF can't access the LMF instance of the LMF ID, by default, AMF replies to corresponding error to GMLC. GMLC could retry or fail the request accordingly; with explicit configuration to serve as backup selection in this, AMF can also select different configured LMF instance.

* * * Next Change * * *

5.x	One Transmission Path used for one Positioning Procedure

The LMF is responsible to select control plane positioning or user plane positioning for a positioning procedure to transfer the supplementary services messages and LPP messages included in the procedure based on user plane positioning capability, control plane congestion status (e.g. AMF load status), UE preference on control plane and user plane positioning and other implementation factor. The UE preference on control plane and user plane positioning is determined by UE based on the charging information, cause value of the user plane positioning dedicated PDU Session release, user plane positioning capabilities and so on. The UE provides the preference along with the user plane positioning capabilities.

When the LMF selects control plane for a positioning procedure,

-	The LMF sends the supplementary services messages and LPP messages over control plane to UE. If the UE receives the DL NAS TRANSPORT message including an LCS Corrleation ID, the UE transfers all the supplementary services messages and LPP messages related to the LCS Corrleation ID over the control plane.

-	During the positioning procedure, if the control plane transmission fails due to control plane congestion. The LMF determines to select user plane to re-trigger the positioning procedure if the user plane connection exists or re-trigger the positioning procedure over the control plane when control plane congestion ceases.

When the user plane is selected by LMF for a positioning procedure,

-	The LMF sends the supplementary services messages and LPP messages over user plane to UE. If the UE receives the user plane transport message including an LCS Corrleation ID from LMF, the UE transfers all the supplementary services messages and LPP messages related to the LCS Corrleation ID over the user plane.

-	During the postioning procedure, if the user plane positioning dedicated PDU Session is released based on the triggers described in clause 4.3.4.2 in TS 23.502[19], the SMF invokes the Nsmf_PDUSession_SMContextStatusNotify service operation to AMF to notify the PDU Session status(i.e. release) and the cause. The AMF notifies the PDU Session status(i.e. release) and the cause to LMF. Based on the notification, the LMF determines to select control plane to re-trigger the positioning procedure or to re-trigger the user plane connection establishment.

NOTE X:	As examples, if the PDU Session is released due to handover or S-NSSAI based congestion control as specified in clause 6.1.6.3.8 in TS 29.502[X], the LMF selects control plane to re-trigger the positioning procedure. If the PDU Session is released due to establish a new PDU Session with an identical PDU Session ID as specified in clause 6.1.6.3.8 in TS 29.502[X], the LMF re-triggers the user plane connection establishment.

* * * Next Change * * *

6.18.3	Modification of User Plane Connection between UE and LMF

The figure 6.18.3-1 shows how a secure user plane connection between UE and LMF is modified (e.g. when the LMF decides that a change of LMF is needed, based on its own load status, event report from the UE, or UE location information notified via Namf_Communication_N1MessageNotify service operation). The flow describes change of LMF but applies also when source and target LMF is the same. The procedure can also be used to terminate the user plane connection to Source LMF by not selecting any Target LMF (e.g., when the LMF is detected to be overloaded or indicates UE  to change the positioning protocol).

Figure 6.18.3-1: Connection modification between UE and LMFs

1a.	[Conditional] The LMF discovers a need to change LMF or re-establish the user plane connection between UE and LMF or terminate the user plane connection. The LMF sends an Nlmf_Location_UPNotify message that includes connection move (termination and establishment) or termination is required and if connection move is requested then message may include target LMF identification. The address of the AMF was provided to LMF as a "Notification Target Address" in latest Nlmf_Location_UPConfig message or Nlmf_Location_UP Subscribe message.

NOTE:	The LMF change procedure is independent from the SSC mode of the PDU Session with dedicated DNN used for positioning. For SSC mode 2/3, PSA UPF connection with the LMF can be relocated with UE movement and LMF can discover the need to change LMF to reduce the user plane path latency.

1b.	[Conditional] AMF based on target LMF identification received from source LMF for user plane positioning, or AMF may perform LMF reselection if UE moves to a new location (which may be out of serving area of source LMF and in serving area of target LMF) and select the target LMF for the current UE location based on LMF service area, LMF user plane positioning capability information and other information listed in clause 5.1. LMF needs to be capable to establish a user plane session for positioning with the UE. Further at AMF relocation the target AMF needs to inform the LMF of the AMF change.

2.	[Conditional] If AMF reallocation has occurred, this step is skipped. Otherwise, steps 3-8 of figure 6.18.2-1 are performed between AMF, UE, and Target LMF with addition that UE also terminate connection to Source LMF.

3.	The AMF sends an Nlmf_Location_UPConfig Request towards the source LMF. The message may include a request for the Source LMF to terminate a specific user plane connection to the UE and the Target LMF identification. Alternatively, it may include information about AMF reallocation.

4-5.[Conditional] The source LMF may invoke an Nlmf_Location_LocationContextTransfer Request service operation towards the target LMF to provide the current location context of the UE, if there is periodic and triggered UE location events report context. The target LMF informs source LMF of the location context transfer operation results.

6.	[Conditional] If the user plane connection to source LMF is still active, the source LMF terminates the connection to the UE with appropriate reject cause (e.g., insufficient resource, change positioning protocol).

7.	The LMF sends Nlmf_Location_UPConfig Response message to AMF to confirm connection termination or acknowledge change of AMF. If this procedure is used for termination, the AMF will release the LCS-UP context after receiving the response message.

* * * End of Change * * *