3GPP TSG-SA2 Meeting #164	S2-2408626

Maastricht, Netherlands, 19 August – 23 August, 2024 	(revision of S2-24xxxxx)

Source:	CATT

Title:	23.700-59: Update the conclusion for key issue#3

Document for:	Approval

Agenda Item:	19.10.1

Work Item / Release:	FS_UAS_Ph3

Abstract of the contribution: This paper is proposed to update the conclusion for key issue#3.

1. Discussion

1.1	Introduction

Based on the conclusion for key issue#3, the following issues need to be decided during normative phase.

NOTE 2:	Whether all options are required will be determined in normative phase.

NOTE 3:	Whether indication about UAV UE's NTZ support to the AMF/MME is needed will be decided during normative phase. This work does not have RAN impacts.

NOTE 5:	Whether extension of Service Restrictions for NTZs using Restricted Transmission Area (RTA) mapped from the NTZ information is needed will be discussed during normative phase. This work does not have RAN impacts.

It is proposed to solve the issues above.

1.2	Proposal for NOTE 2

The NOTE 2 is related to the method to provide the NTZ assistance information to UAV UE.

The above NTZ assistance information is made available to the UAV UE by using one or more following ways:

-	pre-configured;

-	provided/updated by the AMF during the registration procedure;

-	provided/updated by the AMF during the UE Configuration Update procedure specified in clause 4.2.4.2 of TS 23.502 [4];

-	provided/updated by the MME during the attach procedure;

-	provided/updated by the MME during the TAU procedure;

-	provided/updated by the PCF;

-	provided/updated by the AF (e.g. NTZ AF, USS, TPAE).

NOTE 2:	Whether all options are required will be determined in normative phase.

Based on the reply ([R19 FS_UAS_Ph3] Pre-SA2#164 NWM Discussion for KI#3 | NWM Web (etsi.org)) in NWM, different company supports different option(s). Considering there is no objection to any option above, it is proposed to support all options in normative phase and add a NOTE to explain that which option to be used is implementation dependent.

1.3	Proposal for NOTE 3

For the NOTE 3 below:

NOTE 3:	Whether indication about UAV UE's NTZ support to the AMF/MME is needed will be decided during normative phase. This work does not have RAN impacts.

Based on the reply ([R19 FS_UAS_Ph3] Pre-SA2#164 NWM Discussion for KI#3 | NWM Web (etsi.org)) in NWM, the supporting reason is to assist the AMF/MME to determine whether providing/updating NTZ assistance information to UAV UE. Some companies think the indication is useful, but not essential, because the UAV UE can ignore the unsupported NTZ assistance information received from AMF/MME/PCF.

Considering it is useful, it is proposed to support the indication.

1.5	Proposal for NOTE 5

For the following NOTE 5:

NOTE 5:	Whether extension of Service Restrictions for NTZs using Restricted Transmission Area (RTA) mapped from the NTZ information is needed will be discussed during normative phase. This work does not have RAN impacts.

The Restricted Transmission Area includes TA/Cell list which is mapped from the NTZ information. The RTA is sent to UAV UE to enable the UAV UE to determine the correct behavior when in an area impacted by NTZ.

The key point of the RTA is to send the TA/Cell list to UAV UE, considering the geographical area may not map with the 3GPP area exactly, so the TA/Cell list includes the TA(s)/Cell(s) which has overlapping area with the geographical area of NTZ and the TA/Cell list should be larger than the geographical area of NTZ.

To support NTZ, the UAV UE behavior is impacted by both area and frequency band. The NTZ assistance information sent to UAV UE already includes the area information, UAV UE can determine the correct behavior based on NTZ assistance information without the TA/Cell list.

The value we see for the TA/Cell list is that the UAV UE can use it to determine whether it is approaching the NTZ based on the cell broadcasting information instead of performing the positioning procedure which introduces more power consumption. So it is proposed to provide the TA/Cell list to UAV UE to reduce power consumption. Considering the TA/Cell list is determined based on the geographical area of NTZ, to avoid introduce a new parameter, it is proposed that the NTZ assistance information sent to UE includes the TA/Cell list including the TA(s)/Cell(s) which has overlapping area with the geographical area of NTZ. The UAV UE uses the TA/Cell list to determine whether it is approaching NTZ.

2. Proposal

It is proposed to update the conclusion for key issue#3 based on the discussion in clause 1.

* First change *

8.3.2	Configuring/Provisioning NTZ assistance information to UAV UE and NTZ enforcement by UAV UE

The following NTZ assistance information (or NTZ policy) is configured/provisioned in the UAV UE.

-	A list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and height restrictions).

NOTE 1:	It is assumed UAV UE is configured with NTZ assistance information before accessing to the operator's network, if the local regulation requires NTZ.

The above NTZ assistance information is made available to the UAV UE by using one or more following ways:

-	pre-configured;

-	provided/updated by the AMF during the registration procedure;

-	provided/updated by the AMF during the UE Configuration Update procedure specified in clause 4.2.4.2 of TS 23.502 [4];

-	provided/updated by the MME during the attach procedure;

-	provided/updated by the MME during the TAU procedure;

-	provided/updated by the PCF;

-	provided/updated by the AF (e.g. NTZ AF, USS, TPAE).

NOTE 2:	Which option to be used is implementation dependent.

The UAV UE is responsible for NTZ enforcement independently of the UAV UE 3GPP Release.

The UAV UE supporting NTZ (i.e. Rel-19 and onward UAV UEs) performs the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	upon applying NTZ enforcement, the UAV UE does not transmit any signalling or data.

c)	does not select/reselect any cells impacted entirely by an NTZ.

The UAV UE provides an indication about UAV UE’s NTZ support to the AMF/MME.

The NTZ assistance information also includes a TA/Cell list which includes TA(s)/Cell(s) overlapped with the geographical area of NTZ and is determined by OAM. When the UAV UE receives the NTZ assistance information, the UAV UE uses the TA/Cell list to determine whether it is approaching the NTZ.

NOTE 4:	For the network to know about UAV presence inside NTZ, whether registration updates by the UAV UE before entering the NTZ and when exiting the NTZ are needed or the existing mechanism (e.g. AMF subscribing with NG-RAN by providing NTZ as AoI) can be applied will be decided during normative phase. This work does not have RAN impacts.

* End of change *