3GPP TSG-SA2 Meeting #164	S2-2408627

Maastricht, Netherlands, 19 August – 23 August, 2024 	(revision of S2-24xxxxx)

Source:	CATT

Title:	23.700-59: Update the principles to support NTZ

Document for:	Approval

Agenda Item:	19.10.1

Work Item / Release:	FS_UAS_Ph3

Abstract of the contribution: This paper is proposed to update the principles to support NTZ.

1. Discussion

1.1	Add a new principle based on RAN conclusion

Based on RAN LS (S2-2407430) received in SA2#164 meeting, RAN has the following conclusion related to NTZ:

- The RAN network should not be involved in the process of informing the UE on Non Transmit Zone(s)

- The RAN network should not be involved in the process of enforcing UE behavior when it is located within the NTZ(s).

- The UE is responsible of following any applicable the NTZ(s) regulations.

- The RAN network should not receive the NTZ(s) information.

- Any solution that breaks the above principles should be excluded from SA2 discussions.

It is proposed to add a new principle in conclusion for KI#3 to reflect the conclusion above.

Proposal#1: RAN network should not be impacted to support NTZ, e.g. RAN network should not receive the NTZ information and should not be involved in the process of enforcing the NTZ.

1.2	Clarification of downlink data transmission

In this clause, the existing principle of “Reception of downlink data from the network by the UAV UE located in the NTZ is allowed given the regulatory requirements are not violated” is further discussed and a way forward is proposed.

Based on the existing mechanism in TS 38.321, when downlink data is received, UE instructs the physical layer to generate acknowledgement(s) of the data. If the acknowledgement is not received, RAN believes the downlink data transmission fails and re-transmits the data. When RAN determines user inactivity, RAN triggers the AN release procedure in clause 4.2.6 in TS 23.502.

For UAV UE located in NTZ,

- If the frequency band(s) used by the UAV UE is non-restricted frequency band(s), the UAV UE is allowed to send ACK for the downlink data.

- Otherwise, the UAV UE is not allowed to send ACK for the downlink data. In this case, the RAN will retransmit the downlink data which leads to a waste of radio resource. If the UE enters CM-IDLE state, the downlink data will trigger paging procedure which will not responded by UAV UE, in this case the paging fails and introduces a waste of the radio resource.

Based on the analyses above, when the AMF does not know whether the UAV UE is allowed to transmit in the NTZ area, the existing mechanism can work, the drawback is the waste of radio resource for downlink data retransmission and paging.

Observation#1: if the AMF does not know whether the UAV UE is allowed to transmit in the NTZ, the downlink data is sent to UAV UE as usual. In this case, the existing mechanism (e.g. downlink data retransmission, AN release, paging, de-registration) can work no matter the UAV UE is allowed to transmit or not. The drawback is a waste of radio resource for downlink data retransmission and paging if the UE is not allowed to transmit, i.e. the frequency band(s) used by UAV UE is restricted.

To reduce the waste of radio resource above, the AMF needs to know that the UAV UE is not allowed to transmit to avoid sending downlink data to UAV UE. There are two related solutions in NOTE 4 as follows:

- Sol#1: the UAV UE sends Registration Request with an indication that the UAV UE is about to enter an NTZ to AMF before entering the NTZ. In this solution, the AMF cannot know whether the UAV UE is allowed to transmit or not, instead, it can only know that the UAV UE is about to enter an NTZ. As described in sol#7, based on the indication, the network considers the UAV UE unreachable, so this solution blocks any downlink data transmission for UAV UE in NTZ, even the UAV UE can select non-restricted frequency band in the NTZ to send and receive data as normal.

- Sol#2: reuse the existing AoI mechanism in which the AoI is determined based on the NTZ geographical area. In this case, the AMF can only know the UAV UE presence in the AoI, it cannot know whether the UAV UE is allowed to transmit or not, as the frequency band(s) used by UAV UE is transparent to AMF.

Based on the solutions above, sol#1 can solve the radio resource waste issue, but the downlink data transmission is not allowed even the UAV UE can select non-restricted frequency band in the NTZ to send and receive data as normal. Sol#2 uses the existing AoI mechanism to let AMF know the UAV UE presence in NTZ, but not whether the UAV UE can transmit or not, so this solution cannot solve the radio resource waste issue.

Besides the solutions above, there is another solution (sol#3) is to reuse MICO mode. If the UAV UE determines the transmission is not allowed before entering the NTZ area, the UAV UE sends Registration Request to AMF to request the use of MICO mode. Based on the Registration Request, the AMF activates MICO mode for the UAV UE. And then the UAV UE NAS requests the RRC layer to release the RRC connection based on the existing mechanism specified in clause 5.3.9 in TS 38.331. When RRC connection is released, the RAN node triggers N2 release procedure to transfer the UAV UE into IDLE mode. Because the MICO mode is already activated, the AMF considers the UAV UE is unreachable while the UAV UE CM state in the AMF is CM-IDLE. In this case, the following MICO mode mechanism specified in clause 5.4.1.3 in TS 23.501 is reused, as follows:

When the AMF indicates MICO mode to a UE, the AMF considers the UE always unreachable while the UE CM state in the AMF is CM-IDLE. The AMF rejects any request for downlink data delivery for UE in MICO mode and whose UE CM state in the AMF is CM-IDLE with an appropriate cause. For MT-SMS over NAS, the AMF notifies the SMSF that UE is not reachable, then the procedure of the unsuccessful Mobile terminating SMS delivery described in clause 4.13.3.9 of TS 23.502 [3] is performed. The AMF also defers location services, etc. The UE in MICO mode is only reachable for mobile terminated data or signalling when the UE is in CM-CONNECTED.

A UE in MICO mode need not listen to paging while in CM-IDLE.

Compared with the sol#1 and sol#2, the sol#3 solves the radio resource waste issue by reusing the existing MICO mode and UE initiated RRC connection release mechanisms and allows the UAV UE selecting non-restricted frequency band to transmit and receive as normal. Based on the analyses above, it is proposed to use the sol#3 as the wayforward.

Proposal#2: to solve the radio resource waste issue and to avoid impacts as much as possible, it is proposed that when the UAV UE is approaching the NTZ, based on the NTZ assistance information:

-	If the UAV UE determines the transmission is not allowed, before entering NTZ

-	The UAV UE sends Registration Request to AMF to request the use of MICO mode. Based on the Registration Request, the AMF activates MICO mode for the UAV UE.

-	The UAV UE NAS requests the RRC layer to release the RRC connection based on the existing mechanism specified in clause 5.3.9 in TS 38.331. When RRC connection is released, the RAN node triggers N2 release procedure to transfer the UAV UE into IDLE mode. Because the MICO mode is already activated, the AMF considers the UAV UE is unreachable while the UAV UE CM state in the AMF is CM-IDLE.

-	If the UAV UE determines the transmission is allowed, the UAV UE operates as normal.

2. Proposal

It is proposed to update the principle to support NTZ based on the proposals in clause 1.

* First change *

8.3	Interim Conclusion for Key Issue #3: Support of No Transmit Zones

8.3.1	General

The following principles are agreed to support NTZ.

1)	NTZ enforcement applies to the UE that is a UAV UE (aka an aerial UE) with aerial subscription.

2)	NTZs are supported for both LTE and NR.

3)	An NTZ may map to one or more cells or a fraction of a cell, or overlap different cells in a mobile operator network.

NOTE:	Replanning of existing tracking areas and cells based on the presence of NTZs is not assumed.

4)	No transmissions in the restricted frequency bands are allowed from a UAV UE located in the NTZ (in three dimensions to consider height restrictions), regardless of the service type.

5)	No UAV UE transmits or attempts to transmit in the restricted frequencies, location and height defined by NTZ. But UAV UEs can transmit/receive normally outside of these restrictions.

6)	UAV UEs supporting NTZ regulations are configured with NTZ information, so that these UAV UEs will be aware of the presence of NTZs.

7)	Reception of downlink data from the network by the UAV UE located in the NTZ is allowed given the regulatory requirements are not violated.

8)	When the UAV UE is approaching the NTZ, the UAV UE determines the transmission is allowed or not based on the NTZ assistance information and cell broadcasting information:

-	If the transmission is not allowed, before entering NTZ:

-	The UAV UE sends Registration Request to AMF to request the use of MICO mode. Based on the Registration Request, the AMF activates MICO mode for the UAV UE.

-	When the Regitration Accept is received, the UAV UE NAS requests the RRC layer to release the RRC connection based on the mechanism specified in clause 5.3.9 in TS 38.331. When RRC connection is released, the RAN node triggers N2 release procedure to transfer the UAV UE into IDLE mode. Because the MICO mode is already activated, the AMF considers the UAV UE is unreachable while the UAV UE CM state in the AMF is CM-IDLE.

-	Otherwise, the UAV UE operates as normal.

9)	RAN network should not be impacted to support NTZ, e.g. RAN network should not receive the NTZ information and should not be involved in the process of enforcing the NTZ.

* Second change *

8.3.2	Configuring/Provisioning NTZ assistance information to UAV UE and NTZ enforcement by UAV UE

The following NTZ assistance information (or NTZ policy) is configured/provisioned in the UAV UE.

-	A list of the restricted frequency band(s) with Geographical Area (including coordinates with longitude, latitude and height restrictions).

NOTE 1:	It is assumed UAV UE is configured with NTZ assistance information before accessing to the operator's network, if the local regulation requires NTZ.

The above NTZ assistance information is made available to the UAV UE by using one or more following ways:

-	pre-configured;

-	provided/updated by the AMF during the registration procedure;

-	provided/updated by the AMF during the UE Configuration Update procedure specified in clause 4.2.4.2 of TS 23.502 [4];

-	provided/updated by the MME during the attach procedure;

-	provided/updated by the MME during the TAU procedure;

-	provided/updated by the PCF;

-	provided/updated by the AF (e.g. NTZ AF, USS, TPAE).

NOTE 2:	Whether all options are required will be determined in normative phase.

The UAV UE is responsible for NTZ enforcement independently of the UAV UE 3GPP Release.

The UAV UE supporting NTZ (i.e. Rel-19 and onward UAV UEs) performs the following:

a)	enforces NTZ based on the configured/provisioned NTZ assistance information.

b)	upon applying NTZ enforcement, the UAV UE does not transmit any signalling or data.

c)	does not select/reselect any cells impacted entirely by an NTZ.

NOTE 3:	Whether indication about UAV UE's NTZ support to the AMF/MME is needed will be decided during normative phase. This work does not have RAN impacts.

NOTE 5:	Whether extension of Service Restrictions for NTZs using Restricted Transmission Area (RTA) mapped from the NTZ information is needed will be discussed during normative phase. This work does not have RAN impacts.

* End of changes *