3GPP TSG-SA2 Meeting #164	S2-2408629

# Maastricht, Netherlands, 19 August – 23 August, 2024          	  	  	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0566 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add Architecture for MWAB involved Location Services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on conclusion on architecture enhancement in clause 8 in TR 23.700-06, multiple architectures for MWAB operation support will be introduced in TS 23.501. Considering there are multiple new concepts or terms (e.g. broadcasting PLMN, BH PLMN) included in the architectures for MWAB operation, to align the description between TS 23.273 and TS 23.501 and to assist the understanding of the location service procedures involving MWAB, it is proposed to add architecture for MWAB involved Location Services which is based on the Architecture for MWAB operation specified in TS 23.501 and the Architecture for location service in clause 4.2 in TS 23.273. In general, the existing interfaces, configuration and functionalities are reused as much as possible in the architectures introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add architectures for MWAB involved location services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Architectures for MWAB involved location services is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change (all text new)* * *

4.2.X	Reference architecture for MWAB involved location services

4.2.X.1	General

Based on the architectures to support MWAB specified in TS 23.501[18] and the architectures for location services specified in clause 4.2, the reference architectures for MWAB involved location services in different scenarios are described.

4.2.X.2	Architecture for MWAB involved Location Services (MWAB-UE not roaming)

Figure 4.2.X.2-1 presents an example architecture for MWAB involved location service when no roaming is involved for the MWAB-UE.

Figure 4.2.X.2-1: Architecture for MWAB involved Location Services – MWAB-UE not roaming

NOTE 1:	The Broadcated PLMN is the serving PLMN of UE connected to the MWAB.

NOTE 2:	The BH PLMN is the serving PLMN of MWAB-UE.

NOTE 3:	The UE LMF discovers the GMLC in Broadcasted PLMNby query NRF. The GMLC in Broadcasted PLMN discovers the GMLC in BH PLMN (i.e. BH GMLC) by internal configuration or by query NRF.

4.2.X.3	Architecture for MWAB involved Location Services (MWAB-UE roaming with Local Breakout)

Figure 4.2.X.3-1 presents an example architecture for MWAB involved location service when MWAB-UE is roaming with a Local Breakout PDU Session for its operation.

Figure 4.2.X.3-1: Architecture for MWAB involved Location Services – MWAB-UE roaming with Local Breakout

NOTE 1:	The Broadcated PLMN is the serving PLMN of UE connected to the MWAB.

NOTE 2:	The BH PLMN is the serving PLMN of MWAB-UE.

NOTE 3:	The MWAB HPLMN is the HPLMN of MWAB-UE.

NOTE 4:	The UE LMF discovers the GMLC in Broadcasted PLMN (GMLC Broadcasted PLMN) by query NRF. The GMLC in Broadcasted PLMN discovers the GMLC in MWAB HPLMN (GMLC MWAB HPLMN) by internal configuration or by query NRF. The GMLC in MWAB HPLMN discovers the GMLC in BH PLMN (BH GMLC) by internal configuration or by query NRF.

4.2.X.4	Architecture for MWAB involved Location Services (MWAB-UE roaming with Home Routed)

Figure 4.2.X.4-1 presents an example architecture for MWAB involved location service when MWAB-UE is roaming with a Home Routed PDU Session for its operation.

Figure 4.2.X.4-1: Architecture for MWAB involved Location Services – MWAB-UE roaming with Home Routed

NOTE 1:	The Broadcated PLMN is the serving PLMN of UE connected to the MWAB.

NOTE 2:	The BH PLMN is the serving PLMN of MWAB-UE.

NOTE 3:	The MWAB HPLMN is the HPLMN of MWAB-UE.

NOTE 4:	The UE LMF discovers the GMLC in Broadcasted PLMN (GMLC Broadcasted PLMN) by query NRF. The GMLC in Broadcasted PLMN discovers the GMLC in MWAB HPLMN (GMLC MWAB HPLMN) by internal configuration or by query NRF. The GMLC in MWAB HPLMN discovers the GMLC in BH PLMN (BH GMLC) by internal configuration or by query NRF.

* * * End of Change * * *