3GPP TSG-SA2 Meeting #164	S2-2408630

# Maastricht, Netherlands, 19 August – 23 August, 2024          	  	  	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0567 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add 5GC-MT-LR procedure involving MWAB with MWAB-UE roaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on conclusion on architecture enhancement and on location service in clause 8 in TR 23.700-06 and location service, it is proposed to add the 5GC-MT-LR procedure involving MWAB with MWAB-UE roaming. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add 5GC-MT-LR procedure involving MWAB with MWAB-UE roaming. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC-MT-LR procedure involving MWAB with MWAB-UE roaming is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.x (new), 6.1.x.1 (new), 8.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change (all text new)* * *

6.1.x	5GC-MT-LR procedure involving Mobile gNB with Wireless Access Backhauling (MWAB)

6.1.x.1	5GC-MT-LR procedure with MWAB-UE Roaming

Figure 6.1.x.2-1: 5GC-MT-LR procedure involving MWAB with MWAB-UE Roaming

NOTE x:	The procedure can be applied to the architecture in Annex X.2 and Annex X.3.

1.	[Optional] The location services client sends a request to the GMLC (Broadcasted PLMN) for a location for the target UE identified by a GPSI or an SUPI. The UE may or may not be served by a MWAB.

2.	The GMLC (Broadcasted PLMN) invokes the Namf_Location_ProvidePositioningInfo service operation towards the UE AMF serving the target UE to request the current location of the target UE. The location procedure may be also triggered at the UE AMF for a 5GC-NI-LR as at step 1 in clause 6.10.1, or an event report sent by the UE for a deferred 5GC-MT-LR for periodic or triggered location as at step 25 in clause 6.3.1.

3.	The 5GC-MT-LR procedure as specified in clause 6.1.1 step 5-7 or clause 6.1.2 step 6-11 are performed with the difference that the LMF selection also consider LMF capability supporting MWAB involvement. If the AMF is aware that the target UE is served by a MWAB, it would select the LMF that can support the MWAB handling.

4.	UE LMF derives if any MWAB(s) is involved in the positioning of the target UE based on the cell-ID(s) used for positioning measurements in the step 3. If the LMF selects any cell for UE positioning measurements, and the UE LMF determines it needs to perform a TRP information exchange with the cell, it performs TRP Information Exchange procedure as specified in TS 38.455 [13]. As the MWAB can be mobile the UE LMF may need to determine an updated location of the MWAB by either performing step 5-7 (option 1) or performing step 8-12 (option 2) if option 1 is not feasible. If several MWABs were derived, then step 5-7 or step 8-12 are performed for each MWAB.

NOTE Y:	It is assumed that for the case of MWAB Cell Id change due to mobility, the OAM can timely update the Cell Id of the MWAB to the LMF so as to enable it to perform a proper TRP information exchange. Until the Cell Id of the MWAB is provided to LMF by OAM, the Cell of the MWAB is not used for location procedures.

5.	[Conditional] If option 1 is selected by UE LMF in step 4, the LMF initiates a NRPPa TRP Information Exchange procedure to request the updated location information of the TRP(s) associated with MWAB(s) by directing the TRP information exchange messages to the MWAB-gNB associated with the Cell ID(s).

6.	[Conditional] If step 5 is performed, the MWAB-UE initiates a 5GC-MO-LR procedure (as defined in clause 6.2) to get its location information. The MWAB-UE's location info is used by the co-located MWAB-gNB to determine the updated location for its TRP.

7.	[Conditional] If step 6 is performed, the MWAB reports its updated TRP location, e.g. TRP's geo-coordinate, velocity, and the time for obtaining them to the LMF.

8.	[Conditional] If option 2 is selected by UE LMF in step 4, the UE LMF selects GMLC (Broadcasted PLMN) by query NRF and invokes the Ngmlc_Location_ProvideLocation_Request service operation to the GMLC (Broadcasted PLMN) by providing the identifier of MWAB-UE, privacy check skip indication and a scheduled location time. The UE LMF determines the identifier of MWAB-UE based on the received the GPSI of the MWAB in a TRP information exchange when the MWAB was integrated as a TRP. The UE LMF may receive the scheduled location time in step 3. Otherwise, UE LMF may generate the scheduled location time, e.g. based on response time.

9.	[Conditional] If step 8 is performed, the GMLC (Broadcasted PLMN) selects GMLC (MWAB HPLMN) by query NRF or based on the internal configuration and invokes the Ngmlc_Location_ProvideLocation_Request service operation to the GMLC (MWAB HPLMN) by providing the identifier of MWAB-UE, privacy check skip indication and a scheduled location time.

10.	[Conditional] If step 9 is performed, the steps 2-23 in 5GC-MT-LR procedure in clause 6.1.2 are performed. The difference is that in step 2, based on the privacy check skip indication, the GMLC (MWAB HPLMN) skips privacy check.

11.	[Conditional] If step 10 is performed, the GMLC (MWAB HPLMN) sends the Ngmlc_Location_ProvideLocation_Response including the MWAB-UE's location to the GMLC (Broadcasted PLMN).

12.	[Conditional] If step 11 is performed, the GMLC (Broadcasted PLMN) sends the Ngmlc_Location_ProvideLocation_Response including the MWAB-UE's location to the UE LMF.

13.	The LMF performs one of the positioning procedures with the target UE described in clauses 6.11.1 and 6.11.2. To reduce the timing offset of the positioning measurements, the UE positioning may be scheduled with the same scheduled location time as the MWAB positioning in step 9. LMF sends the same time with the scheduled location time in step 8 to the target UE in clause 6.11.1. If Network Assisted procedure is used, the MWAB-gNB may provide the MWAB updated location and velocity information and the time obtained them to the LMF via NRPPa message as defined in TS 38.455 [15].

14.	The LMF uses the received location and velocity of the MWAB(s) when estimating the location of the target UE together with the measurements reports in step 13.

15.	The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the current location of the target UE.

16.	[Conditional] If step 1 is performed, the AMF returns the Namf_Location_ProvidePositioningInfo Response towards the GMLC (Broadcasted PLMN) to return the current location of the target UE.

17.	[Conditional] If step 1 is performed, the GMLC (Broadcasted PLMN) sends the location service response to the location services client.

* * * Second Change * * *

8.4.2.2	Ngmlc_Location_ProvideLocation service operation

Service operation name: Ngmlc_Location_ProvideLocation

Description: Provides UE location information to the consumer NF.

NOTE 1:	For deferred location request, this service operation is used to implicitly subscribe to the notification of the UE location information.

NOTE 2:	For bulk LCS service request from NEF to GMLC, this service operation is used to implicitly subscribe to the notification of UE location information

Input, Required: UE identifier (GPSI, SUPI, Application layer ID of the UEs for Ranging/Sidelink positioning, Internal Group Identifier or External Group Identifier), Client Type.

Input, Optional: Required QoS instance(s), Supported GAD shapes, UE privacy requirements, LCS Client Identification, Service type, indication of requiring reliable UE location information, Notification Target Address, Notification Correlation ID, Event Type (defined in clause 4.1a.5.1), Scheduled Location Time, request for user plane reporting to an LCS Client or AF, user plane address of an LCS Client or AF, user plane security information for an LCS Client or AF, cumulative event report timer, maximum number of user plane event reports to an LCS Client or AF, event report expected area, reporting indication, LPHAP indication, privacy check skip indication and:

-	For periodic event type, optional input further includes the time interval between successive location reports, the total number of reports, location QoS.

-	For area event type, optional input further includes target geographical area(s) with optionally associated required QoS instance, whether the event to be reported is the UE being inside, entering into or leaving the target area, the duration of event reporting, the minimum and maximum time intervals between successive event reports, the maximum event sampling interval, whether location estimates and associated location QoS shall be included in event reports, and whether only one location report is required or more than one.

-	For motion event type, optional input further includes the threshold linear distance, the duration of event reporting, the minimum and maximum time intervals between successive event reports, the maximum event sampling interval, whether location estimates and associated location QoS shall be included in event reports, and whether only one location report is required or more than one.

-	For ranging and Sidelink positioning, input further includes related UE identifiers for Located UEs or SL Reference UEs (GPSI, Application layer ID), requested location type (absolute locations, relative locations or distances and/or directions related to the UEs, velocities and relative velocities), Local Co-ordinate or Global Co-ordinate, and Coordinate ID.

Output, Required: Success/Failure indication

Output, Optional: Geodetic location, Local Location including Coordinate ID, civic location, age of location, Indoor/Outdoor indication, LOS/NLOS measurement indication, position methods used (in the case of success indication provided), failure cause (in the case of failure indication provided), achieved Location QoS Accuracy, the timestamp of the Location, Ranging result.

See clauses 6.3.1 and 6.8 for examples of usage of this service operation.

* * * End of Changes * * *