3GPP TSG-SA2 Meeting #164	S2-2408631

# Maastricht, Netherlands, 19 August – 23 August, 2024          	  	  	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5611 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of BH PDU Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The approved VMR_Ph2 WID aims to specify architecture enhancements, functionalities and procedures to support MWAB based on conclusions in clause 8 of TR 23.700-06. This CR is proposed to support the following conclusions: The following general principles are agreed to be used as the basis for normative work: - MWAB-gNB connection with OAM server is over the IP connectivity provided by the PDU session of MWAB-UE. - MWAB-gNB N2 interface with UE-AMF is provided over the IP connectivity offered by the PDU session of MWAB-UE. - MWAB-gNB N3 interface with UE-UPF is over the IP connectivity provided by the PDU session of MWAB-UE. - MWAB-gNB Xn interface with other gNBs can be supported over the IP connectivity provided by the PDU session of MWAB-UE. - Same or different PDU Sessions of MWAB-UE can be used for carrying the above traffic. NOTE 1: The use of multiple PDU Sessions for N2 vs. N3 vs. Xn vs. OAM access, and what specific configuration is needed will be addressed in normative phase. For the NOTE 1 above, this CR proposes to use OAM configuration or URSP rule to transfer the different types traffic to the same or different PDU Sessions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add BH PDU session support in new clause 5.x.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of VMR_Ph2 feature in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change (all text new)* * *

5.x.1	Support of Backhaul PDU Session

The data transferred over the BH PDU Session of MWAB-UE is associated with one of the classifiers as follows.

-	MWAB-gNB connection with OAM server.

-	MWAB-gNB N2 interface with UE-AMF.

-	MWAB-gNB N3 interface with UE-UPF.

-	MWAB-gNB Xn interface with other gNBs.

To determine BH PDU Session for different class of data, one of the following mechanisms can be used:

-	OAM configuration based BH PDU Session determination:

-	The OAM provides the configuration including the mapping between classifier(s) and S-NSSAI. When the data is generated by MWAB-gNB, it determines the S-NSSAI based on the classfier of the data and the OAM configuration, and then provides the data and the S-NSSAI to MWAB-UE.

-	Based on MWAB-gNB provided S-NSSAI, if the MWAB-UE has one or more PDU Sessions established corresponding to the S-NSSAI, the MWAB-UE routes the data in one of these PDU Sessions. Otherwise, the MWAB-UE requests a new PDU Session corresponding to the S-NSSAI.

-	URSP rule based BH PDU Session determination:

-	The URSP rule of MWAB-UE provided by PCF includes the mapping between traffic descriptor and S-NSSAI. The Connection Capabilities specified in TS 23.503[45] in the traffic descriptor includes one or more of the classifiers above.

-	When the data is generated by MWAB-gNB, it provides the data and the associated classifier to MWAB-UE. Based on the URSP rule and the classifier provided by MWAB-gNB, the MWAB-UE determines the S-NSSAI for the data. If the MWAB-UE has one or more PDU Sessions established corresponding to the S-NSSAI, the MWAB-UE routes the data in one of these PDU Sessions. Otherwise, the MWAB-UE requests a new PDU Session corresponding to the S-NSSAI.

NOTE X:	Same or different BH PDU Session(s) of MWAB-UE can be used for carrying data generated by MWAB-gNB.

NOTE Y:	OAM configuration based or URSP rule based method to be used is implementation specific.

* * * End of Change * * *