3GPP TSG-WG SA2 Meeting #164	S2- 2408647

# Maastricht, Netherland, Aug 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1455 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8: Support of UE based and network based rendering of Avatar communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FS_NG_RTC_Ph2 of Stage2 has concluded that network based rendering and UE based rendering need to be supported for IMS based Avatar communication. WI of NG_RTC_Ph2 was also approved at SA#104 in SP-240985. WT8 of the WI is to enhance IMS architectue and procedures to support IMS based Avatar communication, including functionalities and procedures to support UE based and network based rendering of IMS Avatar communication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new clause to describe the procedure of UE based rendering of IMS Avatar communication. - Add a new clause to describe the procedure of network based rendering of IMS Avatar communication. - Add the term of BAR to the Abbreviation clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of UE based rendering and network based rendering for IMS Avatar communication is missing in normative phase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.x(new), 3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change (all new text)* * * *

AC.x	Support of Avatar Communication

AC.x.1	General

This clause describes the enhancements of the IMS architecture and procedures to additionally support IMS based Avatar communication. UE based rendering and network based rendering are both supported.

AC.x.2	Architecture

Editor's note:	This clause will describe the enhanced architecture to support IMS based Avatar communication.

AC.x.3	Procedures

AC.x.3.1	UE based rendering Procedure

Figure AC.x.3.1-1: UE-A based rendering of IMS Avatar communication

Figure AC.x.3.1-1 depicts a UE-A based rendering of IMS Avatar communication procedure from UE-A perspective. The main steps in the procedure are as follows:

- It is decided to render Avatar data in UE-A side after rendering capability negotiation.

NOTE:	The rendering capability negotiation may be executed among XR AS, UE-A and UE-B. How the negotiation proceed is out of scope of this Release.

- UE-A initiates a re-INVITE adding media descriptors required by Avatar communication. RTP and/or application data channels are established for both UE-A and UE-B. Application data channel may be anchored in the MF/MRF.

- UE-A sends XR AS a request to retrieve UE-A’s based Avatar representation with the related Avatar ID.

- XR AS retrieves UE-A’s based Avatar representation from BAR with the related Avatar ID, and returns it to UE-A.

- UE-A captures the Avatar data of the user (e.g., pose, facial expression, etc.), and locally renders and encodes the based Avatar representation with the Avatar data.

- UE-A sends UE-B the rendered Avatar media over RTP or application data channel.

- UE-B decodes the Avatar media and displays Avatar representation of UE-A.

Figure AC.x.3.1-2: UE-B based rendering of IMS Avatar communication

Figure AC.x.3.1-2 depicts a UE-B based rendering of IMS Avatar communication procedure from UE-A perspective. The main steps in the procedure are as follows:

- It is decided to render Avatar data in UE-B side after rendering capability negotiation.

- UE-A initiates a re-INVITE adding media descriptors required by Avatar communication. RTP and/or application data channels are established for both UE-A and UE-B. Application data channel may be anchored in the MF/MRF.

- UE-A sends XR AS a request for Avatar data rendering with the related Avatar ID.

  - XR AS retrieves UE-A’s based Avatar representation from BAR with the related Avatar ID, and sends it to UE-B.

- UE-A captures the Avatar data of the user (e.g., pose, facial expression, etc.).

- UE-A sends UE-B the Avatar data with related Avatar ID over RTP or application data channel.

- UE-B renders the based Avatar representation of UE-A with the received Avatar data, and displays Avatar representation of UE-A.

AC.x.3.2	Network based rendering Procedure

Figure AC.x.3.2-1: Network based rendering of IMS Avatar communication

Figure AC.x.3.2-1 depicts a network based rendering of IMS Avatar communication procedure from UE-A perspective. The main steps in the procedure are as follows:

- It is decided to render Avatar data in network side after rendering capability negotiation.

- Application data channel is established between UE-A and XR AS.

- IMS initiates a media re-Negotiation request with UE-B, to connect UE-B's media stream to MF/MRF.

- UE-A sends XR AS a request for Avatar data rendering with related Avatar ID.

  - XR AS retrieves UE-A’s based Avatar representation from BAR with the related Avatar ID, and sends it to the MF/MRF.

- UE-A captures the Avatar data of the user (e.g., pose, facial expression, etc.).

- UE-A sends the Avatar data to the MF/MRF with related Avatar ID.

- MF/MRF renders and encodes the based Avatar representation of UE-A with the received Avatar data.

- MF/MRF sends the rendered Avatar media of UE-A to UE-B.

- UE-B decodes the Avatar media and displays Avatar representation of UE-A.

* * * * End of the 1st Change * * * *

* * * * Start of 2nd Change* * * *

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [68] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [68].

5GS	5G System

API	Application Program Interface

APN	Access Point Name

AS	Application Server

A2P	Application to Person

BAR	Based Avatar Repository

BCSM	Basic Call State Model

BG	Border Gateway

BGCF	Breakout Gateway Control Function

BS	Bearer Service

CAMEL	Customised Application Mobile Enhanced Logic

CAP	Camel Application Part

CDR	Charging Data Record

CN	Core Network

CS	Circuit Switched

CSCF	Call Session Control Function

CSE	CAMEL Service Environment

DC	Data Channel

DCAR	Data Channel Application Repository

DCMTSI	Data Channel Multimedia Telephony Service for IMS

DCSF	Data Channel Signalling Function

DHCP	Dynamic Host Configuration Protocol

DNN	Data Network Name

DNS	Domain Name System

ECN	Explicit Congestion Notification

ENUM	E.164 Number Mapping

GGSN	Gateway GPRS Support Node

GLMS	Group and List Management Server

GMLC	Gateway Mobile Location Centre

GRUU	Globally Routable User Agent URI

GUP	Generic User Profile

HSS	Home Subscriber Server

IBCF	Interconnection Border Control Function

I-CSCF	Interrogating-CSCF

IETF	Internet Engineering Task Force

IM	IP Multimedia

IMC	IMS Credentials

IMS	IP Multimedia Core Network Subsystem

IMS ALG	IMS Application Level Gateway

IMSI	International Mobile Subscriber Identifier

IN	Intelligent Network

IP	Internet Protocol

IPv4	Internet Protocol version 4

IPv6	Internet Protocol version 6

IP-CAN	IP-Connectivity Access Network

IP-SM-GW	IP Short Message Gateway

ISDN	Integrated Services Digital Network

ISIM	IMS SIM

ISP	Internet Service Provider

ISUP	ISDN User Part

IWF	Interworking Function

NP	Number portability

MAP	Mobile Application Part

MGCF	Media Gateway Control Function

MGF	Media Gateway Function

MRB	Media Resource Broker

MRFC	Multimedia Resource Function Controller

MRFP	Multimedia Resource Function Processor

NAI	Network Access Identifier

NAPT	Network Address Port Translation

NAT	Network Address Translation

NA(P)T-PT	Network Address (Port-Multiplexing) Translation-Protocol Translation

II-NNI	Inter-IMS Network to Network Interface

OSA	Open Services Architecture

P2A	Person to Application

P2P	Person to Person

P-CSCF	Proxy-CSCF

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCRF	Policy and Charging Rules Function

PDN	Packet Data Network

PDP	Packet Data Protocol e.g. IP

P-GRUU	Public Globally Routable User Agent URI

PLMN	Public Land Mobile Network

PSI	Public Service Identity

PSTN	Public Switched Telephone Network

QoS	Quality of Service

RAB	Radio Access Bearer

RFC	Request for Comments

SCS	Service Capability Server

S-CSCF	Serving-CSCF

SDP	Session Description Protocol

SGSN	Serving GPRS Support Node

SLF	Subscription Locator Function

SNPN	Stand-alone Non-Public Network

SSF	Service Switching Function

SS7	Signalling System 7

SIM	Subscriber Identity Module

SIP	Session Initiation Protocol

S-GW	Signalling Gateway

TAS	Telephony Application Server

T-GRUU	Temporary Globally Routable User Agent URI

THIG	Topology Hiding Inter-network Gateway

TrGW	Transition Gateway

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

URL	Universal Resource Locator

USIM	UMTS SIM

* * * * End of 2nd Change * * * *