3GPP TSG-WG SA2 Meeting #164	S2-2408649

# Maastricht, Netherland, Aug 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1456 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#6: Support of Standalone IMS DC Session |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | WI of NG_RTC_Ph2 was approved at SA#104 in SP-240985. WT6 of the WI is to enhance IMS procedures to support Standalone IMS data channel session, including establishing IMS session with standalone bootstrap DC or a combination of standalone bootstrap DC and application DC, changing an IMS session with audio/video/DC media components to a standalone IMS DC session and adding audio/video media components to an established standalone IMS DC session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a new clause to describe the procedure of standalone bootstrap DC. - Add a new clause to describe the procedure of combination of standalone bootstrap DC and application DC. - Add a new clause to describe the procedure of IMS DC session mode change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of Standalone IMS data channel session is missing in normative phase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change (all new text)* * * *

AC.7.x	Standalone data channel procedure

AC.7.x.1	Standalone bootstrap DC

An IMS session with only standalone bootstrap data channel is used for downloading application list and applications from DCSF, which could be further updated with application data channel.

When the UE initiates an IMS session with standalone bootstrap data channel:

-	The originating UE follows existing procedures to establish bootstrap data channel as specified in clause AC.7.1 for standalone data channel session establishment with the addition that the UE may only include bootstrap data channel media components when generating SDP Offer in the initial INVITE request.

-	The request URI in the INVITE request for standalone bootstrap data channel is set to a specific PSI indicating standalone bootstrap data channel which is used by IMS AS to target to the DCSF. The PSI is configured on the UE. The DCSF serving the UE instructs IMS AS to terminate the session and reserve only the originating side media resource based on the received DC media components.

AC.7.x.2	Combination of Standalone bootstrap DC and application DC

An IMS session with a combination of standalone bootstrap data channel and application data channel is used to establish application data channel for a downloaded application to the remote UE, which allows the remote UE to download the application if not yet downloaded after accepting the bootstrap DC, and update the session to use the application in the same session.

When the UE initiates an IMS session with combination of standalone bootstrap data channel and standalone application data channel:

- The originating UE only includes DC media components for both bootstrap data channel and application data channel when generating SDP offer in the initial INVITE request, with the request URI to be the IMPU of the remote UE.

- If the terminating UE has downloaded the application and if the user accepts the session, the terminating UE responds with the SDP answers for application data channel.

- If the terminating UE has not yet downloaded the application, the terminating UE responds with the SDP answer for bootstrap data channel and set ports of application data channel to specific value and downloads the application after the bootstrap data channel is established. After receiving the response, the originating UE updates the session with the SDP offer of application data channel again and the terminating UE responds the SDP answer of application data channel after the application is downloaded.

NOTE:	In the above scenario, the specific value of ports of application data channel is defined in SA4 and needs to be further aligned.

AC.7.x.3	Session mode change

A normal IMS session with audio/video/DC media components can be changed to standalone IMS DC session without audio/video components. A standalone IMS DC session without audio/video components can also be changed to IMS session with audio/video components.

- An SDP Offer from originating UE can be used to indicate the session mode change request of the IMS DC session.

- Whether to change the session mode of current IMS DC session is determined according the response with SDP answer from the terminating UE.

- A normal IMS session can be changed to standalone IMS DC session by removing the audio/video components from the normal IMS session.

- A standalone IMS DC session can be changed to normal IMS session by adding audio/video components to the standalone IMS DC session.

* * * * End of the 1st Change * * *