3GPP TSG SA WG2#164	            S2-2408652

Maastricht, 19-23 August 2024 	(revision of)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5617 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MWAB: authorization aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The approved VMR_Ph2 work item (SP-240632) is set to specify the architecture enhancements, functionalities and procedures to support MWAB based on conclusions of TR 23.700-06 (clause 8). This CR introduces the descriptions on how the MWAB is authorized to operate. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause added to describe how the MWAB is authorized to operate. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MWAB feature description on how it is authorized is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (new) 5.X.3, (new) Annex X.Y,(new) Annex X.Y.1,(new) Annex X.Y.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.X.3	Authorization aspects

#### 5.x.3.1	General

There are two aspects related to the authorization of a MWAB. One aspect is the authorization of the MWAB-UE to establish a backhaul PDU session in a BH PLMN/SNPN. The other aspect is related to the authorization of the MWAB-gNB to operate as a gNB logically belonging to a MWAB Broadcasted PLMN/SNPN, which the MWAB-gNB's cell(s) broadcasts. These two authorizations are managed by different entities, and the BH PLMN/SNPN and MWAB Broadcasted PLMN/SNPN can be different. The Authorization aspects which can be implemented using existing mechanisms are illustrated in Annex X.Y.

Annex X (informative): 
MWAB

# X.Y	Authorization aspects

### X.Y.1	MWAB-UE authorization

The BH PLMN authorizes the MWAB-UE based on its subscription information stored in its HPLMN UDM which included dedicated S-NSSAI(s)+DNN for MWAB operation. Such dedicated network slices S-NSSAI(s) and DNN(s) shall be part of the subscription information for the MWAB-UE. The MWAB-UE may be configured with dedicated S-NSSAI(s)+DNNs for MWAB operation.

The MWAB-UE registers to the BH PLMN following procedures defined in clause 4.2.2 of TS 23.502 [3]. The authorization of the MWAB-UE to establish connectivity in a BH PLMN/SNPN is based on whether it is allowed to register with network slice(s) dedicated to the MWAB-UEs operation in the BH PLMN/SNPN, and whether it can establish BH PDU session(s) in the BH PLMN/SNPN using the dedicated network slice(s) and DNN.

The MWAB-UE authorization supports time based or location-based control. For time-based control, existing time-based network slice subscription control can be reused (including the enhancements in clause 5.15.16). For location-based control, existing mechanism, e.g. service area restriction, LADN based control (see e.g. clauses 5.6.5.and 5.6.5a) can be reused.

NOTE:	When the MWAB-UE's authorization status changes from "authorized" to "unauthorized", the dedicated S-NSSAI for MWAB operation are removed from the UE subscription and a UE subscription update step happens. If the AMF is configured to do so for the  the dedicated S-NSSAI/DNN for MWAB operation, the AMF may determine to delay the corresponding PDU session release based on an operator configuration (e.g. local configured timer to allow time for the MWAB UE to trigger the MWAB-gNB to handover the UE it serves).

The MWAB-UE may be connected to the BH PLMN for other services, even if the BH PDU Session(s) is not authorized.

The AMF may choose to de-register the MWAB-UE, if MWAB-UE's subscription does not allow it to be registered to the BH PLMN.

### X.Y.2	MWAB-gNB authorization

OAM of the MWAB Broadcasted PLMN/SNPN determines when/where the MWAB-gNB can operate or when/where it needs to shut down. The OAM of the MWAB Broadcasted PLMN may pre-configure the MWAB-gNB to turn on/shut down accordingly. When the MWAB-gNB is no longer authorized to operate by OAM of the MWAB Broadcasted PLMN/SNPN, the MWAB-gNB should hand over the UE(s) it serves to other cells.

NOTE: 	It is expected the HPLMN ensures that the BH PDU sessions authorization for MWAB UE does not interfere with the ability of the OAM of the MWAB Broadcasted PLMN/SNPN to manage the MWAB-gNB , .e.g. the S-NSSAI(s) for the BH PDU session(s) of the MWAB-UE is(are) maintained long enough in the MWAB-UE subscription and are available for the MWAB-UE for a location and time that is sufficient for the MWAB-UE so the BH PDU sessions are available for the MWAB-gNB to be able to perform OAM controlled shut down, which may include handing over UEs it serves.

When the MWAB-gNB stops operating under OAM control, the MWAB-gNB can  also trigger the MWAB-UE to release the BH PDU sessions.

End of CHANGES