3GPP TSG-WG SA2 Meeting #164	S2-2408660

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1186 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VFL Procedures for client discovery and sample/feature alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introducing support for VFL client discovery and Sample and Feature Alignment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - New clause for VFL client discovery, Sample and Feature Alignment - New service operation is proposed to define new NEF service operations and NWDAF service operations for VFL. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL client discovery, Sample and Feature Alignment will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.2X	VFL Sample and Feature alignment with AF as a VFL server

- VFL server (AF) may determine to use VFL model training and may send Nnef_VFLDiscovery Request to discover VFL clients to NEF which includes Analytics ID (e.g. Observed Service Experience), filter information (e.g. requested ML model information), VFL client capability.

Editor’s Note: Whether and how VFL server aware Analytics ID is FFS.

2.	Based on the request from AF, the NEF authorizes the request information and applies parameter mapping (e.g. geographical area mapping to TA(s)/ Cell-id(s)). The NEF may request NRF discovery of VFL clients to support requested Analytics ID and filter information.

3.	NRF returns list of NWDAF(s) as VFL clients to the NEF which support analytics ID and satisfy filter information. The response may include VFL client’s capability information (supported ML model per analytic ID, additional information for ML model (e.g., input features, output features), Service Area, etc) and the requested analytic ID.

4.	NEF may send Nnef_VFLDiscovery Response to AF which includes list of VFL clients with VFL client’s capability information. After receiving list of VFL clients, VFL server may select list of VFL clients as construct list of candidate VFL for VFL operation of analytic ID based on VFL client’s capability information.

Editor’s Note: When multiple NWDAFs as VFL clients are involved, how AF may communicates with multiple NWADF is FFS.

5.	VFL server (i.e., AF) may send Nnef_VFLAlignment Request to NEF for sample alignment and/or feature alignment. The request may include analytic ID, requested ML models, list of sample UEs and list of features for alignment for VFL training for analytic ID. (For example, for “observed service experience” analytic service in clause 6.4, list of features may include all or part of input data in clause 6.4.2. and may include part of output analysis such as Network Slice instance service experience prediction, Application service experiences in clause 6.4.3, etc)

6.	NEF may map the external UE IDs in the list of sample UEs to the internal UE IDs.

7.	NEF may send a Nnwdaf_MLModelAlignment Request to VFL clients which include analytic ID, requested ML models, internal UE IDs of list of sample UEs and list of features for alignment for VFL training for analytic ID.

8.	VFL clients may collect input data related to the internal UE IDs and list of features from associated NFs for data collection and verify the availability of features for list of sample UEs. (For example, for “observed service experience” analytic service in clause 6.4, VFL clients may collect input data from SMF and UPF, and supported input features from VFL clients may be part of information in table 6.4.2-2 in clause 6.4.2 and output features may be Network Slice instance service experience prediction, Application service experiences in clause 6.4.3, etc)

NOTE: The availability of data may vary per UE’s status (UE’s mobility state, UE’s location, User Consent or Privacy Profile, etc).

9.	VFL clients may send Nnwdaf_MLModelAlignment Response which include the analytic ID, supported ML models, list of UE IDs and list of features available at VFL clients for data collection.

10.	NEF may map internal UE IDs to the external UE IDs for the list of UE IDs included in the response from VFL clients.

11.	NEF may send Nnef_VFLAlignment Response which include analytic ID, supported ML models, list of UE IDs, and list of available features at each VFL clients.

12.	AF may check whether the supported samples and features are in line with the requested ML models for analytic ID.

13. As a result of sample alignment and feature alignment, VFL server may determine global ML model, list of VFL clients, and initial ML models for each VFL client based on available list of UEs and available features and VFL capability of each VFL client.

# * * * * End of changes * * * *