3GPP TSG-WG SA2 Meeting #164	S2-2408663

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1188 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enabling VFL Inference in 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add support for VFL Inference in 5GC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new clause to describe VFL Inference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL Inference will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.2X	VFL Inference with AF as Server

Figure 6.2X-1: Procedure for VFL inference with AF as VFL server

- VFL server (i.e. AF) may determine to perform VFL inference using the VFL well-trained ML model which is represented by VFL model correlation ID.

- When there is not a VFL client available for VFL inference which has been involved for VFL training of the VFL model correlation ID, VFL server (i.e. AF) may send Nnef_VFLDiscovery Request to NEF to discover VFL clients for VFL inference by indicating Analytics ID, External UE IDs, filter information (e.g. Area of Interest, requested ML model), VFL client capability. In the discovery request, filter information (e.g., Area of Interest, requested ML model) is determined based on the stored ML model information of VFL model correlation ID.

Editor’s Note : Whether and how VFL server aware Analytics ID is FFS.

Editor’s Note : When multiple NWDAF is involved, how AF communicate with multiple NWADF is FFS.

NOTE:	NEF is needed only when the AF is untrusted AF.

2.- 4. NEF may perform VFL client discovery procedure as specified in subclause X.

5.	NEF may response Nnef_VFLDiscovery Response to NEF which include list of VFL clients satisfy filter information. VFL server(i.e., AF) may select a VFL clients to join VFL inference.

6.	VFL server may initiate VFL inference by sending NNef_VFLInference Request as a VFL inferencing request to VFL client(s) via NEF including the external UE IDs that are used by the VFL server as the data sample for the inference, the VFL model correlation ID to indicate which VFL model should be used by the VFL client to generate local inference result, and the feature profile to indicate what are the feature used by the VFL client to generate the intermediate inference result.

When VFL client(s) are selected for VFL inference, VFL server may provide analytic ID, ML model ID and additional ML model information which include information of locally trained ML model to be used for local inferencing for each newly selected VFL client.

6.	NEF maps the external UE IDs to the internal UE IDs based on the information exchanged with the UDM.

7.	NEF sends the Nnwdaf_MLModel Inference Request as a VFL inference Request to the VFL client NWDAF including the internal UE IDs, VFL model correlation ID. For some VFL clients, if NEF received additional ML model information in step 6, NEF may include analytic ID, ML model ID, and additional ML model information.

8.	When receiving a Nnwdaf_MLModel Inference Request, VFL client may derive the local ML model based on the VFL model correlation ID or apply the received additional ML model information to construct local ML model. And VFL client may generate local inference result using local data, which may include newly collected data for requested UE IDs and local ML model.

9.	VFL client NWDAF(s) may send the Nnwdaf_MLModel Inference Response to the NEF including the inference results and the VFL model correlation ID.

10.	NEF may send Nnef_VFLInference Response to the AF including the inference results from VFL client NWDAF(s) and the VFL model correlation ID.

# * * * * End of changes * * * *