3GPP TSG SA WG2 Meeting # 164	S2-2408665

# Maastricht, Netherlands,  19 - 23 August, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0568 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LMF selection for AI/ML based positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support of AI/ML based positioning may greately improve Position accuracy at the LMF and therefore this capability should be considered in LMF selection. Not all LMFs support AI/ML based positioning, but any AIML-capable LMF should support traditional methods as well. Thus, since it is expected that the accuracy of Positioning mechanisms at the LMF when using AI/ML based positioning will increase when compared to legacy methods (as per TR 38.843, clause 8), the selection of AIML-capable LMF should be prioritized. Whether the LMF uses AIML based method or not is left to LMF decision. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AIML capability is considered in LMF selection. And AIML-capable LMF is prioritized in LMF selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIML-capable LMF may not be selected and the positioning result is inferior. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 6.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## 5.1	LMF Discovery and Selection

LMF selection functionality is supported by the AMF to determine an LMF for location estimation of the target UE or Ranging/Sidelink Positioning between Target UE and SL Reference UE. The LMF selection functionality is also supported by the LMF if it determines that it is unsuitable or unable to support location for the current UE access network or serving cell for the deferred 5GC-MT-LR procedure for periodic, triggered location events, or Modification of User Plane Connection. The LMF selection functionality may also be supported by the GMLC and the GMLC provides the selected LMF ID to AMF.

LMF reselection is a functionality supported by AMF when necessary, e.g. due to UE mobility.

The LMF selection/reselection may be performed at the AMF or LMF or GMLC based on the locally available information i.e. LMF profiles are configured locally at AMF or LMF or GMLC, or by querying NRF.

The following factors may be considered during the LMF selection:

-	LCS client type.

-	Requested Quality of Service information, e.g.:

-	LCS accuracy,

-	Response time (latency),

-	Access Type (3GPP /N3GPP).

NOTE 1:	Location methods may differ depending on the Access Type, e.g. in the case of WLAN Access Location determination may just correspond to retrieval of IP addressing information from the N3IWF/TNGF; As another example, for Wireline access, Location determination may just correspond to retrieval of geo coordinates corresponding to a GLI as defined in clause 4.7.8 of TS 23.316 [21] or a HFC Node ID.

-	RAT type (e.g. 5G NR, eLTE, or any of the RAT Types specified for NR satellite access) and/or the serving AN node (i.e. gNB or NG-eNB) of the target UE.

-	RAN configuration information.

-	LMF capabilities, including:

-	the support of Uu based positioning as defined in clause 4.3.8;

-	Ranging/Sidelink positioning as defined in clause 4.3.8 of TS 23.586 [40];

-	LMF user plane positioning capabilities (the capability to support LCS-UPP);

-	the support of AI/ML based positioning methods.

-	LMF load.

-	LMF location.

-	Indication of either a single event report or multiple event reports.

-	Duration of event reporting.

-	Network slicing information, e.g. S-NSSAI and/or NSI ID.

-	LMF Service Area consisting of one or more TA(s).

-	Supported GAD shapes.

-	Support LCS when MBSR is involved.

-	Requested UE has maintained user plane connection with certain LMFs.

When receiving a NAS message from UE, including an LMF ID together with a LPP message (refer to step 25 in clause 6.3.1 for event reporting for a deferred 5GC-MT-LR), AMF sends the LPP message to the LMF, as indicated by the LMF ID.

NOTE 2:	Description on how UE encapsulates the LMF ID in the NAS message is documented in TS 24.571 [36].

UDM may store the LMF ID in UE subscription data. During the location procedure, GMLC received the LMF ID from the UDM and provides it to AMF.

GMLC may be configured with the following parameters:

-	LMF ID and/or

-	per group ID and its correlating LMF ID.

The AMF may use locally provisioned configuration to determine LMF based on UE identify or its group information.

NOTE 3:	It is AMF implementation specific for the priority of different selection criteria from GMLC, AMF and LMF.

When the GMLC receives a MT location request from LCS client/AF, GMLC determines the LMF ID based on the configured parameters for an LCS Client/AF. In case a group ID is provided or derived from the location request, GMLC determines the correlating LMF ID based on the provisioned group ID.

GMLC may have configuration with one or several dedicated LMF ID(s), irrelevant to any LCS client/AF. When the GMLC receives a MT location request from LCS client/AF, GMLC only determines the LMF ID within the configuration for all LCS client/AF.

NOTE 4:	When AMF can't access the LMF instance of the LMF ID, by default, AMF replies to corresponding error to GMLC. GMLC could retry or fail the request accordingly; with explicit configuration to serve as backup selection in this, AMF can also select different configured LMF instance.

* * * Next Change * * *

## 6.1	5GC-MT-LR Procedure

### 6.1.1	5GC-MT-LR procedure for the regulatory location service

Figure 6.1.1-1 illustrates the general network positioning for the LCS clients external to the PLMN for the regulatory location service for non-roaming scenario. In this scenario, it is assumed that the target UE is identified using an SUPI or GPSI.

This procedure is applicable to a request from an LCS client for a current location of the target UE, and it is assumed that the LCS client is authorised to use the location service and no privacy verification is required.

Figure 6.1.1-1: 5GC-MT-LR procedure for the regulatory location service

1.	The external location services client sends a request to the GMLC for a location for the target UE identified by an GPSI or an SUPI. The request may include the required QoS, UE unaware indication and Supported GAD shapes. If location is required for more than one UE, the steps following below may be repeated and in that case the GMLC shall verify whether the number of Target UEs in the LCS request is equal to or less than the Maximum Target UE Number of the LCS client. If Maximum Target UE Number is exceeded, the GMLC shall reject the LCS request, the step 2-10 are skipped, and then GMLC respond to the client with proper error cause in the step 11.

2.	The GMLC invokes a Nudm_UECM_Get service operation towards the home UDM of the target UE to be located with the GPSI or SUPI of this UE.

3.	The UDM returns the network addresses of the current serving AMF.

NOTE:	For backward compatibility, the GMLC can use a Nudm_SDM_Get service operation to retrieve the SUPI of the target UE from a Rel-15 UDM as defined in clause 5.2.3.3.2 of TS 23.502 [19].

4.	The GMLC invokes the Namf_Location_ProvidePositioningInfo service operation towards the AMF to request the current location of the UE. The service operation includes the SUPI, and client type and may include the required QoS, UE unaware indication and Supported GAD shapes.

5.	If the UE is in CM IDLE state and UE unaware indication is not included, the AMF initiates a network triggered Service Request procedure as defined in clause 4.2.3.3 of TS 23.502 [19] to establish a signalling connection with the UE.

If UE unaware indication is received by AMF, and the UE is in CM_IDLE state or in RRC_INACTIVE state (if known by AMF by requesting the NG-RAN to report RRC state information), the steps 6 - 9 are skipped.

6.	The AMF selects an LMF based on the available information as defined in clause 5.1 or based on AMF local configuration. The LMF selection takes the 5G-AN currently serving the UE into account. The selection may use a NRF query. The AMF may prioritize the selection of the LMF capable of AI/ML based positioning.

7.	The AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF to request the current location of the UE. The service operation includes a LCS Correlation identifier, the serving cell identity of the Primary Cell in the Master RAN node and the Primary Cell in the Secondary RAN node when available based on Dual Connectivity scenarios, and the client type and may include an indication if UE supports LPP, the required QoS, UE Positioning Capability if available, UE unaware indication and Supported GAD shapes. If any of the procedures in clause 6.11.1 or clause 6.11.2 are used the service operation includes the AMF identity.

8.	The LMF performs one or more of the positioning procedures described in clauses 6.11.1, 6.11.2 and 6.11.3. During this step the LMF may use the Namf_Communication_N1N2MessageTransfer service operation to request the transfer of a Positioning related N1 message to the UE or the transfer of a Network Positioning message to the serving NG-RAN node (gNB or NG-eNB) for the UE. The LMF shall determine a geographical location and optionally a location in local coordinates.

If UE unaware indication is received from AMF in step 7, the LMF selects Uplink Positioning method to obtain UE location and performs the positioning procedure described in clause 6.11.2.

9.	The LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF to return the current location of the UE and UE Positioning Capability if the UE Positioning Capability is received in step 8 including an indication that the capabilities are non-variable and not received from AMF in step 7. The service operation includes the LCS Correlation identifier, the location estimate, its age and accuracy and may include information about the positioning method and the timestamp of the location estimate.

If UE unaware indication is received from AMF in step 7 and the NG-RAN rejects the Network Positioning message as described in clause 6.11.2 with appropriate rejection cause (e.g. UE cannot be paged) in step 8, the LMF rejects the Nlmf_Location_DetermineLocation Request with appropriate rejection cause (e.g. UE cannot be paged).

10.	The AMF returns the Namf_Location_ProvidePositioningInfo Response towards the GMLC/LRF to return the current location of the UE. The service operation includes the location estimate, its age and accuracy and may include information about the positioning method and the timestamp of the location estimate. The AMF stores the UE Positioning Capability in UE context when received from LMF.

If the AMF decides to skip steps 5 - 9 in step 5 based on the UE unaware indication and the UE state or the LMF rejects the Nlmf_Location_DetermineLocation Request with appropriate rejection cause (e.g. UE cannot be paged), the AMF may convert any last known location in the form of a cell ID or TAI into geographical information based on TS 23.032 [8] and as defined in clause 6.2.6.2.5 of TS 29.518 [16] and clauses 5.4.4.7, 5.4.4.8 and 5.4.4.9 of TS 29.571 [33] and answer to the GMLC with the geographical information together with the age of this location.

11.	The GMLC sends the location service response to the external location services client.

* * * End of Changes * * *