SA WG2 Meeting SA2#164	S2- 2408685

19–23 August 2024, Maastricht, Netherlands	(Revision of S2-24xxxx)

# Source:	CATT, Sateliot

# Title:	Revised WID on Integration of satellite components in the 5G architecture Phase 3

# Document for:	Approval

# Agenda Item:	30.1

TSG SA Meeting #104	SP-240986

18-21 June 2024, Shanghai, China	(Revision of SP-240628)

# Source:	SA WG2

# Title:	New WID on Integration of satellite components in the 5G architecture Phase III

# Document for:	Approval

Agenda Item:	6.2.2

3GPP TSG-SA WG2 Meeting #163	S2-2407368

Jeju, Korea, May 27 – May 31, 2024

# Source:	Thales, CATT

# Title:	New WID on Integration of satellite components in the 5G architecture Phase III

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: Integration of satellite components in the 5G architecture Phase 3

Acronym:	5GSAT_Ph3_ARCH

Unique identifier:	1040027

Potential target Release:	Rel-19

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_5GSAT_Ph3_ARCH | SA2 | 1010033 | Study on Integration of satellite components in the 5G architecture Phase III |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 80048 | Stage 1 of 5GSAT | Service requirements of satellite access in 5G |
| 960016 | Stage 1 of 5GSAT: FS_5GSAT_Ph3 Study on satellite access - Phase 3 | Defines new services requirements for R19 |
| 80026 | Study on architecture aspects for using satellite access in 5G | Unresolved key issue leftover from R17 |
| 940074 | 5GC enhancement for satellite access Phase 2 | SA2 Rel-18 SID for 5G System enhancement for satellite access |
| 980014 | 5GC/EPC enhancement for satellite access Phase 2 | SA2 Rel-18 WID for 5G System enhancement for satellite access |
| 940060 | Study on 5G System with Satellite Backhaul | SA2 Rel-18 SID for 5G System with Satellite Backhaul |
| 970018 | 5G System with Satellite Backhaul | SA2 Rel-18 WID for 5G System with Satellite Backhaul |

# 3	Justification

Integration of satellite components into EPS and 5GS has been studied in Rel 16, Rel 17 and 18.

As part of Rel-19, SA1 developed a TR 22.865 that captures a set of use cases and potential service requirements related to the 5G system with satellite access considering new capabilities:

1.	Store and forward satellite operation for delay-tolerant communication services: Store and Forward (S&F) operation is an operation mode of a 5G system with satellite-access, where the 5G system can provide some level of service (in storing and forwarding the data) when satellite connectivity is intermittently/temporarily unavailable, e.g. to provide communication service for UEs under satellite coverage without a simultaneous active feeder link connection to the ground segment. This is particularly relevant for delay-tolerant IoT services via NGSO space segment.

2.	UE-Satellite-UE communication without going through the ground network: In some scenarios, UEs need to communicate using satellite access without going to the ground network in order to avoid long delays and limited data rate as well as reducing the consumption of backhaul resources.

R17 and R18 assume transparent mode satellite access.

In Release 19 SA2 intends to address three items.

(Item 1) Regenerative payloads, i.e.: embedding RAN and CN 3GPP functions, are being considered by SA1 in Rel-19 for example to support store and forward satellite operation (e.g. for mMTC services) or to support Inter Satellite Links (e.g. for eMBB services).

(Item 2) S&F feature, very relevant for IoT use cases will require EPC enhancement.

(Item 3) UE-Satellite-UE communication, will require 5GS enhancements.

These three items, have been studied in Study on Integration of satellite components in the 5G architecture Phase III (unique ID 1010033) with results and conclusions documented in TR23.700-29 which also provides guidance on required normative work.

# 4	Objective

The objective of this work item is to produce normative specifications fulfilling the results of the feasibility study as documented in the conclusions section of TR 23.700-29 v 1.0.0 for each of the three key issues investigated.

For Key Issue 2 (WT2) "Support of Store and Forward satellite operation"

-	Store and Forward is only supported by EPS.

SA2 shall specify means for the EPS to:

-	inform the UEs whether S&F Satellite operation is applied (when feeder link is not available)

-	define the timers and satellite list to facilitate the completion of Attach or TAU procedure when these procedures cannot be completed by the MME

-	provide to the UE a monitoring list that the UE uses to determine which satellites to use to perform MO/MT data/signalling transfer with the CN.

-	manage potential inconsistent status in HSS due to multiple registration attempts

-	indicate to external SCS/AS whether the UE is registered in S&F Mode and the estimated delivery time

For Key Issue 3 (WT3) "Support of UE-Satellite-UE communication"

-	In this release roaming is not supported and the two UEs engaged in a IMS communication belong to the PLMN providing access via satellite link.

SA2 shall specify means for the 5GS to:

-	Support determination of possibility of setting up a UE-Satellite-UE communication

-	Subject to SA3-LI feedback:

- 	Define, as appropriate, architectural enhancement to support use of UPF or use of UPF and IMS AGW onboard the satellite.

-	Define, as appropriate, enhancements of IMS procedures and functions when serving satellite of either UE changes including:

-	Minimisation of  service interruption

-	disabling UE-Satellite-UE communication

-	Support regulatory service requirements, and PLMN country or regional service restrictions.

NOTE: support of Lawful interception will require the validation from SA3-LI

# 5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Functional description of features as stated in the objectives | SA#106(Dec24) |  |
| TS 23.502 | Updates to procedures | As above |  |
| TS 23.503 | Update of policy framework | As above |  |
| TS 23.682 | Support for notification of ongoing S&F operations and expected delivery time to SCS and AS. | As above |  |
| TS 23.228 | Enhancements of IMS procedures and functions | As above |  |
| TS 23.401 | Update of Attach &TAU procedures | As above |  |

# 6	Work item Rapporteur(s)

Primary rapporteur: Hucheng Wang (CATT) wanghucheng@catt.cn

Secondary rapporteur: Ramon Ferrús (Sateliot) ramon.ferrus@sateliot.space

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

Potential impacts on security are addressed by SA3 and SA3-LI with regards to lawful interception.

Potential impacts on OAM and charging are to be addressed by SA5

Support of regenerative satellite access (KI1/WT1) is due to be developed in RAN3 with SA2 performing alignment if required

# 9	Supporting Individual Members

Supporting IM name

Thales

CATT

SHARP

TNO

Novamint

ESA

Deutsche Telekom

NOKIA

DISH Network

FirstNet

Intel

LG Electronics

Tencent

OPPO

Sateliot

ETRI

China Mobile

Samsung

China Telecom

Xiaomi

CSCN

Apple

Eutelsat

Orange

Leonardo

Mediatek

NEC

BT