3GPP TSG-WG SA2 Meeting # 164	S2-2408711

# August 19 - 23, 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4932 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting UPF event exposure subscription directly using UE’s IP address |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per the Rel-19 conclusions for FS_UPEAS_Ph2, a consumer of UPF event exposure services can use the UE’s IP address to subscribe direct for the UserDataUsage Events. The consumer of the Event Exposure service first needs to request the NRF for the UPF which caters the target UE IP address (NRF stores the IP address range in the UPF’s NF profile) Using the information of serving UPF, consumer of UPF event exposure services can directly subribe to the particular UPF by providing target as UE IP address and relevant Event ID(s) that is need to be subscribed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new clause under clause 4.15.4.5 to reflect the new procedure flow to use for direct subscription to UPF event exposure service if UE’s IP address is known. AF/NEF are added as NFs that can use the Nupf_EventExposure Subscribe/Unsubscribe service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.1, 4.15.4.5.x (new clause), 5.2.26.1, 5.2.26.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 01: - Removed port number and clarified that cases when NAPT is deployed is not specified. - Added impacts on clause 5.2.26.2.1 |  |  |  |  |  |  |  |  |

* * * Change 1 * * *

#### 4.15.4.5	Exposure of Events from UPF for UPF Data Collection

##### 4.15.4.5.1	General

This clause contains the detailed description and the procedures for how the UPF event exposure service (see clause 5.2.26.2) is used for UPF data collection.

The list of NF consumer which may receive UPF event notifications is defined in clause 5.8.2.17 of TS 23.501 [2].

To get exposure data from UPF, NF consumer may subscribe to the UPF directly or indirectly via SMF. This is further defined in clause 5.8.2.17 of TS 23.501 [2].

The UPF event exposure events are described in clause 5.2.26.2. In this Release of the specification, the following events are used for UPF Data collection:

-	QoS Monitoring. This event provides QoS Flow performance information.

-	User DataUsage Measures. This event provides information of user data usage of the User PDU Session.

-	User DataUsage Trends. This event provides statistics related to user data usage of the User PDU Session.

A consumer of UPF event exposure can subscribe to QoS monitoring event via SMF only and UPF sends the QoS Flow Performance information directly to this consumer. For this event, the interaction between SMF and UPF is over PFCP (TS 29.244 [69]). TS 23.501 [2] describes the QoS monitoring parameters that can be measured by means of QoS monitoring and how to enable the measurements for QoS flows. When the Subscription request for QoS monitoring event indicates that it is for the QoS Flow associated with the default QoS rule, based on local configuration the subscription request may trigger SMF to enable QoS monitoring.

NOTE 1:	Packets from multiple applications may share the QoS Flow associated with the default QoS Rule which may diminish the relevance of some measurements like data rate.

The subscription to QoS monitoring event can target the QoS flows bound to an application by including an Application Identifier. In this case, at subscription request and/or when the PCC rules change, SMF identifies the active PCC Rule that includes a DataCollection_ApplicationIdentifier matching that Application Identifier. SMF enables this consumer (e.g. NWDAF) to receive the QoS Monitoring reports enabled by that PCC Rule. The consumer may indicate that it can receive QoS Flow Performance information for the QoS Flow associated with the default QoS rule if there are no measurements available for the Application Identifier (that is, if no PCC rule is identified). In this case the SMF may instruct the UPF to perform QoS monitoring for the QoS Flow associated with the default QoS rule and include the Indication of QoS Flow associated with the default QoS Rule (see clause 5.8.2.18 of TS 23.501 [2]). The UPF will then include the Indication of QoS Flow associated with the default QoS Rule in the Nupf_EventExposure_Notify service operation when sending reports. Otherwise, the SMF may accept the request and indicate in the response that reporting will be activated when the measurements are enabled by a PCC rule or the SMF may reject the subscription request for that Application Identifier.

NOTE 2:	Extensive usage of QoS Monitoring has significant impact on load and signalling.

A consumer of UPF event exposure such as NWDAF/AF/NEF may subscribe to User Data Usage events (i.e. User Data Usage Measures and User Data Usage Trends) directly to UPF (under the conditions defined in clause 5.8.2.17 of TS 23.501 [2]) or via SMF, and UPF sends the event notifications directly to this consumer. For these events, the interaction between SMF and UPF is over SBI. For User Data Usage events, the subscription request targets the traffic matching Event Filter Information (details are described in clause 5.2.26.2.1) for a user's PDU Session or for each PDU Session served by the UPF. The subscription request may indicate the granularity requested, that is whether the measurement reports should be provided per data flow, per application, or per PDU Session.

If the event notification can be delayed, i.e. delay tolerant, Reporting suggestion information is included. The Reporting suggestion information includes Report urgency and Reporting time information. Reporting urgency information indicates whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates "delay tolerant", the Reporting time is also provided, which defines the last valid reporting time and UPF shall report the detected event before the last valid time.

If a consumer subscribes to an UPF event via the SMF including an AoI ("Area of Interest"), the SMF starts the subscription to the UPF only when the UE is located in the requested AoI. When the UE leaves the AoI, the SMF stops the subscription on the UPF. The SMF may subscribe to AMF about UE moving in or out of an AoI for this purpose. Clause 4.15.4.5.4 describes an alternative procedure for subscription via SMF to UPF event exposure service related with AOI.

Table 4.15.4.5.1-1: Input parameters in subscription to UPF event Exposure events

| Information | To SMF | To UPF (NOTE 4) |
| --- | --- | --- |
| UE identification (NOTE 5) | Y | Y |
| GroupID | Y | N |
| Any UE | Y | Y (NOTE 6) |
| DNN | Y | Y (NOTE 6) |
| S-NSSAI | Y | Y |
| AOI | Y | N |
| BSSID/SSID | Y | N |
| DNAI (NOTE 3) | Y | N |
| UPF Id (NOTE 3) | Y | not applicable |
| Type of Measurement | Y | Y |
| Granularity of Measurement (NOTE 2) | Y | Y |
| Application ID (NOTE 1) | Y | Y |
| Traffic Filtering Information (NOTE 1), (NOTE 2) | Y | Y |
| Reporting suggestion information | Y | Y |
| NOTE 1: Application ID and Traffic Filtering Information are exclusive. NOTE 2: This parameter does not apply to QoS monitoring event. NOTE 3: This parameter is used to indicate a UP Path as defined in Table 6.4.1-1 of TS 23.288 [50]. NOTE 4: This column is not applicable to events where SMF-UPF interactions are using PFCP (e.g. QoS Monitoring event). That interaction is described in clause 5.8.5 of TS 23.501 [2]. NOTE 5: Input parameter when the target is a UE. SUPI is the input to SMF and UE IP address is the input to UPF. NOTE 6: In a deployment where UPF reporting filtered per DNN and /or S-NSSAI is expected, the SMF should provide UPF with the necessary information over N4. If the SMF is not configured to provide UPF with this information then such UPF reporting filtering is not possible. |  |  |

* * * New text * * *

##### 4.15.4.5.X	Information flow for direct subscription to UPF event exposure service for certain UE using UE’s IP address

Figure 4.15.4.5.2-1: Subscription to UPF event exposure service for certain UE(s) directly

In the case, UPF event consumer (e.g. AF (when in trusted domain) or NEF) is aware of the UE IP address and the UPF Event ID(s) that are to be subscribed meets the criteria for direct subscription to UPF as defined in clause 5.8.2.17 of TS 23.501 [2]), then UPF Event consumer can subscribe to UPF directly for Event Exposure using the following procedure:

- The UPF event consumer triggers the UPF discovery to NRF by Nnrf_NFDiscovery_Request providing the UE IP address and optionally the DNN, S-NSSAI, IP domain. UE IP address provided to NRF maybe the IP address as seen by the DN (e.g. a Public IP address)

- The NRF responds with Nnrf_NFDiscovery response message including the information of UPF for the provided UE IP address.

3.	The UPF event consumer sends the Nupf_EventExposure Subscription request for the UPF Event(s) including the following information:

-	Notification Target Address (UPF event consumer address), Notification Correlation Information.

-	UPF Event Id.

-	Event Filter Information, i.e. one or more of the following parameters: S-NSSAI, DNN, either Application Id(s) or Traffic Filtering Information

-	Target of Event Reporting: UE IP address.

-	Reporting suggestion information.

-	Type of Measurement and Granularity of Measurement.

NOTE X:	Case when NAPT is deployed in UPF is not specified.

4.	As per the information provided in the previous step, UPF reports the collected data to the UPF event consumer by invoking Nupf_EventExposure Notify.

* * * Change 2 * * *

### 5.2.26	UPF Services

#### 5.2.26.1	General

The following table shows the UPF Services and UPF Service Operations.

Table 5.2.26.1-1: NF services provided by the UPF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nupf_EventExposure | Notify | Subscribe/Notify | NEF, AF, NWDAF, TSNAF/DCCF/MFAF/TSCTSF |
|  | Subscribe | Subscribe/Notify | NWDAF/DCCF, SMF, AF, NEF |
|  | Unsubscribe | Subscribe/Notify | NWDAF/DCCF, SMF, AF, NEF |
| Nupf_GetUEPrivateIPaddrAndIdentifiers | Get | Request/Response | NEF |

#### 5.2.26.2	Nupf_EventExposure Service

##### 5.2.26.2.1	General

Service description: This service can expose UPF related information to other NFs. There are several operations for this service:

-	Notifying events on the PDU Session to the NFs.

-	Allow consumer NFs to subscribe and unsubscribe for an Event ID on UPF.

The following events can be notified to a NF consumer:

-	QoS Monitoring. This event provides QoS Flow level performance information and can be used for direct event notification of QoS Monitoring reports to a different NF than the SMF as defined in clause 5.8.2.18 of TS 23.501 [2], or it may be for UPF Data collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.5.

Subscription to this event is always indirect via SMF. The subscription specifies the type of measurement that is being requested. A combination of the information listed below can be requested.

UPF and SMF interact using Session Reporting Rules as defined in clause 5.8.5.11 of TS 23.501 [2].

The event notification may contain following information:

-	QoS monitoring result for the QoS monitoring parameter(s) defined in clause 5.45 of TS 23.501 [2], e.g. UL packet delay, DL packet delay, or round trip packet delay.

-	Indication of QoS Flow associated with the default QoS Rule (if requested by SMF, see clause 4.15.4.5.1).

-	User Data Usage Measures or User Data Usage Trends. These events provide information about traffic matching Event Filter Information (see below) for a user's PDU Session or for each PDU Session served by the UPF. It can be used for UPF Data Collection by NWDAF for analytics (see TS 23.288 [50]) as described in clause 4.15.4.5.

SMF shall use Service Based Interface subscription service operation to subscribe this UPF event. Other direct consumers can subscribe directly to UPF under the conditions defined in clause 5.8.2.17 of TS 23.501 [2]. The subscription request contains the Type of Measurement that is being requested, the Target of Event Reporting and the Event Filter Information (which describes the target traffic) and the required Granularity of Measurement. The Target of Event Reporting can be either a specific PDU Session or "any UE". The Event Filter Information may contain one or more of the following parameters: DNN, S-NSSAI, either Application ID(s) or Traffic Filtering Information. The Granularity of Measurement can indicate per data flow, per application, or per PDU Session. A combination of the information listed below for User Data Usage Measures can be requested.

For User Data Usage Measures, the event notification may contain following event related information:

-	Volume Measurement: measurements of data volume exchanged (UL, DL and/or overall) and/or number of packets exchanged (UL, DL and/or overall) determined for the requested Granularity of Measurement.

-	Throughput Measurement: measurements of data throughput (UL and DL) determined for the requested Granularity of Measurement.

-	Application related Information: URL(s) and/or Domain information (domain name and protocol) detected for the target traffic. This Type of Measurement requires that Application Id(s) or Traffic Filtering Information is provided (i.e. this measurement is not possible to be applied for all traffic handled by the UPF).

For User Data Usage Trends, the event notification may contain following event related information:

-	Throughput Statistic Measurement (average and/or peak throughput) over the measurement period determined for the requested Granularity of Measurement.

For User Data Usage Measures and User Data Usage Trends, if the subscription is targeting any UE, the UPF shall perform the requested measurements for every PDU Session that matches the Event Filter Information and send the information per PDU Session, including the DNN and S-NSSAI and optionally, the sampling ratio achieved by the UPF. When the subscription requests that information is provided per data flow, the notification includes the Packet Filter Set and the Applications Identifier, if available.

NOTE 1:	If the Target of Event Reporting is "any UE", care needs to be taken with regards to load and major signalling impacts and it is expected that the subscription request contains Event Filter Information and appropriate Event Reporting Information (like Sampling ratio and Partitioning criteria, see Table 4.15.1-1) in order to minimize the UPF performance impact of the per PDU Session measurement.

NOTE 2:	The UPF can combine information for different PDU Sessions into one notification message (see TS 29.564 [93]).

-	TSC management information (UMIC, PMIC, NW-TT port number) as defined in clause 5.8.5.14 of TS 23.501 [2].

TSC management information event can be the result of an implicit subscription of the PCF on behalf of the TSNAF/TSCTSF via the Npcf_SMPolicyControl service as described in clause 5.28 of TS 23.501 [2].

If the consumer of UPF service is NWDAF and the target of UE is any UE, according to the Analytic ID from consumer, the NWDAF can decide which kind of information should be collect from UPF and event ID to use.

If the consumer of UPF service requires data collection (e.g., usage report for a specific UE, usage report for a specific application of UE), the consumer NF can subscribe UPF service using UE IP address directly.

Subscription may be performed directly towards UPF or via SMF as described in clause 5.8.2.17 of TS 23.501 [2]. The UPF collects the data according to the event ID and exposes the related information directly regardless of whether the subscription has been relayed by the SMF.

* * * End of Changes * * *