SA WG2 Meeting #164	S2-2408771

Maastricht, NL, 19-23 August 2024

Source:	Nokia, Qualcomm Inc., Ericsson

Title:	TR 23.700-66 revised conclusions

Document for:	approval

Agenda Item:	19.4.1

Work Item / Release:	FS_EnergySys/ Rel-19

Abstract: proposes an update of the TR conclusions

# 1. Introduction/Discussion

Following further discussions in recent weeks it is apparent that it is possible to reach more precision in the conclusions of the TR 23.700-66. More precisely: it is now clearer the granularity at which the exposure of the energy information is required, and that SA5 involvement is clearer.

# 2. Proposal

It is proposed that the following text modifications to 23.700-66 are approved.

START OF CHANGES

# 8	Conclusions

## 8.1	Conclusion for KI#1: Network energy related information exposure

The following bullets are recommended for normative work for KI#1:

SA2, in close cooperation with SA5, will focus on supporting the collection information needed for the calculation of energy related information at UE, PDU session and QoS flow granularities.

The expected energy related information estimates to be calculated and exposed are estimates of:

-	Energy consumption

-	Renewable Energy Consumption

-	Carbon emissions.

The authorized consumers including 5GC NFs and /or AF, may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

SA2 shall focus on energy consumption information exposure to authorized AFs via NEF, by using information exposed from CHF. The CHF can also provide the information to authorized 5GC NFs.

It is expected the system needs to be evolved to support of the reporting to CHF of the ULI when the gNB changes and that the Identifiers of any I-UPF(s) upon the insertion of any I-UPF are also included in Charging Data Request operations to the CHF. It is expected I-SMF reports within charging data to SMF also and the identifiers of the I-UPF(s) and identifiers of any local PSA UPF that are used. Flow based charging for reporting at QoS flow granularity is tiggered by the CHF as via the PCF. The Data Volume reporting to CHF remains with the PSA UPFs only.

NOTE: 	It is expected that the Charging subsystem and the OAM subsystem interact to derive the energy consumption over specific time period T that need to be configured to be the same PLMN-wide in all UPFs and gNBs. The time interval T is a PLMN configuration parameter independent from the frequency of reporting to the the AFs and NFs (the configured time T shall be associated to the highest possible frequency of reporting). The CHF accesses energy consumption information per UPF and per gNB from OAM by using the existing OAM KPIs exposure capabilities.

The required granularities for energy consumption exposure include:

-	per application of the UE,

-	per UE, and

-	per DNN/S-NSSAI for the UE

The information for the calculation (e.g. by using formulas such as those in  S2-2408122) of the Energy Consumption information in the PLMN-configured configured period of time T is obtained from the following sources:

a)	OAM: provides the energy consumption information at the gNB(s) and UPF(s) serving the UE in time interval T

b)	OAM: provides the overall data volume for the gNB and in a UPF over the period of time T;

c)	UPF: provides via the SMF the data volume for a UE, PDU session and the QoS flow  to CHF. The timing of reporting the data volume per UE needs to be aligned to the time interval T.

Only the energy-related information of user plane communication (not control plane signalling) is supported.

NOTE 1:	subject to TSG RAN decision, it may be possible to consider information reported from the RAN if it will be available.

Editor's note:	Whether in this release the NG-RAN will provide per UE level energy consumption information and state depending on RAN WGs. SA WG2 will only start the work based on RAN decision to work on energy efficiency topic.

## 8.2	Conclusion for Key Issue #2: Subscription and policy control to support energy efficiency and energy saving as service criteria

The following principles for enhancements on subscription and policy control are agreed for the normative work:

-	The following information is stored as part of the subscription data in the UDM/UDR:

-	Energy saving subscription information per UE to assist the network to perform energy saving strategies for the UE.

-	The PCF may receive UE subscription data and notification related to the energy related information to trigger making policy decisions (reusing the existing parameters) based on operator policy.

NOTE 1:	Further details and what additional energy related information can be taken into account for policy control will be defined during the normative work.

NOTE 2:	Further details, more parameters and associated procedures will be defined during the normative work.

NOTE 3:	The interaction for PCF with CHF for policy control and charging (e.g. how the PCC rules are determined based on energy criteria) will be considered in the normative phase, based on collaboration with SA WG5.

## 8.3	Conclusion for Key Issue #3: 5GS enhancements for network energy saving and efficiency

It is proposed to proceed to normative work based on the following principles:

1)	For Enhancements on NF discovery and (re-)selection based on energy related information:

-	NF profile may be extended (e.g. include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy.

NOTE 1:	Which energy related information is included in the NF profile is to be further discussed in the normative phase. Only parameters captured in existing solutions will be considered. Coordinated with SA WG5 may be needed.

NOTE 2:	The information within NF profile is expected not to change frequently, e.g. constantly reflecting changes in energy consumption. The procedure for updating the information will be addressed in normative phase.

-	NF discovery and (re-)selection is enhanced to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

2)	For enhancements on existing operations and procedures for energy saving and energy efficiency:

-	UP path of PDU session may be adjusted.

NOTE 3:	The energy related decision will also consider operator’s policy.

END OF CHANGES