3GPP TSG-WG SA2 Meeting #164	S2-2409050

# Maastricht, Netherlands, 19 August – 23 August, 2024		                                was S2-2407654

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4858 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mapping between NSAGs and S-NSSAIs – 23.502 R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Nnssf_NSSAIAvailability_Update service operation (23.502 5.2.16.3.2) is currently misaligned with 23.501 5.15.14. DP S2-2407652 explains the problem(s) in a great detailed. In short, RAN provides to the AMF (as defined in 38.413 9.3.1.238/9.3.1.191) an association of one NSAG to many sNSSAIs. AMF can provide to the NSSF a collection of such associations (obtained from multiple RANs), However, the basic format of one NSAG to many sNSSAIs still has to be kept. Otherwise, we might be in breach of the rule of having one NSAG at the most for an sNSSAI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to align NSAG aspect of Nnssf_NSSAIAvailability_Update service operation with NSAG aspect of 38.413, while allowing the AMF to provide a collection of NSAG-sNSSAIs associations received from multiple RANs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Misalignment between 23.502 5.2.16.3.2 and 38.413 9.3.1.238/9.3.1.191, which may lead to a breach of the rule of max of one NSAG for sNSSAI defined in 23.501 5.15.14. 2. UE’s operation of selecting RAN node based on NSAG – sNSSAIs association Logic may fail due to ambigous information coming from the AMF/NSSF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.16.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

##### 5.2.16.3.2	Nnssf_NSSAIAvailability_Update service operation

Service operation name: Nnssf_NSSAIAvailability_Update

Description: This service operation enables the AMF to update the NSSF with the S-NSSAIs the AMF supports per TA and get the availability of the S-NSSAIs and optionally NSI IDs per TA for the S-NSSAIs the AMF supports.

Inputs, Required: Supported S-NSSAIs per TAI.

The supported S-NSSAIs per TAI, is a list of TAIs and for each TAI the NSAGs and S-NSSAIs supported by the AMF.

Inputs, Optional: Supported NSAGs per TAI.

The supported NSAGs per TAI, is a list of TAIs and for each TAI the NSAGs and for each NSAG the associated S-NSSAIs supported by the AMF.

Outputs, Required: A list of TAIs and for each TAI, the S-NSSAIs and optionally NSI IDs supported by the AMF and 5G-AN and authorized by the NSSF for the TAI.

Outputs, Conditional Required: A list of TAIs and for each TAI, the NSAGs authorized by the NSSF for the TAI.

If the NSSF has NSAG information, it should provide the authorized NSAGs per TAI. This is a list of TAIs and for each TAI the NSAGs and for each of the NSAGs the associated S-NSSAIs.

NOTE:	This NSAG information can include additional NSAG – S-NSSAIs associations (e.g. NSAG information obtained from other AMFs).

Outputs, Optional: For each TAI, a list of S-NSSAIs restricted per PLMN for the TAI.

# *** End of Change ***