3GPP TSG-WG SA2 Meeting #164	S2-2409088

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024	revision of S2-2408406

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1171 | rev | 1 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Registration and Discovery procedure for Vertical Federated Learning among NWDAF(s) and/or AF(s) with NWDAF as the VFL server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ICS, OPPO, InterDigital, Lenovo, Vivo, NTT DOCOMO, Huawei, ZTE, LG Electronics, KDDI, Samsung? Ericsson? Nokia? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introducing a new FL variant (i.e. Vertical Federated Learning) in 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [1st Change] Adding a new subsection to describe required procedures to perform VFL Registration and Discovery when NWDAF(s) and/or AF(s) are the VFL client with a VFL Server NWDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new feature that was concluded in the study phase will not be adopted in the related specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.2X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501, TS 23.502 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 6.2X.X.X	Registration and Discovery procedure for Vertical Federated Learning among NWDAF(s) and/or AF(s) with NWDAF as the VFL server

Figure 6.2X.X.X-X: Registration and Discovery procedure for Vertical Federated Learning when NWDAF is the VFL server and NWDAF(s) and/or AF(s) are the VFL client.

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is NWDAF.

1-3.	VFL Server/Client NWDAF (containing MTLF)  and VFL Client AF (containing model training capability) register to NRF with its NF profile, which includes NF Type (see clause 5.2.7.2.2 of TS 23.502 [X]), Analytics ID(s), Address information of NWDAF/AF, Service Area, FL capability information which includes  FL capability type (i.e. FL server and/or FL client), FL type (e.g. VFL), optional Vendor ID(s), and Time interval supporting FL as described in clause 5.2.

Editor’s note: Whether to use Vendor ID(s) or introduce new identifiers for interoperability among NWDAF and AFs is FFS.

For an untrusted AF, the NEF registers based on the configuration at the NRF within its NF profile information about the AF as specified in clause 6.2.2.3 and includes other information related to the AF capability, including FL capability information, which includes FL capability type (i.e. FL server and/or FL client), FL type (i.e. VFL)).

Steps 4 to 6 are the NWDAF and AF Discovery procedures when the VFL server is NWDAF.

4-6.	NWDAF containing MTLF (as the VFL server) determines that the ML Model requires VFL based on operator policy (e.g., a pre-configured list of ML Models), Analytic ID, Service Area/DNAI.

NOTE: Step 4 in Figure 6.2X.X.X-X may data be triggered at the VFL Server NWDAF by VFL server NWDAF itself or a request from an analytics consumer.

If the NWDAF containing MTLF can not perform as VFL Server NWDAF, the MTLF first discovers and selects VFL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the VFL ML Model required, ML Model filter information as defined in clause 6.2A.2, Time Period of Interest, Service Area, FL capability information (FL capability Type (i.e. FL server and/or FL client), and, FL type (i.e. VFL)).

Once the VFL Server NWDAF is determined, the VFL Server NWDAF discovers other NWDAF(s) containing MTLF and/or AF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML Model required, FL capability information (FL capability Type (i.e. FL client), FL type (i.e. VFL)), and Service Area.

Editor's note:	After performing the VFL discovery, whether to perfrom the sample and feature alignemnt to let VFL server determine the final VFL client is FFS.

# * * * * End of changes * * *