3GPP TSG-SA WG2#164	S2-2409193

19-23 August 2024, Maastricht, Netherlands	(was S2-2408127, S2-2406823)

Source:	Apple, ETRI

Title:	Conclusion for KI#1.1

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes conclusion for KI#1.1.

# 1.	Introduction

This contribution proposes interim conclusion for KI#1.1 based on NWM discussions report in S2-2408126 and S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v1.0.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

### 8.1.x 	Interim Conclusions for KI#1.1

The following principles are concluded for the normative work:

-	The two SUPIs for the DualSteer Device should be correlated in the UDM via subscription information by enhancing t

he AM subscription data with a link to the associated SUPI of the DualSteer Device.

-	The SM subscription data is enhanced by including the DNNs/S-NSSAIs enabled for DualSteer.

-	Optionally, a Primary SUPI can be assigned statically in the subscription per operator’s policy. The HPLMN can inform the role of the SUPI to the DualSteer Device during the registration procedure if DualSteer can be enabled for this leg.

- The way how each of the SUPI is used will be determined by network policies defined by the operator.

# >>>>END OF CHANGES<<<<