SA WG2 Meeting #164	S2-2408771

Maastricht, NL, 19-23 August 2024

Source:	Nokia, Qualcomm Inc., Ericsson

Title:	TR 23.700-66 revised conclusions

Document for:	approval

Agenda Item:	19.4.1

Work Item / Release:	FS_EnergySys/ Rel-19

Abstract: proposes an update of the TR conclusions

# 1. Introduction/Discussion

Following further discussions in recent weeks it is apparent that it is possible to reach more precision in the conclusions of the TR 23.700-66. More precisely: it is now clearer the granularity at which the exposure of the energy information is required, and that SA5 involvement is clearer.

# 2. Proposal

It is proposed that the following text modifications to 23.700-66 are approved.

START OF CHANGES

# 8	Conclusions

## 8.1	Conclusion for KI#1: Network energy related information exposure

The following bullets are recommended for normative work for KI#1:

SA2, in close cooperation with SA5, will focus on supporting the collection information needed for the calculation of energy related information at UE, PDU session and QoS flow granularities.

The expected energy related information estimates to be calculated and exposed are estimates of:

-	Energy consumption

-	Renewable Energy Consumption

The authorized consumers including 5GC NFs and /or AF, may request the energy consumption information exposure with reporting request e.g. Periodic reporting or Threshold based reporting.

SA2 shall focus on energy consumption information exposure to authorized AFs via NEF, by using information exposed from CHF. The CHF can also provide the information to authorized 5GC NFs.

It is expected the system needs to be evolved to support of the reporting to CHF of the ULI when the gNB changes and that the Identifiers of any I-UPF(s) upon the insertion of any I-UPF are also included in Charging Data Request operations to the CHF. It is expected I-SMF reports within charging data to SMF also and the identifiers of the I-UPF(s) and identifiers of any local PSA UPF that are used. Flow based charging for reporting at QoS flow granularity is tiggered by the CHF as via the PCF. The Data Volume reporting to CHF remains with the PSA UPFs only via the SMF (i.e. no I-UPF reporting data volume directly via (I-SMF) to the CHF is expected to be required).

NOTE: 	It is expected that the Charging subsystem and the OAM subsystem interact to derive the energy consumption over specific time period T that need to be configured to be the same PLMN-wide in all UPFs and gNBs. The time interval T is a PLMN configuration parameter independent from the frequency of reporting to the the AFs and NFs (the configured time T is associated to the highest possible frequency of reporting). The CHF accesses energy consumption information per UPF and per gNB from OAM by using the OAM KPIs exposure capabilities.

The required granularities for energy consumption exposure include:

-	per application of the UE,

-	per UE, and

-	per DNN/S-NSSAI for the UE

The information for the calculation (e.g. by using formulas such as those in  S2-2408122) of the Energy Consumption information in the PLMN-configured configured period of time T is obtained from the following sources:

a)	OAM: provides the energy consumption information at the gNB(s) and UPF(s) serving the UE in time interval T

b)	OAM: provides the overall data volume for the gNB and in a UPF over the period of time T;

c)	UPF: provides via the SMF the data volume for a UE, PDU session and the QoS flow  to CHF. The timing of reporting the data volume per UE needs to be aligned to the time interval T.

Only the energy-related information of user plane communication (not control plane signalling) is supported.

NOTE 1:	subject to TSG RAN decision, it may be possible to consider information reported from the RAN if it will be available.

Editor's note:	Whether in this release the NG-RAN will provide per UE level energy consumption information and state depending on RAN WGs. SA WG2 will only start the work based on RAN decision to work on energy efficiency topic.

NOTE2: 	It is assumed that the measurement time period T needs to be aligned over the UPF/NG-RAN nodes for which information is collected, i.e. the start/stop of the time periods is time-synchronized.

END OF CHANGES