3GPP TSG-SA2 Meeting #164	S2-2409305

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024                  (revision of S2-2409060)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0442 | rev | 4 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for end-to-end PC5 link management for L2 U2U Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_relay_enh-Core, 5G_ProSe_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. In TS 23.304, there is a single signalling diagram with details on the setup of new end-to-end PC5 link via L2 UE-to-UE relay in clause 6.7.2 In this procedure, the addition of a new end-to-end PC5 link may be treated as a modification of an exsitng Layer-2 PC5 direct link between End UE and U2U relay UE. This modification has been implemented in ProSe Direct Link Modification procedure (as specified in TS 24.554). With the same design logic, when the end-to-end PC5 link between two End-UEs is released, the link shall also be modified with a similar PC5-S link modification procedure to synchronize the PC5 link context among the End UE and Relay UE. It is not suitable to let lower layer (PC5-RRC) to exchange this e2e link release informaiton because the lower layer does not know the full context of the released PC5 link as “User info ID” are never exposed to lower layers, the only identifier known by the lower layer is just Layer-2 address. As a result, the upper layer of ProSe Relay UE may continue regard itself serving an outdated “User Info ID”of a ProSe End UE and may also include it in its Model-A discovery message falsely. It is proposed to capture the above procedure in the TS 23.304 with a concise description in 6.4.3.7.2 2. TS 23.304, clause 6.4.3.7.2 provides a blanket statement that end-to-end PC5-S messages are exactly as the same as what has been specified as clause 6.4.3.1 to 6.4.3.5. However, for UE-to-UE relay case, there are some problems to reuse the direct PC5 link procedures in 6.4.3.1~6.4.3.5. One problem is that the Layer-2 ID which are supposed to encapsulated in the lower layer headers are no longer available end-to-end. To make the above statement work, RAN2 has made the agreement that lower layer of End UE provides the per-hop Layer-2 ID of peer UE to ProSe Layer after receiving any PC5-S signalling (RAN2 LS S2-2403891/R2-2401918). TS 23.304 lacks text on how the ProSe layer supports in destination Layer-2 ID mapping. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Added the desciption text to explain that for L2 U2U relay, End UE releasing the e2e link can trigger the Layer-2 link release/modificaiton procedure towards the relay UE. 2. Description added in 6.4.3.7.2 on how the Layer-2 ID for the end-to-end PC5 connection is set . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1.The L2 U2U relay UE will not be able to cleanly remove the end-to-end link context between a pair of User Info IDs of End UEs. 2. Ambiguity about the Layer-2 ID relations between end-to-end link and per-hop link remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### ****************************** First Change ****************************************

##### 6.4.3.7.2	Layer-2 link management over PC5 reference point for 5G ProSe Layer-2 UE-to-UE Relay

For the 5G ProSe Communication via 5G ProSe Layer-2 UE-to-UE Relay as described in clause 6.7.2, the description in clause 6.4.3.7.1 applies. In case of ProSe communication via 5G ProSe Layer-2 UE-to-UE Relay, an end-to-end Direct Communication Request message between the source 5G ProSe End UE and the target 5G ProSe End UE is transmitted after the per-hop connections have been established. The information from end-to-end unicast link profile in clause 6.4.3.7.1 is used for mapping of the destination Layer-2 ID as specified in TS 38.300 [12] and TS 38.331 [16].

The message contents over PC5 reference point for unicast mode 5G ProSe Direct Communication as depicted from clause 6.4.3.1 to clause 6.4.3.5 are same for the end-to-end connection between peer 5G ProSe End UEs.

When the 5G ProSe End UEs initiate End-to-End PC5-S signalling to release an end-to-end ProSe link via 5G ProSe Layer 2 UE-to-UE relay, the 5G ProSe End UE which has initiated the PC5-S release procedure triggers Layer-2 link modification or a Layer-2 link release procedure as described in clause 6.4.3.7 towards 5G ProSe Layer-2 UE-to-UE relay.

##### ****************************** End of Change ****************************************