3GPP TSG-SA WG2#164	S2-2409382

19-23 August 2024, Maastricht, Netherlands	(was S2-2409194, S2-2408131, S2-2406824)

Source:	Apple, ETRI

Title:	Conclusion for KI#1.2

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes conclusion for KI#1.2.

# 1.	Introduction

This contribution proposes conclusion for KI#1.2 based on NWM discussion report in S2-2408126 and S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v1.0.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

### 8.1.x 	Interim Conclusions for KI#1.2

The following principles are concluded for the normative work:

-	The UE(s) of the DualSteer Device include(s) information in the Registration request whether it is (they are) DualSteer capable.

-	In roaming scenario, DualSteer can be supported if the serving PLMN of each registered access supports the DualSteer feature.

-	A DualSteer Device performs a separate registration procedure for each of the SUPIs of the DualSteer Device.

- 	If the DualSteer Device sends its DualSteer capability as part of Registration Request, the serving network sends that information to the HPLMN. The HPLMN enables the DualSteer feature based on the subscription data associated with the SUPI, the capability of DualSteer Device and the serving network. The AMF provides the DualSteer capability to UE PCF. The AMF indicates to the DualSteer Device whether the network authorizes the DualSteer feature during registration. If DualSteer is not enabled on one of the registered 3GPP access, the DualSteer feature cannot be enabled.

Editor’s Note: Whether there are any impacts on the second SUPI’s PLMN selection are pending on SA1 and CT1.

Editor’s Note: The above conclusions are work-in-progress and they are not meant to represent a complete set of conclusions of this Key Issue.

# >>>>END OF CHANGES<<<<