3GPP SA WG2 Meeting #164	 S2-2408904

# Maastricht, NL, 19-23 August 2024											             was S2-2408683

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.032 | CR | 0029 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Relative Velocity Clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ranging_SL |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per RAN2 LS S2-2407428/ R2-2405987, “RAN2 adopted the following parametrization of the two components: Parameters of the radial velocity component: - radial velocity [measured in meters / second] - uncertainty and confidence for radial velocity Parameters of the transverse velocity component: - azimuth rate of change in time [measured in degrees / second] - elevation rate of change in time [measured in degrees / second] - uncertainty and confidence for azimuth rate of change - uncertainty and confidence for elevation rate of change” Clause 8.4a description should merge with above contents. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updates to clause 8.4a according to the measurements’ list in RAN LS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing descriptions of units used for Ralative Velocity . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>First Change<<<<

## 8.4a	Relative Velocity with Uncertainty

The relative velocity with uncertainty of a device B relative to a device A is characterised by a radial velocity component (measured in meters / second) and a perpendicular transverse velocity component (measured in degrees / second). The radial velocity component is characterized by a rate of change of a range between the device A and device B. The transverse velocity component is characterized by a rate of change of a direction to the device B from the device A, where the change rate of the direction includes the change rate of an angle of azimuth (measured in degrees / second) and change rate of an angle of elevation (measured in degrees / second). The angle of azimuth is measured clockwise from North in a horizontal plane through the device A and the angle of elevation measured upwards or downwards in a vertical plane through the devices A and B from a horizontal plane through the device A. The rates of change of the range and the angles of azimuth and elevation can be each independently included or excluded in the relative velocity and each has an uncertainty and a confidence,

Figure 12a: Description of a Relative Velocity with Uncertainty

# >>>>END OF CHANGES<<<<