3GPP TSG-SA WG2 Meeting #164	S2-2409429

19 August - 23 August 2024, Maastricht, Netherlands	(revision of S2-2408107,8817,9125 )

Source:	CATT

Title:	Update the Conclusion for KI#9

Document for:	Discussion/Approval

Agenda Item:	19.3.1

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: this contribution proposes to update the conclusion for KI#9.

1.	Introduction

This contribution proposes to update the conclusion for KI#9.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First Change * * * *

## 8.9	Conclusions for Key Issue #9

The following aspects are concluded for the normative work:

-	For GBR QoS Flow, exposure of available data rate:

-	Based on the AF requests, the PCF instructs SMF to enable the available data rate report and exposure for a GBR QoS Flow.

-	Following the instruction of SMF:

-	The NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes based on mechanisms defined in clause 5.8.2.17 of TS 23.501 [2], and the available data rate is reported to the AF directly or via the (local) NEF.

-	 Available data rate refers to the data rate that RAN is able to provide within a duration. Such duration is   based on RAN implementation.

NOTE 1:	It is recommended that the available data rate exposure is not reported very frequently.

NOTE 2：How the RAN measures the available UL and/or DL data rate provided by the RAN node is to be refined based on the potential feedback from RAN WG3.

# * * * * End of change* * * *