SA WG2 Meeting #S2-165	S2-2409605

14 - 18 October, 2024, Hyderabad, India

3GPP TSG-SA3 Meeting #117	S3‑243533

Maastricht, Netherlands  19 - 23 August 2024				    revision of S3-242764

Title:	LS on reply to LS on FS_5GSAT_Ph3_ARCH conclusions

Response to:	S2-2407350

Release:	Rel-19

Work Item:	FS_5GSAT_Ph3_ARCH

Source:	SA3

To:	SA2

Cc:	SA3-LI, RAN2

Contact person:	Rakshesh P. Bhatt

Rakshesh.p_bhatt@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 thanks SA2 for the Liaison S2-2407350 and provides observations and feedback on the conclusions of FS_5GSAT_Ph3_ARCH for support of Store and Forward and UE-Satellite-UE communication captured in the TR 23.700-29 Clause 8.2 and 8.3 respectively. SA3 also seeks some clarifications as detailed below.

- SA3 understands that in Rel-19, only S&F operations for delay-tolerant/non-real-time IoT satellite services including SMS services, which don’t need an end-to-end connectivity between the end-points will be supported. Also, S&F mode is only supported for EPS in Rel-19.

- From clause 8.2 of TR 23.700-29, for full CN onboard each satellite, it is understood that proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. These proxies and the interface between them are out of 3GPP scope.

  - Observation 1: SA3 considers the security of communications between the proxies on satellite and ground is out of 3GPP scope.

  - Observation 2: With HSS also on board each satellite, SA3 assumes that the subscription data will be uploaded and synchronized to HSS in each satellite via these proxies. Therefore, the security and privacy of this subscription data being uploaded and synchronized is also out of the scope of 3GPP..

  - Observation 3: If full CN is onboard each satellite, SA3 understands that for services like SMS, MO or MT data transfer involves fresh security context establishment with UEs every time UE gets coverage from a new satellite. This does not require any additional security considerations and legacy security procedures should suffice.

  - Observation 4: For a full EPC including HSS in a satellite, UE security credentials that are exposed in a satellite must not include long term keys.

- From clause 8.2 of TR 23.700-29, SA3 understands that for Split MME architecture:

  - Observation 5: For the split MME solution with HSS on the ground, only one network entity at a time must have UE’s current NAS security context in order to avoid security problems (e.g. replay attacks) that can occur when a UE attempts to access a satellite with a prior NAS security state.

  - SA3 has not yet concluded on whether NAS security context should be terminated in MME-ground or MME-onboard. The answers for the following questions would be helpful for solution discussion in SA3.

    - Question 1a: Does SA2 consider any use cases that need NAS security context to be terminated in MME-onboard or MME-ground?

    - Question 1b: Does SA2 consider any use cases where multi-satellites can be involved in the S&F operation?

# 2	Actions

To SA2:

ACTION: SA3 kindly asks SA2 to take the above information into account in their further work and provide feedback.

# 3	Dates of next TSG SA WG 3 meetings

SA3#118	14 - 18 October 2024		Hyderabad (India)

SA3#119	11 - 15 November 2024		Orlando (US)