3GPP TSG-WG SA2 Meeting #165	S2-2409664

14 - 18 October, 2024, Hyderabad, India	(revision of S2-240xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5007 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription of UPF event via I-SMF for any UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For any UE case, there are two scenarios that handling for UPF event exposure: Case 1: If the consumer provides the DNAI to SMF (as indicated in step 2 of 4.15.4.5.3, the consumer converts the EAS IP address/FQDN to DNAI), the DNAI will be used for the SMF discovery. If the SMF can not serve this DNAI, and the procedure defined in single UE for introducing I-SMF will be reused. Case 2: If the consumer doesn’t provide the DNAI to SMF (for example, only contains DNN, S-NSSAI), the DNN or S-NSSAI will be used for the SMF discovery. There may exist multiple SMFs after discovery from NRF. And the consumer may send the subscription to all of the discovered SMFs. And all of the UEs that under control by this SMF will be subscribed the event. Case 2-1: For some of the UE, the SMF determines that this UE including the DNAI that SMF supports and serving, the SMF can subscribe the event to the UPF directly. Case 2-2: For some of the UE, the SMF determines that this UE doesn’t include the DNAI that SMF determines this UE doesn’t have the edge traffic routing, the SMF can subscribe the event to the UPF directly. Case 2-3: For some of the UE, the SMF determines that this UE including the DNAI that SMF can not serve, the SMF can only subscribe the event to the UPF indirectly via I-SMF. This is the same as the single UE case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For any UE case, if the DNAI is included in the subscription request to SMF and the SMF doesn’t support to serve this DNAI, the same procedure as single UE. If the DNAI is not included in the subscription request to SMF, several SMFs are discovered from NRF. And each SMF should check all of the UEs under controlled. If this UE includes the DNAI and SMF supports and serving, the SMF can subscribe the event to UPF directly. If SMF doesn’t support and serve the DNAI, it follows the I-SMF assisted subscription procedure. If no DNAI included for this UE, SMF determines this UE doesn’t have the edge traffic routing, the SMF can subscribe the event to the UPF directly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to support the UPF Information Exposure for any UE in the case of I-SMF is FFS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

4.15.4.5.3	Information flow for UPF event exposure service for any UE

Figure 4.15.4.5.3-1: UPF Information Exposure to the UPF event consumer (e.g. NWDAF) of any UE scenario

1.	(For the case when the UPF event consumer is NWDAF) The analytics consumer sends a request to the NWDAF for analytics on any UE. The analytics consumer provides the value "any UE" in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, Area of Interest, Application server IP address/FQDN, Application ID, DNAI, etc.

2.	(Optional and only when the UPF event consumer is NWDAF) If the Analytic Filter Information does not contain DNN/S-NSSAI, but only includes application server IP address/FQDN, the NWDAF should first obtain the DNAI from NEF. The NWDAF invokes Nnef_DNAIMapping_Subscribe service to request the DNAI information. The request includes EAS IP/IP range and/or FQDN.

3.	(Conditional, if step 2 took place) The NEF determines the suitable DNAI(s) and provides them to NWDAF.

4.	The UPF event consumer triggers the SMF(s)/UPF(s) discovery to NRF by Nnrf_NFDiscovery_Request providing the DNN, S-NSSAI, DNAI etc. This procedure is to discover the related SMF(s)/UPF(s) associated with any UE and support the indicated DNAI. Either SMF(s) or UPF(s) are discovered depending on whether the subscription request to UPF events meets the criteria for direct subscription to UPF as defined in clause 5.8.2.17 of TS 23.501 [2]).

5.	The NRF provides Nnrf_NFDiscovery_Response that may refer to several SMFs/UPFs.

6.	(Option 1) If the subscribed UPF events needs the SMF(s) to do a third-party subscription onto UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), the same procedure as Indirect subscription via several SMFs (steps 3 - 5 in Figure 4.15.4.5.2-1(for single UE)) takes place via each discovered SMF.

If the UPF event consumer subscribe the SMF(s) with providing DNAI, each SMF(s) should check whether the DNAI can be served or not. If not, the SMF sends the subscription request to UPF through I-SMF as described in step 4b of clause 4.15.4.5.2.

If the DNAI is not included in the subscription request to SMF(s), each SMF should check all of the UEs under controlled. If the SM context of this UE includes the DNAI that SMF can serve, or if the SM context of this UE doesn’t include the DNAI, the SMF follows the step 4a or 4b in clause 4.15.4.5.2 that sends the subscription request to UPF. If the SM context of this UE includes the DNAI that SMF can’t serve, the SMF sends the subscription request to UPF through I-SMF as described in step 4b of clause 4.15.4.5.2.

7.	(Option 2) If the subscribed UPF events allows to directly subscribe to UPF (as defined in clause 5.8.2.17 of TS 23.501 [2]), the UPF event consumer (e.g. NWDAF) triggers the Nupf_EventExposure_Subscribe to all discovered UPFs. The information included in the subscription is:

-	Notification Target Address (UPF event consumer address), Notification Correlation Information.

-	UPF Event Id.

-	Event Filter Information, i.e. one or more of the following parameters: S-NSSAI, DNN, either Application Id(s) or Traffic Filtering Information.

-	Target of Event Reporting: any UE.

-	Reporting suggestion information.

-	Type of Measurement and Granularity of Measurement.

NOTE:	As the Target of Event Reporting is "any UE", care needs to be taken with regards to load and major signalling impacts and it is expected that the subscription request contains Event Filter Information and appropriate Event Reporting Information (like Sampling ratio and Partitioning criteria, see Table 4.15.1-1) in order to minimize the UPF performance impact.

8.	Each of the UPFs invokes Nupf_EventExposure_Notify service operation directly to the UPF event consumer (e.g. NWDAF).

* * * End of Change * * *