3GPP TSG-WG SA2 Meeting #165	S2-2409665

14 - 18 October, 2024, Hyderabad, India

Title:	LS on for UPF event subscription via I-SMF for any UE

Release:	Rel-18

Work Item:	UPEAS

Source:	SA2

To:	CT4

Cc:

Contact Person:

Name:	Dan Wang

Tel. Number:

E-mail Address:	wangdanyjy@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

The SA2 group has the following consensus about the UPF event subscription via I-SMF for any UE case.

- If the UPF event consumer subscribe the SMF(s) with providing DNAI, each SMF(s) should check whether the DNAI can be served or not. If not, the SMF sends the subscription request to UPF through I-SMF as described in step 4b of TS 23.502 clause 4.15.4.5.2.

- If the DNAI is not included in the subscription request to SMF(s), each SMF should check all of the UEs under controlled. If the SM context of this UE includes the DNAI that SMF can serve, or if the SM context of this UE doesn’t include the DNAI, the SMF follows the step 4a or 4b in clause 4.15.4.5.2 that sends the subscription request to UPF. If the SM context of this UE includes the DNAI that SMF can’t serve, the SMF sends the subscription request to UPF through I-SMF as described in step 4b of TS 23.502 clause 4.15.4.5.2.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly asks CT4 group to take the comments above into account.

3. Date of Next SA2 Meetings:

3GPP SA2#166	   18-22 November 2024		Orlando, America

3GPP SA2#166-AdHoc-e   20-24 January 2024		Online