3GPP TSG-SA2 Meeting #165	S2-2409678

# 14 – 18 October, 2024, Hyderabad, India

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1198 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Refinements for VFL feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Federated Learning are composed of Horizontal Federated Learning and Vertical Federated Learning. Horizontal Federated Learning have been supported in R18. In order to avoid any confusion, the majority agree to use the terminology Vertical Federated Learning e.g. feature naming, feature descrption,VFL model ID and capability indiation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the existing teminology from FL to HFL; - Refine the HFL capability information with a new IE FL type (i.e. HFL); - Refine the VFL capablity information for NWDAF/trusted AF; - Add VFL capability information registration for untrused AF via NEF. - Complete the general discovery procedure for VFL process with the case that the AF as the VFL server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Result in confusing and inconsistency regading Horizontal Federated Learning and Vertical Federated Learning; The VFL registration and discovery procedure is not clear and not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.2H, 6.2H.1, 6.2H.2, 6.2H.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

-	If the analytics are related to UE(s) and if NWDAF instances indicate weights for TAIs in their NF profile (see clause 6.3.13 of TS 23.501 [2]), the NWDAF service consumer may use the weights for TAIs to decide which NWDAF to select.

-	If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with analytics accuracy checking capability, the consumer may query NRF providing also the analytics accuracy checking capability in the discovery request.

-	If the NWDAF service consumer needs to discover an NWDAF containing AnLF which can use the Model provided by the specific NWDAF containing MTLF (e.g., in case of analytics context transfer), the consumer should discover the NWDAF(s) whose vendor ID is in the ML Model Interoperability indicator of the NWDAF containing MTLF.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types or to collect data from particular NWDAF Serving Area, the consumer may query NRF providing the NF Set IDs or NF types or Area of Interest in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources or NWDAF Serving Area information as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	When one or more trained ML Models are available for one or more Analytics ID(s) the NWDAF containing MTLF shall include the Analytics ID(s) that is(are) supported per service in the registration towards NRF. The NWDAF containing MTLF may wait to register in NRF the above services until at least one trained model is available. The NWDAF containing MTLF may provide to the NRF a list of Analytics IDs corresponding to the trained ML Models and possibly the ML Model Filter Information for the trained ML Model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML Model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. If the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF includes, in the registration to the NRF, an ML Model Interoperability indicator for each Analytics ID.

Editor's note:	How to indicate supporting model training for the LMF-based AI/ML positioning as defined in TS 23.273 [39] by the MTLF to NRF, e.g., using a specific analytics ID or a new indication is FFS.

Editor's note:	For model training for LMF-based AI/ML positioning, whether Positioning case information (e.g. Uplink/downlink, or case 2b, 3b) can be included in NF profile as an optional parameter is FFS.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML Models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML Models requested by the NWDAFs from the vendors in the list.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (e.g. an NWDAF containing AnLF, an NWDAF containing MTLF as FL server or FL client) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required and Vendor ID. The NRF returns one or more candidate instances of NWDAF containing MTLF to the NF consumer and each candidate instance of NWDAF containing MTLF includes the Analytics ID(s), possibly the ML Model Filter Information for the available trained ML Models and ML Model Interoperability indicator, if available.

-	If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with ML Model accuracy checking capability, the consumer may query NRF also providing the ML Model accuracy checking capability in the discovery request.

In order to discover an NWDAF containing MTLF with Horizontal Federated Learning (HFL) capability via NRF, in addition to the procedures described above for discovering NWDAF containing MTLF:

-	An NWDAF containing MTLF supporting HFL as a server shall additionally include FL capability type (i.e. FL server), FL type (i.e. HFL) and may include Time interval supporting HFL as HFL capability information during the registration in NRF.

-	An NWDAF containing MTLF supporting HFL as a client shall additionally include FL capability type (i.e. FL client), FL type (i.e. HFL) and may include Time interval supporting HFL as HFL capability information during the registration in NRF, and it may also include, NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s) where data can be collected as input for local model training.

NOTE 5:	An NWDAF containing MTLF may indicate to support both HFL server and HFL client in the HFL capability for specific Analytics ID.

-	During the discovery of NWDAF containing MTLF as HFL server, a consumer (e.g. a NWDAF containing MTLF) may include in the request the FL capability type as FL server, FL type (i.e. HFL) and may include Time Period of Interest and ML Model Filter information for the trained ML Model(s) per Analytics ID(s), if available. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

-	During the discovery of NWDAF containing MTLF as HFL client, a consumer (e.g. an HFL server) may include in the request FL capability type as FL client, FL type (i.e. HFL) and may include Time Period of Interest, a list of NF type(s) and/or NF set ID(s) of the data source(s). The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

NOTE 6:	The service consumer to discover an NWDAF containing MTLF with HFL capability is limited to NWDAF containing MTLF in this Release.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming exchange capability (RE-NWDAF) to request analytics or input data is discovered via the NRF. A consumer in the same PLMN as the RE-NWDAF discovers the RE-NWDAF(s) by querying for NWDAF(s) where the roaming exchange capability is indicated in its (their) NF profile. A consumer in a peer PLMN (i.e. RE-NWDAF) discovers the RE-NWDAF(s) by querying for NWDAF(s) in the target PLMN that is (are) supporting the specific services defined for roaming. A RE-NWDAF discovers the RE-NWDAF(s) in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

To discover a VFL server or a VFL client via NRF,

an NWDAF/trusted AF:

-	Acting as VFL server shall include its VFL capability information including FL capability type (i.e. FL server), FL type (i.e. VFL) and may include Time interval supporting VFL, during the registration in NRF.

-	Acting as VFL client shall include its VFL capability information including FL capability type (i.e. FL client),  FL type (i.e. VFL) and may include Time interval supporting VFL, during the registration in NRF.

an untrusted AF:

-	Acting as VFL server shall registers to the NEF: AF identification, AF service identification (e.g. endpoint information of AF’s VFL service), external Analyics ID(s) and its VFL capability information. The AF’s VFL capability information includes FL capability type (i.e. FL server), FL type (i.e. VFL) and may include Time interval supporting VFL. The NEF generates internal analytics ID(s) mapping from the external analytics ID(s), then update its registration information (i.e. NEF Profile) including the generated internal analytics ID(s) and associated AF identification, AF service identification (e.g. endpoint information of AF’s VFL service), and VFL capability information of the AF.

-	Acting as VFL client shall registers to the NEF: AF identification, AF service identification (e.g. endpoint information of AF’s VFL service), external Analyics ID(s) and its VFL capability information. The AF’s VFL capability information includes FL capability type (i.e. FL client), FL type (i.e. VFL) and may include Time interval supporting VFL. The NEF generates internal analytics ID(s) mapping from the external analytics ID(s), then update its registration information (i.e. NEF Profile) including the generated internal analytics ID(s) and associated AF identification, AF service identification (e.g. endpoint information of AF’s VFL service), and VFL capability information of the AF.

Editor's note:	Details of ML Model handling, supported features, Feature alignment and Sample alignment are FFS.

* * *Next Change * * *

6.2H	Vertical Federated Learning among NWDAFs and AFs

6.2H.1	General

This clause specifies how NWDAF as a VFL server can register and also discover other NWDAF(s) and/or AF(s) as VFL client via NRF, and how AF as a VFL server can register and also discover other NWDAF(s) as VFL client via NRF.

6.2H.2	Procedures

6.2H.2.1	Registration and Discovery procedure for Vertical Federated Learning among NWDAFs and/or AFs

Figure 6.2H.2.1-1: Registration and Discovery procedure for Vertical Federated Learning when NWDAF is the VFL server and NWDAF(s) and/or AF(s) are the VFL client

Steps 1 to 3 are the NWDAF and AF Registration procedures for VFL.

1-3.	VFL Server/Client NWDAF and VFL Server/Client AF (trusted) register to NRF with its NF profile, which includes NF Type (as defined in clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF/AF, Service Area, VFL capability information as described in clause 5.2.

Editor's note:	Whether the VFL server/client needs to register into NRF any interoperability information is FFS.

Editor's note:	Whether to use Vendor ID(s) or introduce new identifiers for interoperability among NWDAF and AFs is FFS.

For an untrusted AF as VFL server/VFL client, it registers to NRF via NEF as defined in clause 5.2..

Steps 4 to 5 are the NWDAF and AF Discovery procedures when the VFL server is NWDAF.

4-5.	NWDAF as the VFL server determines that the ML Model requires VFL based on operator policy, Analytics ID, and Service Area.

NOTE:	Step 4 in Figure 6.2H.2.1-1 may be triggered at the VFL Server NWDAF by VFL server NWDAF itself or a request from a consumer.

If the NWDAF can not perform as VFL Server, it first discovers and selects another VFL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, FL capability type as FL server, FL type (i.e. VFL), Time Period of Interest, and Service Area.

Once the VFL Server NWDAF is determined, the VFL Server NWDAF discovers other NWDAF(s) and/or AF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytics ID, FL capability type as FL client, FL type (i.e. VFL), Time Period of Interest and Service Area.

Steps 6 to 7 are the NWDAF Discovery procedures when the VFL server is AF.

6-7.	AF as the VFL server determines that the ML Model requires VFL based on triggering from VFL server AF itself or a request from a consumer (e.g. NWDAF containing AnLF).

If the AF as the VFL server is a trusted one, it discovers other NWDAF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used by the trusted AF: Analytics ID, required NF type (i.e. NWDAF type), FL capability type as FL client, FL type (i.e. VFL), Time Period of Interest and Service Area.

If the AF is untrusted, it discovers the NWDAF as VFL client via the NEF, the NEF discovers VFL client (i.e. NWDAF) on behalf of the AF from the NRF by invoking the Nnrf_NFDiscovery_Request. The following criteria might be used by the NEF: Analytics ID, required NF type (i.e. NWDAF type), FL capability type as FL client, FL type (i.e. VFL), Time Period of Interest and Service Area. The NEF anonymizes NWDAF instances ID(s) into temporary NWDAF ID(s) and sends the temporary NWDAF ID(s) to the AF.

* * *End of Changes * * *