3GPP TSG-SA WG2 Meeting #165	S2-2409681

# Hyderabad, India, Oct 14 – Oct 18, 2024										   (revision of S2-24xxxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5636 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new network function for energy related information, its definition and corresponding Architecture Reference Model |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rakuten Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2024-09-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Further to: - the conclusions in the TR 23.700-66 v19.0.0, clause 8.1 for KI#1; and - approved WID (SP-241388) from SA#105 this contribution proposes the below new feature addition to TS 23.501 “A new network functionality is defined to collect and calculate the energy related information” - A new NF called Energy Efficiency and Consumption Function (EECF) introduced. - Architecture Reference Model to support Energy Efficiency and Consumption Functionality is introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New EnergySys feature addition for KI#1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | KI#1 conclusion is not implemented in TS 23.501 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2.2, 4.2.6, 4.2.7, 4.2.x (new), 6.2.x (new), 6.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * *

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5G DDNMF	5G Direct Discovery Name Management Function

5G LAN	5G Local Area Network

5GS	5G System

5G-AN	5G Access Network

5G-AN PDB	5G Access Network Packet Delay Budget

5G-EIR	5G-Equipment Identity Register

5G-GUTI	5G Globally Unique Temporary Identifier

5G-BRG	5G Broadband Residential Gateway

5G-CRG	5G Cable Residential Gateway

5G GM	5G Grand Master

5G NSWO	5G Non-Seamless WLAN offload

5G-RG	5G Residential Gateway

5G-S-TMSI	5G S-Temporary Mobile Subscription Identifier

5G VN	5G Virtual Network

5QI	5G QoS Identifier

ADRF	Analytics Data Repository Function

AF	Application Function

AI/ML	Artificial Intelligence/Machine Learning

AKMA	Authentication and Key Management for Applications

AnLF	Analytics Logical Function

AMF	Access and Mobility Management Function

AoI	Area of Interest

AS	Access Stratum

ATSSS	Access Traffic Steering, Switching, Splitting

ATSSS-LL	ATSSS Low-Layer

AUSF	Authentication Server Function

BMCA	Best Master Clock Algorithm

BSF	Binding Support Function

CAG	Closed Access Group

CAPIF	Common API Framework for 3GPP northbound APIs

CH	Credentials Holder

CHF	Charging Function

CN PDB	Core Network Packet Delay Budget

CP	Control Plane

CQRCI	Clock Quality Reporting Control Information

DAPS	Dual Active Protocol Stacks

DCCF	Data Collection Coordination Function

DCS	Default Credentials Server

DetNet	Deterministic Networking

DL	Downlink

DN	Data Network

DNAI	DN Access Identifier

DNN	Data Network Name

DRX	Discontinuous Reception

DS-TT	Device-side TSN translator

EAC	Early Admission Control

ePDG	evolved Packet Data Gateway

EBI	EPS Bearer Identity

EECF	Energy Efficiency and Consumption Function

EUI	Extended Unique Identifier

FAR	Forwarding Action Rule

FL	Federated Learning

FN-BRG	Fixed Network Broadband RG

FN-CRG	Fixed Network Cable RG

FN-RG	Fixed Network RG

FQDN	Fully Qualified Domain Name

GBA	Generic Bootstrapping Architecture

GEO	Geostationary Orbit

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network Selection

GMLC	Gateway Mobile Location Centre

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

HMTC	High-Performance Machine-Type Communications

HR	Home Routed (roaming)

IAB	Integrated access and backhaul

IMEI/TAC	IMEI Type Allocation Code

IPUPS	Inter PLMN UP Security

I-SMF	Intermediate SMF

I-UPF	Intermediate UPF

LADN	Local Area Data Network

LBO	Local Break Out (roaming)

LEO	Low Earth Orbit

LMF	Location Management Function

LoA	Level of Automation

LPP	LTE Positioning Protocol

LRF	Location Retrieval Function

L4S	Low Latency, Low Loss and Scalable Throughput

MBS	Multicast/Broadcast Service

MBSF	Multicast/Broadcast Service Function

MBSR	Mobile Base Station Relay

MBSTF	Multicast/Broadcast Service Transport Function

MB-SMF	Multicast/Broadcast Session Management Function

MB-UPF	Multicast/Broadcast User Plane Function

MEO	Medium Earth Orbit

MFAF	Messaging Framework Adaptor Function

MCX	Mission Critical Service

MDBV	Maximum Data Burst Volume

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

MINT	Minimization of Service Interruption

ML	Machine Learning

MPQUIC	Multi-Path QUIC

MPS	Multimedia Priority Service

MPTCP	Multi-Path TCP Protocol

MTLF	Model Training Logical Function

MWAB	Mobile gNB with wireless access backhauling

N3IWF	Non-3GPP InterWorking Function

N3QAI		Non-3GPP QoS Assistance Information

N5CW	Non-5G-Capable over WLAN

NAI	Network Access Identifier

NAT	Network Address Translation

NCR	Network Controlled Repeater

NCR-MT	NCR Mobile Termination

NEF	Network Exposure Function

NF	Network Function

NGAP	Next Generation Application Protocol

NID	Network identifier

NPN	Non-Public Network

NR	New Radio

NRF	Network Repository Function

NS-AoS	Network Slice Area of Service

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

NSAG	Network Slice AS Group

NSI ID	Network Slice Instance Identifier

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-specific and SNPN Authentication and Authorization Function

NSSAI	Network Slice Selection Assistance Information

NSSF	Network Slice Selection Function

NSSP	Network Slice Selection Policy

NSSRG	Network Slice Simultaneous Registration Group

NSWO	Non-Seamless WLAN offload

NSWOF	Non-Seamless WLAN offload Function

NW-TT	Network-side TSN translator

NWDAF	Network Data Analytics Function

ONN	Onboarding Network

ON-SNPN	Onboarding Standalone Non-Public Network

PCF	Policy Control Function

PDB	Packet Delay Budget

PDR	Packet Detection Rule

PDU	Protocol Data Unit

PDV	Packet Delay Variation

PEGC	PIN Element with Gateway Capability

PEI	Permanent Equipment Identifier

PEMC	PIN Element with Management Capability

PER	Packet Error Rate

PFD	Packet Flow Description

PIN	Personal IoT Network

PINE	PIN Element

PLR	Packet Loss Rate

PNI-NPN	Public Network Integrated Non-Public Network

PPD	Paging Policy Differentiation

PPF	Paging Proceed Flag

PPI	Paging Policy Indicator

PSA	PDU Session Anchor

PSDB	PDU Set Delay Budget

PSER	PDU Set Error Rate

PSIHI	PDU Set Integrated Handling Information

PTP	Precision Time Protocol

PVS	Provisioning Server

QFI	QoS Flow Identifier

QMC	QoE Measurement Collection

QoE	Quality of Experience

RACS	Radio Capabilities Signalling optimisation

(R)AN	(Radio) Access Network

RG	Residential Gateway

RIM	Remote Interference Management

RQA	Reflective QoS Attribute

RQI	Reflective QoS Indication

RSN	Redundancy Sequence Number

RTT	Round Trip Time

SA NR	Standalone New Radio

SBA	Service Based Architecture

SBI	Service Based Interface

SCP	Service Communication Proxy

SD	Slice Differentiator

SEAF	Security Anchor Functionality

SEPP	Security Edge Protection Proxy

SF	Service Function

SFC	Service Function Chain

SMF	Session Management Function

SMSF	Short Message Service Function

SN	Sequence Number

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SO-SNPN	Subscription Owner Standalone Non-Public Network

SSC	Session and Service Continuity

SSCMSP	Session and Service Continuity Mode Selection Policy

SST	Slice/Service Type

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

SV	Software Version

TA	Tracking Area

TAI	Tracking Area Identity

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TNL	Transport Network Layer

TNLA	Transport Network Layer Association

TSC	Time Sensitive Communication

TSCAC	TSC Assistance Container

TSCAI	Traffic Assistance Information

TSCTSF	Time Sensitive Communication and Time Synchronization Function

TSN	Time Sensitive Networking

TSN GM	TSN Grand Master

TSP	Traffic Steering Policy

TSS	Timing Synchronization Status

TT	TSN Translator

TWIF	Trusted WLAN Interworking Function

UAS NF	Uncrewed Aerial System Network Function

UCMF	UE radio Capability Management Function

UDM	Unified Data Management

UDR	Unified Data Repository

UDSF	Unstructured Data Storage Function

UL	Uplink

UL CL	Uplink Classifier

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

URRP-AMF	UE Reachability Request Parameter for AMF

URSP	UE Route Selection Policy

VID	VLAN Identifier

VLAN	Virtual Local Area Network

W-5GAN	Wireline 5G Access Network

W-5GBAN	Wireline BBF Access Network

W-5GCAN	Wireline 5G Cable Access Network

W-AGF	Wireline Access Gateway Function

# * * * * Next change * * *

### 4.2.2	Network Functions and entities

The 5G System architecture consists of the following network functions (NF):

-	Authentication Server Function (AUSF).

-	Access and Mobility Management Function (AMF).

-	Data Network (DN), e.g. operator services, Internet access or 3rd party services.

-	Unstructured Data Storage Function (UDSF).

-	Network Exposure Function (NEF).

-	Network Repository Function (NRF).

-	Network Slice Admission Control Function (NSACF).

-	Network Slice-specific and SNPN Authentication and Authorization Function (NSSAAF).

-	Network Slice Selection Function (NSSF).

-	Policy Control Function (PCF).

-	Session Management Function (SMF).

-	Unified Data Management (UDM).

-	Unified Data Repository (UDR).

-	User Plane Function (UPF).

-	UE radio Capability Management Function (UCMF).

-	Application Function (AF).

-	User Equipment (UE).

-	(Radio) Access Network ((R)AN).

-	5G-Equipment Identity Register (5G-EIR).

-	Network Data Analytics Function (NWDAF).

-	CHarging Function (CHF).

-	Time Sensitive Networking AF (TSN AF).

-	Time Sensitive Communication and Time Synchronization Function (TSCTSF).

-	Data Collection Coordination Function (DCCF).

-	Analytics Data Repository Function (ADRF).

-	Messaging Framework Adaptor Function (MFAF).

-	Non-Seamless WLAN Offload Function (NSWOF).

NOTE:	The functionalities provided by DCCF and/or ADRF can also be hosted by an NWDAF.

-	Edge Application Server Discovery Function (EASDF).

The 5G System architecture also comprises the following network entities:

-	Service Communication Proxy (SCP).

-	Security Edge Protection Proxy (SEPP).

The functional descriptions of these Network Functions and entities are specified in clause 6.

-	Non-3GPP InterWorking Function (N3IWF).

-	Trusted Non-3GPP Gateway Function (TNGF).

-	Wireline Access Gateway Function (W-AGF).

-	Trusted WLAN Interworking Function (TWIF).

-	Energy Efficiency and Consumption Function (EECF)

# * * * * Next change * * *

### 4.2.6	Service-based interfaces

The 5G System Architecture contains the following service-based interfaces:

Namf:	Service-based interface exhibited by AMF.

Nsmf:	Service-based interface exhibited by SMF.

Nnef:	Service-based interface exhibited by NEF.

Npcf:	Service-based interface exhibited by PCF.

Nudm:	Service-based interface exhibited by UDM.

Naf:	Service-based interface exhibited by AF.

Nnrf:	Service-based interface exhibited by NRF.

Nnsacf:	Service-based interface exhibited by NSACF.

Nnssaaf:	Service-based interface exhibited by NSSAAF.

Nnssf:	Service-based interface exhibited by NSSF.

Nausf:	Service-based interface exhibited by AUSF.

Nudr:	Service-based interface exhibited by UDR.

Nudsf:	Service-based interface exhibited by UDSF.

N5g-eir:	Service-based interface exhibited by 5G-EIR.

Nnwdaf:	Service-based interface exhibited by NWDAF.

Nchf:	Service-based interface exhibited by CHF.

Nucmf:	Service-based interface exhibited by UCMF.

Ndccf:	Service based interface exhibited by DCCF.

Nmfaf:	Service based interface exhibited by MFAF.

Nadrf:	Service based interface exhibited by ADRF.

Naanf:	Service-based interface exhibited by AANF.

NOTE 1:	The Service-based interface exhibited by AANF is defined in TS 33.535 [124].

N5g-ddnmf:	Service-based interface exhibited by 5G DDNMF.

Nmbsmf:	Service-based interface exhibited by MB-SMF.

Nmbsf:	Service-based interface exhibited by MBSF.

NOTE 2:	The Service-based interfaces exhibited by MB-SMF and MBSF are defined in TS 23.247 [129].

Ntsctsf:	Service-based interface exhibited by TSCTSF.

Nbsp:	Service-based interface exhibited by an SBI capable Boostrapping Server Function in GBA.

NOTE 2:	The Service-based interfaces exhibited by an SBI capable Boostrapping Server Function are defined in TS 33.220 [140] and TS 33.223 [141].

Neasdf:	Service-based interface exhibited by EASDF.

NOTE 3:	The Service-based interfaces exhibited by EADSF is defined in TS 23.548 [130].

Nupf:	Service-based interface exhibited by UPF.

Neecf:	Service-based interface exhibited by EECF.

### 4.2.7	Reference points

The 5G System Architecture contains the following reference points:

N1:	Reference point between the UE and the AMF.

N2:	Reference point between the (R)AN and the AMF.

N3:	Reference point between the (R)AN and the UPF.

N4:	Reference point between the SMF and the UPF.

N6:	Reference point between the UPF and a Data Network.

N9:	Reference point between two UPFs.

The following reference points show the interactions that exist between the NF services in the NFs. These reference points are realized by corresponding NF service-based interfaces and by specifying the identified consumer and producer NF service as well as their interaction in order to realize a particular system procedure.

N5:	Reference point between the PCF and an AF or TSN AF.

N7:	Reference point between the SMF and the PCF.

N8:	Reference point between the UDM and the AMF.

N10:	Reference point between the UDM and the SMF.

N11:	Reference point between the AMF and the SMF.

N12:	Reference point between AMF and AUSF.

N13:	Reference point between the UDM and Authentication Server function the AUSF.

N14:	Reference point between two AMFs.

N15:	Reference point between the PCF and the AMF in the case of non-roaming scenario, PCF in the visited network and AMF in the case of roaming scenario.

N16:	Reference point between two SMFs, (in roaming case between SMF in the visited network and the SMF in the home network).

N16a:	Reference point between SMF and I-SMF.

N17:	Reference point between AMF and 5G-EIR.

N18:	Reference point between any NF and UDSF.

N19:	Reference point between two PSA UPFs for 5G LAN-type service.

N22:	Reference point between AMF and NSSF.

N23:	Reference point between PCF and NWDAF.

N24:	Reference point between the PCF in the visited network and the PCF in the home network.

N27:	Reference point between NRF in the visited network and the NRF in the home network.

N28:	Reference point between PCF and CHF.

N29:	Reference point between NEF and SMF.

N30:	Reference point between PCF and NEF.

NOTE 1:	The functionality of N28 and N29 and N30 reference points are defined in TS 23.503 [45].

N31:	Reference point between the NSSF in the visited network and the NSSF in the home network.

NOTE 2:	In some cases, a couple of NFs may need to be associated with each other to serve a UE.

N32:	Reference point between a SEPP in one PLMN or SNPN and a SEPP in another PLMN or SNPN; or between a SEPP in a SNPN and a SEPP in a CH/DCS, where the CH/DCS contains a UDM/AUSF.

NOTE 3:	The functionality of N32 reference point is defined in TS 33.501 [29].

N33:	Reference point between NEF and AF.

N34:	Reference point between NSSF and NWDAF.

N35:	Reference point between UDM and UDR.

N36:	Reference point between PCF and UDR.

N37:	Reference point between NEF and UDR.

N38:	Reference point between I-SMFs and between V-SMFs.

N40:	Reference point between SMF and the CHF.

N41:	Reference point between AMF and CHF in HPLMN.

N42:	Reference point between AMF and CHF in VPLMN.

NOTE 4:	The functionality of N40, N41 and N42 reference points are defined in TS 32.240 [41].

N43:	Reference point between PCFs.

NOTE 5:	The functionality of N43 reference point is defined in TS 23.503 [45].

NOTE 6:	The reference points from N44 up to and including N49 are reserved for allocation and definition in TS 32.240 [41].

N50:	Reference point between AMF and the CBCF.

N51:	Reference point between AMF and NEF.

N52:	Reference point between NEF and UDM.

N55:	Reference point between AMF and the UCMF.

N56:	Reference point between NEF and the UCMF.

N57:	Reference point between AF and the UCMF.

NOTE 7:	The Public Warning System functionality of N50 reference point is defined in TS 23.041 [46].

N58:	Reference point between AMF and the NSSAAF.

N59:	Reference point between UDM and the NSSAAF.

N60:	Reference point between AUSF and NSWOF.

NOTE 8:	The functionality of N60 reference point is defined in TS 33.501 [29].

N80:	Reference point between AMF and NSACF.

N81:	Reference point between SMF and NSACF.

N82:	Reference point between NSACF and NEF.

N83:	Reference point between AUSF and NSSAAF.

N84:	Reference point between TSCTSF and PCF.

N85:	Reference point between TSCTSF and NEF.

N86:	Reference point between TSCTSF and AF.

N87:	Reference point between TSCTSF and UDM.

N88:	Reference point between SMF and EASDF.

N89:	Reference point between TSCTSF and AMF.

N96:	Reference point between TSCTSF and NRF.

N97:	Reference point between two NSACFs in different PLMNs.

N99:	Reference point between two NSACFs within the same PLMN.

Nx1:	Reference point between EECF and AF.

Nx2:	Reference point between EECF and NEF.

Nx3:	Reference point between EECF and UDM.

Nx4:	Reference point between EECF and AMF.

Nx5:	Reference point between EECF and SMF.

Nx6:	Reference point between EECF and PCF.

NOTE 9:	The reference points from N90 up to and including N95 are reserved for allocation and definition in TS 23.503 [45].

NOTE 10:	The reference points from N100 up to and including N109 are reserved for allocation and definition in TS 32.240 [41].

The reference points to support SMS over NAS are listed in clause 4.4.2.2.

The reference points to support Location Services are listed in TS 23.273 [87].

The reference points to support SBA in IMS (N5, N70 and N71) are described in TS 23.228 [15].

The reference points to support AKMA (N61, N62 and N63) are described in TS 33.535 [124].

The reference points to support 5G ProSe are described in TS 23.304 [128].

The reference points to support 5G multicast-broadcast services are described in TS 23.247 [129].

The reference points to Support Uncrewed Aerial Systems (UAS) connectivity, identification and tracking are described in TS 23.256 [136].

The reference points to support SBA in GBA and GBA push (N65, N66, N67 and N68) are described in TS 33.220 [140] and TS 33.223 [141].

The reference points to support SMS delivery using SBA are described in TS 23.540 [142].

The reference points to support Ranging based services and Sidelink Positioning are described in TS 23.586 [180].

# * * * * Next change * * *

### 4.2.x	Architecture Reference Model for Energy Efficiency and Consumption Feature Support

In order to support the Energy Efficiency and Consumption feature, the 5G System Architecture is extended as shown in Figure 4.2.x-1 and Figure 4.2.x-2. The additional functionality that is supported by the network functions is specified in clause 5.x. In summary:

#### 4.2.x.1	Non-roaming architecture

Figure 4.2.x-1 depicts the non-roaming reference architecture using service-based interfaces. The figure depicts only the NFs enhanced to support the Energy Efficiency and Consumption functionality

Figure 4.2.x-2 depicts the non-roaming reference architecture, using the reference point representation showing how various network functions interact with each other. The figure depicts the direct interface of the EECF with other NFs only.

### Figure 4.2.x-1: Non-roaming architecture for Energy Efficiency and Consumption

Figure 4.2.x-1: Non-roaming architecture for Energy Efficiency and Consumption in reference point representation

NOTE: 	The interface between EECF and MnS producer is not depicted in above figures.

Editor’s note: 	The interface between the EECF and the MnS producer will be defined in coordination with SA5   and is FFS.

# * * * * End of changes * * *

### 6.2.x	EECF

The Energy Efficinecy and Control Function (EECF) supports the following functionality:

-	Support of collection and calculation of the end-to-end energy related information of NG-RAN and UPF at various granualties. The energy related information includes energy consumption information and renewable energy information.

Note :	Various supported granualities are defined in clause 5.x

-	Support of exposure of the energy consumption information to AF and other NFs.

-	Support of interaction with OAM via MnS producer to retrieve the required information in order to perform the evalution of the energy related parameters.

The details of the EECF functionality are defined in clause 5.x.

# * * * * Next change * * *

### 6.3.x	EECF discovery and selection

The NFs (e.g. NEF, AF, PCF) may utilise the NRF to discover EECF instance(s) unless EECF information is available by other means, e.g. locally configured in NF consumers.

The following factors may be considered by the NF consumer for EECF selection:

-	SUPI or Internal Group ID